Haryana Building and Other Construction Workers (Regulation of

Employment and Conditions of Service) Rules, 2005.

Haryana   Building   and   Other   Construction   Workers   (Regulation   of

Employment and Conditions of Service) Rules, 2005

CHAPTER-1

Preliminary

1.

Short title, application and commencement.

(I)

These rules may be called the Haryana Building and Other Construction

Workers (Regulation of Employment and Conditions of Service) Rules,

2005.

(2)

They shall apply to the building or other construction work relating to

any  establishment  in  relation  to  which  appropriate  Government  is  the

State Government under the Act

(3)

They  shall  come  into  force  on  the  date  of  their  publication  in  the

Official Gazette.

2.      Definitions

(1)

In these rules, unless the context otherwise requires.-

(a)

“Act”  means  the  Building  and  Other  Construction  Workers

(Regulation  of  Employment  and  Conditions  of  Service)  Act,

1996 (Central Act 27 of 1996);

(b)

“access”  or  “egress”  means  passageways,  corridors,  stairs,

platforms, ladders and any other means to be used by a building

workers  for  normally  entering  or  leaving  the  workplace  or  for

escaping in case of danger;

(c)

“approved” means approved in writing by the Chief  Inspector

or the State Government, as the case may be;

(d)

“base  plate”  means  a  plate  for  distributing  the  load  from  a

standard in the case of metal scaffolds;

(e)

“bay” in relation to scaffolds, means that portion of the scaffold

between  horizontal  or  vertical  supports  whether  standards  or

supports  from  which  the  portion  is  suspended,  which  are

adjacent longitudinally;

(f)      “brace”   means   a   member   incorporated   diagonally   in   a

scaffold

(g)

“bulkhead” means an airtight structure separating the working

chamber from free air or from another chamber under a lower

pressure than the working pressure ;

1

(h)

“caisson”  means  an  air  and  watertight  chamber  in  which  it  is

possible  for  men  to  work  under  air  pressure  greater  than

atmospheric  pressure  at  sea  level  to  excavate  material  below

water level;

(i)

“cofferdam” means a structure constructed entirely or in part

below water level or below the level of the water Table in the

ground and intended to provide a place for work that is free of

water;

(j)

“competent  person”  means  a  person  so  approved  by  the

State  Government,  who  belongs  to  a  testing  establishment  in

State possessing adequate qualification, experience and skill for

the   purposes   of   testing,   examination   or   annealing   and

certification  of  lifting  appliances,  lifting  gears,  wire  ropes  or

pressure plant or equipment;

(k)

“compressed air” means air mechanically raised to a pressure

higher than atmospheric pressure at sea level;

(l)

“construction  site”  means  any  site  at  which  any  of  the

processes   or   operations,   related   to   building   or   other

construction work, are carried on;

(m)    “conveyor”  means  a  mechanical  device  used  in  building  or

other  construction  work  for  transport  of  building  material,

articles,  or  packages  or  solid  bulk  from  one  point  to  another

point;

(n)     “danger”  means  danger  of  accident  or  of  injury  or  to

health;

(o)

“decanting”  means  the  rapid  decompression  of  person  in  a

man-lock to atmospheric pressure at sea level followed promptly

by  their  recompression  in  a  decant  lock,  where  they  are  then

decompressed according to the appropriate decompression table

in accordance with approved decompression procedures;

(p)

“demolition work” means the work incidental to or connected

with the total or partial dismantling or razing of a building or a

structure  other  than  a  building  and  includes  the  removing  or

dismantling or machines or other equipment;

(q)     “excavation”  means  the  removal  of  earth,  rock  or  other

material in connection with construction or demolition work;

2

(r)

“false  works”  means  the  structural  supports  and  bracing  for

formworks or forms;

(s)

“flashpoint” means  the  minimum  liquid  temperature  at  which

a  spark  or  flame  causes  an  instantaneous  flash  in  the  vapour

space above the liquid;

(t) 
(u)

“forms” means the form appended to these rules;

“frame or modular scaffold” means a scaffold manufactured

in  such  a  way  that  the  geometry  of  the  scaffold  is  pre-

determined  and  the  relative  spacings  of  the  principal  members

are fixed;

(v)

“guardrail” means  a  horizontal  rail  secured  to  uprights  and

erected  along  the  expoused  sides  of  scaffolds,  floor  openings,

ruways and gangways to prevent persons from falling;

(w) 
(x)

“hazard” means danger or potential danger;

“hazardous  substance”  means  any  substance  which  due  to

its explosiveness, inflammability, radio-activity, toxic or corrosive

properties, or other similar characteristics, may—

(i) 
(ii)

cause injury; or

affect adversely the human system; or

(iii)     cause  loss  of  life  or  damage  to  property  on  work

environment, while handling, transporting or storing and

classified as such under the national standards or in case

such  national  standards  do  not  exist  to  the  generally

accepted international standards;

(y)

“high  pressure  air”  means  air  used  to  supply  power  to

pneumatic tools and devices

(z)

“independent  tied  scaffold”  means  a  scaffold,  the  working

platform  of  which  is  supported  from  the  base  by  two  or  more

rows  of  standards  and  which  apart  from  the  necessary  ties

stands completely free of the building;

(za)   “ledger”   means   a   member   spanning   horizontally   and

typingscaffolding longitudinally and which acts as a support for

putlogs or transoms;

(zb)

“lifting    appliance”  means  a  crane,  hoist,  derrick,  winch,  gin

pole, sheer legs, jack, pulley block or other equipment used for

lifting materials, objects or, building worker;

3

(zc)    “lifting  gear”  means  ropes,  chains  hooks,  sling  and  other

accessories of a “ lifting appliance”;

(zd)

“lock attendant” means the person in charge of a man-lock or

medical lock and who is immediately responsible for controlling

the compression, recompression or decompression or persons in

such locks ;

(ze)

“low  pressure  air”  means  air  supplied  to  pressurize  working

chamber and man-locks and medical locks;

(zf)

“magazine” means  a  place  in  which  explosives  are  stored  or

kept, whether above or below ground;

(zg)   “man-lock”  means  any  lock,  other  than  a  medical  lock,

used   for   the   compression   or   decompression   of   persons

entering or leaving a working chamber;

(zh)   “material  hoist”  means  a  power  or  manually  operated  and

suspended platform or bucket operating in guide rails and used

for  raising  or  lowering  material  exclusively  and  operated  and

controlled from a point outside the conveyance;

(zi)

“materials lock” means a chamber through which   materials

and equipments pass from one air pressure

environment into

another;

(zj)    “national   standards”   means   standards   as   approved   by

Bureau  of  Indian  Standards  and  in  the  absence  of  such

standards   of   Bureau   of   Indian   Standards,   the   standards

approved by the State Government for a specific purpose;

(zk)

“outrigger”   means   a   structure   projecting   beyond   the

façade  of  a  building  with    the  inner  end  being  anchored

and includes a cantilever or other support:

(zl)

“plant  or  equipment”  includes  any  plant,  equipment,  gear,

machinery, apparatus or appliance, or any part thereof;

(zm)   “pressure”  means  air  pressure  in  bars  above  atmospheric

pressure;

(zn)   “pressure  plant”  means  the  pressure  vessel  alongwith  its

piping and other fittings operated at a pressure greater than the

atmospheric pressure;

(zo)   “putlog”  means  a  horizontal  member  on  which  the  board,

plank or decking of a working platform are laid;

4

(zp)   “responsible  person”  means  a  person  appointed  by  the

employer to be responsible for the performance of specific duty

or duties and who has sufficient knowledge and experience and

the requisite authority for the proper performance of such duty

or duties;

(zq)

“reveal  tie”    means  the  assembly  of  a  tie  tube  and  a  fittings

used for tightening a tube between two opposite surfaces;

(zr)

“right  angle  coupler”  means  a  coupler,  other  than  a  swivel

or putlog coupler, used for connecting tubes at right angles;

(zs)

“rock bolt” means a mechanical expansion bolt or a bolt used

with  cementitious  or  resin  anchoring  system  which  is  set  in

drilled  hole  in  the  arch  or  wall    of  a  tunnel  to  improve  rock

competency;

(zt)    “roofing  bracket”  means  a  bracket  used  in  sloped  roof

construction  and  having  sharp  points  or  other  means  for

fastening to prevent slipping;

(zu)   “safety  screen”  means  an  air  and  water  tight  diaphragm

placed  across  the  upper  part  of  a  compressed  air  tunnel

between the face and hulkhead, in order to prevent flooding the

crown of tunnel between the safety screen and the bulkhead to

provide  a  safe  means  of  refuge  and  exit  from  a  flooding  or

flooded tunnel;

(zv)

“safe working load” in relation to an article of lifting gear or

lifting  appliance,  means  the  load  which  is  the  maximum  load

that may be imposed on such article or appliance with safety in

the  normal  working  conditions  as  assessed  and  certified  by  a

competent person.;

(zw)   “scaffold”  means  any  temporarily  provided  structure  on  or

from  which  building  workers  perform  work  in  connection  with

building  or  other  construction  work  to  which  these  rules  apply,

and  any  temporarily  provided  structure  which  enables  building

workers  to  obtain  access  to  or  which  enables  materials  to  be

taken  to  any  place  at  which  such  work  is  performed,  and

includes  any  working  platform,  gangway,  run,  ladder  or  step-

ladder ( other than a ladder or step-ladder which does not form

part of such structure) together with any guardrail, toe board or

5

other  safeguards  and  all  fixing,  but  does  not  include  lifting

appliance  or  a  lifting  machine  or  a  structure  used  merely  to

support  such  an  appliance  or  such  a  machine  or  to  support

other plant or equipment;

(zx)

“Schedule  ” means a Schedule appended to these rules;

(zy)

“section” means the section of the Act;

( zz)   “segment” includes a cast iron or precast concrete segmented

structure  formed  to  the  curvature  of  the  tunnel  cross-section

and used to support the ground surrounding the tunnel:

(zza)  “service  shaft”  means  a  shaft  for  the  passage  of  building

workers or materials to or from a tunnel under construction;

(zzb)  “shaft”  means  an  excavation  having  a  longitudinal  axis  at  an

angle greater than forty five degree from the horizontal-

(i)

for  the  passage  of  building  workers  or  materials  to  or

from a tunnel; or

(ii)

leading to an existing tunnel;

(zzc)  “shield” means  a  movable  frame  which  supports  the  working

face  of  a  tunnel  and  the  ground  immediately  behind  it  and

includes  equipment  designed  to  excavate  and  support  the

excavated areas in a tunnel;

(zzd)  “sole plate” means a member used to distribute the load from

the  base  plate  or  the  standard  of  wooden  scaffolds  to  the

supporting surface;

(zze)  “sound    or    good    construction”    means    construction

conforming  to  the  relevant  national  standards  or  in  case  such

national  standards  do  not  exist,  to  other  generally  accepted

international engineering standards or code of practices;

(zzf)   “sound  or  good  material”  means  materials  of  a  quality

conforming  to  the  relevant  national  standards  or  in  case  such

national  standards  do  not  exist,  to  other  generally  accepted

international engineering standards or code of practices;

(zzg)  “standard” means  a  member  used  as  a  vertical  support  or

column in the construction of scaffolds which transmits a load to

the ground or to the solid construction;

(zzh)  “standard  safe  operating  practices”  means  the  practice

followed  in  building  and  other  construction  activities  for  the

6

safety and health of workers and safe operation of machineries

and  equipment  used  in  such  activities  and  such  practices

conforms to all or any of the following, namely;-

(i)       relevant   standards   approved   by   Bureau   of   India

Standards;

(ii) 
(iii)

National Building Code;

manufacturer’s instructions on safe use of equipment and

machinery;

(iv)

code of practice on safety and health in  construction

industry  published  by  International  Labour  Organisation

and amended from time to time;

(zzi)   “State    Government”    or    “Government”   means   the

Government of the State of Haryana;

(zzj)   “steel  rib”  includes  all  steel  beams  and  other  structural

members   shaped   to   conform   to   the   requirements   of   a

particulars  tunnel  cross  section,  used  for  the  purpose  of

supporting and stabilizing  the excavated areas;

(zzk)  “suspended scaffold” means a scaffold suspended by means

of  ropes  or  chains  and  capable  of  being  raised  or  lowered  but

does not include a boatswain’s chair or similar appliance;

(zzl)

“testing establishment” means an establishment with testing

and examination facilities as approved by the State Government

for carrying out testing, examination, annealing or similar other

test  or  certification  of  lifting  appliances  or  lifting  gear  or  wire

rope as required under these rules;

(zzm) “tie” means an assembly used to connect a scaffold to a rigid

anchorage;

(zzn)  “toe  board”  means  a  member  fastened  above  a  working

platform access landing access way, wheel barrow run, ramp or

other platform to prevent building workers and materials falling

there from;

(zzo)  “transom” means a member placed horizontally and used to tie

transversely one ledger to another or one standard to another in

an independent tie scaffold;

7

(zzp)  “trestle scaffold” includes a scaffold in which the supports for

the platform are any of the following which are self supporting,

namely;-

(i) 
(ii)

(iii) 
(iv)

(v)

split heads;

folfing;

step-ladder

tripods; or

movable contrivances similar to any of the foregoing;

(zzq)  “tobular  scaffold”  means  a  scaffold  constructed  from  tubes

and couplers;

(zzr)   “tunnel” means  a  subterranean  passage  made  by  excavating

beneath the over-burden into which a building worker enters or

is required to enter to work;

(zzs)  “underground”  means  any  space  within  the  confines  or  a

shaft, tunnel, caisson or cofferdam;

(zzt)

“vehicle” means a vehicle propelled or driven by mechanical or

electrical  power  and  includes  a  trailer,  traction  engine,  tractor,

road-building machine and transport equipment;

(zzu)  “working chamber” means the part of construction site where

work  in  a  compressed  air  environment  is  carried  out  but  does

not include a manlock or medical lock;

(zzv)  “working  platform”  means  a  platform  which  is  used  to

support  building  workers  or  materials  and  includes  a  working

stage;

(zzw) “working  pressure”  means  pressure,  in  a  working  chamber,

to which building worker is exposed;

(zzx)  “workplace”   means  all  places  where  building  workers  are

required  to  be  present  or  to  go  for  work  and  which  are  under

the control of an employer;

(2).   Words and expressions not defined in these rules but defined or used

in  the  Act,  shall  have  the  same  meaning  as  assigned  to  them  in  the

Act.

8

CHAPTER-II

State Advisory Committee

3.

Constitution of State Advisory Committee Sec. 4-

The State Building and Other Construction  Workers’ Advisory

Committee (hereinafter referred to as the State Advisory Committee) shall

consist of

(a) 
(b)

a Chairperson to be appointed by the State Government;

two  members  of  State  Legislature  who  shall  be  elected  by  the

State legislature as members.

(c) 
(d)

(e)

one member to be nominated by the Central Government;

Chief Inspector- member ex-officio ;

three persons representing building workers to be nominated by

the State Government as members;

(f)

three  persons  to  be  nominated  by  the  State  Government  for

representing  the  employers  connected  with  the  building  and

other construction work-members;

(g)     five   persons   to   be   nominated   by   the   State   Government

representing  one  from  the  State  level  association  of  architects

and  one  from  engineers  and  one  from  an  accident  insurance

institution  having  experience  of  building  construction,  one

welfare expert and one from the media.

4.

Terms of Office Sec. 4

(1)

The Chairperson of the State Advisory Committee shall hold office as

such for a period of five years from the date on which his appointment

is notified in the Official Gazette.

(2)

Each member referred to in clause (b) of rule 3 shall hold the office for

five years or till he remains a member of the State legislature, as the

case may be, whichever is earlier.

9

(3)

The  members  referred  to  in  clause(c)  and  clause  (d)  of  rule  3  shall

hold  office  as  such  during  the  period  as  the  State  Government  may

specify.

(4)

Each of the members referred to in clause (e), clause (f) and clause (g)

of rule 3 shall hold office as such for a period of five years commencing

from  the  date  on  which  his  appointment  is  notified  in  the  Official

Gazette.

Provided  that  where  the  appointment  of  the  successor  of  any

such member has not been notified in the Official Gazette on or before

the  expiry  of  the  said  period  of  five  years,  such  member  shall,

notwithstanding the expiry of the period of his office, continue to hold

such  office  until  the  appointment  of  his  successor  is  notified  in  the

Official Gazette.

(5)

If  a  member  is  unable  to  attend  a  meeting  of  the  Committee,  the

Chairman  may,  after  notice  in  writing  to  such  member,  nominate  a

substitute of such member to attend the meeting and such a substitute

member  shall  have  all  the  rights  and  privileges  of  such  member  in

respect of that meeting.

(6)

The  State  Advisory  Committee  shall  be  reconstituted  after  every  five

years.

5.      Resignations        4

(1)

A  member  of  the  State  Advisory  Committee,  not  being  an  ex  officio

member, may resign his office by a letter in writing addressed to the

State Government through its Secretary, Labour with prior information

to the Chairperson of such Committee.

(2)

The seat of such a member shall fall vacant from the date on which his

resignation  is  accepted  by  the  State  Government,  or  on  the  expiry  of

thirty days from the date of receipt of the letter of resignation  by State

Government ,whichever is earlier.

6.

Cessation of membership Sec. 4

If any member of the State Advisory Committee not being an ex-officio

member,  fails  to  attend  three  consecutive  meetings  of  such  Committee,

without  obtaining  the  leave  of  the  Chairperson  of  such  Committee  for  such

absence, he shall cease to be a member of such Committee:

10

Provided  that  the  State  Government  may,  if  it  is  satisfied  that  such

member was prevented by sufficient cause from attending three consecutive

meetings, direct that such cessation shall not take place and on such direction

being made, such member shall continue to be a member of such Committee.

7.

Disqualification on for membership Sec. 4

(1)

A person shall be disqualified for being a member of the State Advisory

Committee--

(i)

if he is of unsound mind and stands so declared by a competent

court;

(ii)

if he is an undischarged insolvent; or

(iii)

If he has been convicted of an offence which, in the opinion of

the State Government involves moral turpitude.

(2)     Where  a  question  arises  as  to  whether  a  disqualification  has  been

incurred  under  sub-rule  (1),  the  State  Government  shall  decide  such

question

8.

Removal from membership Sec. 4

The  State  Government  may  remove  from  office  any  member  of  the

State  Advisory  Committee,  if  in  its  opinion  such  member  has  ceased  to

represent the interest which he purports to represent on such Committee.

Provided  that  no  such  member  shall  be  removed  unless  a  reasonable

opportunity is given to him of making a representation against the proposed

action under this rule.

9.

Manner of filling vacancies Sec. 4

When a vacancy occurs or is likely to occur in the membership of the

State Advisory Committee, the Chairperson of such Committee shall submit a

report  to  the  State  Government  and  on  receipt  of  such  report,  the  State

Government  shall  take  steps  to  fill  the  vacancy  by  making  an  appointment

from  amongst  the  category  of  persons  to  which  the  person  vacating

membership  belonged  and  the  person  so  appointed  shall  hold  office  for  the

remainder  of  the  term  of  office  of  the  member  in  whose  place  he  is

appointed.

10.

Staff of State Advisory Committee Sec. 4

11

(1)

The  State  Government  may  appoint  one  of  its  officers  not  below  the

rank  of  Labour  Commissioner    as  Secretary  to  the  State  Advisory

Committee  and  appoint  such  other  staff  being  in  the  service  of  that

Government, as it may think necessary, to enable such Committee to

carry out its functions.

(2)     The  remuneration  payable  to  such  staff  shall  be  such  as  may  be

decided by the State Government from time to time.

11

Duties of Secretary Sec. 4

The Secretary of the State Advisory Committee-

(i)

shall assist the Chairperson of such Committee in convening meetings

of the Committee;

(ii)

may attend the meetings of such Committee but shall not be entitled

to vote at such meetings.

(iii)

shall keep a record of the minutes of the meetings of such Committee;

and

(iv)

shall take necessary measures to carry out the decisions taken at the

meetings of such Committee.

12.

Allowances of members Sec. 4

(1)

The  traveling  allowance  of  an  official  member  of  the  State  Advisory

Committee shall be governed by the rules applicable to him for journey

performed by him on official duties and shall be paid by the authority

paying his salary.

(2)

The  non  official  members  of  the  State  Advisory  Committee  shall  be

paid traveling allowance for attending the meeting of such Committee

at such rates as are admissible to a Grade-1 officer of the State.

13.

Disposal of business Sec. 4

(1)

Every  matter  which  the  State  Advisory  Committee  is  required  to  take

into consideration shall be considered at a meeting of that Committee,

or  if  the  Chairperson  of  such  Committee  so  directs,  by  sending  the

necessary papers to every member for opinion, and the matter shall be

disposed of in accordance with the decision of the majority:

12

Provided that where there is no opinion of majority on a matter

and  the  members  of  such  Committee  are  equally  divided,  the

Chairperson of such Committee shall have a second or a casting vote.

Explanation.-

The expression “Chairperson of the State Advisory

Committee” for the purpose of this rule shall include the Chairperson of

such Committee nominated or chosen under sub-rule (2) of rule 15 to

preside over a meeting.

(2)

No act or proceedings of the State Advisory Committee shall be invalid

merely  for  reasons  of  any  vacancy  in  or  any  defect  in  constitution  of

the Committee.

14.

Meetings Sec. 4

(1)

The  State  Advisory  Committee  shall  meet  at  such  places  and  at  such

times as may be decided by the Chairperson of such Committee and it

shall meet at least once in six months.

(2)

The  Chairperson  of  such  Committee  shall  preside  over  every  meeting

of  the  Committee  in  which  he  is  present  and  in  his  absence  he  may

nominate a member of the Committee to preside over such a meeting

in his place and in the absence of such nomination by the Chairperson,

the members of such Committee present in such meeting may choose

from amongst themselves a member to preside over such a meeting.

15.

Notice of meetings and list of business Sec. 4

(1)

Ordinarily two weeks notice shall be given to the members of the State

Advisory Committee of a proposed meeting:

Provided  that  the  chairperson  of  such  Committee,  if  he  is

satisfied that it is expedient to do so, may give notice of longer period

for such meeting which shall not exceed one month.

(2)     No  business  except  which  is  included  in  the  list  of  business  for  a

meeting  of  such  Committee  shall  be  considered  at  such  meeting

without the permission of the Chairperson of the Committee.

16.    Quorum        4

No  business  shall  be  transacted  at  any  meeting  of  the  State  Advisory

Committee unless at least six members of such Committee are present in that

meeting which shall include at least one member of State Legislature:

13

Provided  that  if  in  any  meeting  of  such  Committee  less  than  six

members  are  present,  the  Chairperson  of  such  Committee  may  adjourn  the

meeting to another date informing members present and giving notice to the

other members that he proposes to dispose of the business at the adjourned

meeting whether there is prescribed quorum or not, and it shall thereupon be

lawful  for  him  to  dispose  of  the  business  at  the  adjourned  meeting

irrespective of the number of members attending.

CHAPTER-III

(Part-I)

Registration of Establishments

17.

Manner of making application for registration of establishments Sec.

7

(1)

The  application  referred  to  in  sub  section  (1)  of  section  7  of  the  Act

shall  be  made  in  triplicate,  in  Form-1  annexed  to  these  rules  to

registering officer of the area appointed under section 6 of the Act in

which the building or other construction work is to be carried on by the

establishment.

(2)

Every  application  referred  to  in  sub-rule  (1)  shall  be  accompanied  by

treasury receipt showing payment of the fees for the registration of the

establishment.

(3)

Every  application  referred  to  in  sub-rule  (1)  shall  be  either  personally

delivered to the registering officer or sent to him by registered post.

(4)

On receipt of the application referred to in sub rule (1), the registering

officer  shall,  after  noting  thereon  the  date  of  receipt  by  him  of  the

application, grant an acknowledgement to the applicant.

18.

Grant of certificate of registration Sec. 7

(1)

The Registering Officer after receiving application under sub rule(1) of

rule  17  shall  register  the  establishment  and  issue  a  certificate  of

registration to the applicant within 30 days of receipt of application if

such  applicant  has  complied  with  all  the  requirement  as  laid  down  in

14

these  rules  and  has  made  the  application  within  such  period  as

specified under clause (a) and clause (b) of sub-section (1) of section 7

of  the  Act.  The  certificate  of  registration  to  be  granted  by  the

registering officer shall be in Form-II annexed to these rules.

(2)

The  registering  officer  shall  maintain  a  register  in  Form-III  annexed

to  these  rules  showing  the  particulars  of  establishments  in  relation  to

which certificates of registration have been issued by him.

(3)

If, in relation to an establishment, any change occurs in the ownership

or  management  or  other  particulars  specified  in  the  certificate  of

registration,  the  employer  of  the  establishment  shall  intimate  the

registering officer, within thirty days from the date when such change

takes place, the date and particulars of such change, and the reasons

thereof.

19.

Payment of additional fees and amendment of register etc Sec. 7

(1)

Where  on  receipt  of  the  intimation  under  sub-rule  (3)  of  rule  18  the

registering officer is satisfied that an amount higher than the amount,

which has been paid by the employer as fees for the registration of the

establishment  is  payable,  he  shall  require  such  employer  to  pay

additional sums which, together with the amount already paid by such

employer,  would  be  equal  to  such  higher  amount  of  fees  payable  for

the registration of the establishment.

(2)

Where, on receipt of the intimation referred to in sub-rule (3) of rule

18 , the registering officer is satisfied that there has occurred a change

in  the  particulars  of  the  establishment,  as  entered  in  the  register  in

Form-III annexed to these rules, he shall amend the said register and

record therein the change which has occurred:

Provided  that  the  registering  officer  shall  not  carry  out  any

amendment in the register in Form-III annexed to these rules unless

the appropriate fees have been deposited by the employer.

20.

Conditions of registration Sec. 7

(1)

Every certificate of registration issued under rule 18 shall be subject to

the following conditions, namely:-

(a)

the certificate of registration shall be non transferable;

15

(b)

the  number  of  workmen  employed  as  building  workers  in  an

establishment  shall  not,  on  any  day,  exceed  the  maximum

number specified in the certificate of registration; and

(c)

save  as  provided  in  these  rules,  the  fees  paid  for  the  grant  of

registration certificate shall be non refundable.

(2)

The  employer  shall  intimate  the  change,  if  any,  in  the  number  of

workmen  or  the  conditions  of  work  to  the  registering  officer  within

fifteen days.

(3)

The  employer,  shall,  before  thirty  days  of  the  commencement  and

completion of any building or other construction work, submit a written

notice  to  the  Inspector,  having  jurisdiction  in  the  area  where  the

proposed  building  or  other  construction  work  is  to  be  executed,

intimating the actual date of the commencement or, as the case may

be, completion of such building or other construction work in Form-IV

annexed to these rules.

(4)

The certificate of registration of an establishment shall be valid only for

such  building  and  other  construction  work  carried  out  by  such

establishment  for  which  intimation  required  under  sub  rule  (3)  has

been given.

(5)

A  copy  of  the  certificate  of  registration  shall  be  displayed  at  the

conspicuous  place  at  the  premises  where  the  building  and  other

construction work is being carried on.

21. Fees Sec. 7

(1)

The fees to be paid for the grant of a certificate of registration under

rule 19  shall be as specified below, namely:

If the number of workers proposed to be employed as building

workers, for a building or other construction work on any day;

100

2000.00

(b)

exceeds 100 but does not exceed 500

Rs.  5000.00

Exceeds

Rs.10,000.00

16

CHAPTER III

(Part-II)

Appeals, Copies of Orders, Payment of Fees, etc.

22.

Filing of appeals before the appellate officer Sec. 9

(1)      Every  appeal  under  sub-section  (1)  of  section  9  of  the  Act  shall  be

preferred  in  the  form  of  a  memorandum  signed  by  the  aggrieved

person  or  his  authorized  advocate  and  presented  to  the  appellate

officer in person or sent to him by registered post.

(2)

The  memorandum  shall  be  accompanied  by  a  certified  copy  of  the

order appealed against and treasury receipt for rupees one hundred.

(3)

The  memorandum  shall  set  forth  concisely  and  under  distinct  heads

the grounds of appeal.

(4)    Where  the  memorandum  of  appeal  does  not  comply  with  the

provisions  of  sub-rule  (2)  and  sub-rule  (3)  it  may  be  returned  to

appellant for the purpose of being amended within a time to be fixed

by  the  appellate  officer  which  shall  not  exceed  thirty  days  from  the

date on which the order appealed against has been communicated to

the appellant.

(5)

Where  the  memorandum  of  appeal  is  in  order,  the  appellate  officer

shall  admit  the  appeal,  endorse  thereon  the  date  of  hearing  of  such

17

appeal,  and  shall  register  the  appeal  in  a  book  to  be  kept  for  the

purpose called the register of appeals.

(6)

(i)

When  the  appeal  has  been  admitted,  under  sub  rule  (5)  the

appellate  officer  shall  send  the  notice  of  the  appeal  to  the

registering  officer  against  whose  order  the  appeal  has  been

preferred  and  the  registering  officer  shall  thereupon  send  the

record of the case to the appellate officer.

(ii)

On receipt of the record, the appellate officer shall send a notice

to the appellant to appear before him at such date and time as

may be specified in the notice for the hearing of the appeal.

23.

Failure to appear on the date of hearing Sec. 9

If  on  the  date  fixed  for  hearing,  the  appellant  does  not  appear,  the

appellate  officer  may  dismiss  the  appeal  for  default  of  appearance  of  the

appellant.

24.

Restoration of appeals Sec. 9

Where an appeal has been dismissed under rule 23, the appellant may

apply  to  the  appellate  officer  for  the  restoration  of  the  appeal  and  if  the

appellate  officer  is  satisfied  that  the  appellant  was  prevented  by  sufficient

cause  from  appearing,  the  appellate  officer  shall  restore  the  appeal  on  its

original number :

Provided that an application for restoration under this rule shall not be

entertained  by  the  appellate  officer  after  thirty  days  from  the  date  of    such

dismissal.

25.

Hearing of appeals Sec. 9

(1)

If the appellant is present when the appeal is called on for the hearing,

the  appellate  officer  shall  proceed  to  hear  the  appellant  or  his

authorized   advocate   and   pass   an   order   on   the   appeal,   either

confirming, reversing or varying the order appealed against.

(2)     The   order   of   the   appellate   officer   shall   state   the   points   for

determination, the decisions thereon and reasons for such decisions.

(3)

The  order  shall  be  communicated  to  the  appellant  and  copy  thereof

shall be sent to the registering officer against whose order the appeal

has been preferred.

18

26.

Copy of the order of the registering or order in appeal Sec. 9

Copy  of  the  order  of  the  registering  officer  or  of  the  appellate  officer

may be obtained by the person concerned or a person authorized by him on

payment of fees of one hundred rupees for each order on making application

to  the  registering  officer  or  the  appellate  officer,  as  the  case  may  be,

specifying  the  date  and  other  particulars  of  the  order  made  by  the  officer

concerned.  A  copy  of  the  certificate  of  registration  on  loss  or  mutilation  of

such certificate may also be obtained in the like manner and on payment of

like fees.

27.

Payment of fees Sec. 9

All  amounts  of  money  payable  on  account  of  registration,  appeal,

supply of copies or duplicate copies of certificate of registration shall be paid

in the local treasury under the head of account number, as may be specified

in this behalf and receipt obtained shall be submitted with the application or

memorandum of appeal , as the case may be in original.

CHAPTER-IV

Registration of Building Workers as Beneficiaries

28.

Membership Sec. 12

(1)

Every  building  workers  who  has  completed  eighteen  years  of  age  but

has not completed sixty years of age and who is not a member in any

other  welfare  fund  established  under  any  law  for  the  time  being  in

force  and  who  has  completed  ninety  days  of  service  as  a  building

worker  in  the  year  immediately  preceding  shall  be  eligible  for

membership in the Fund.

(2)

A  certificate  to  prove  age  as  specified  below,  shall  also  be  submitted

alongwith the application:-

(i)     school

(ii)

certificate from the Registrar of Births and Deaths;

(iii)     In  the  absence  of  the  above  certificates,  a  certificate  from  a

Medical  Officer  not  below  the  rank  of  a  Medical  Officer  in

Government service.

(3)     Certificate  from  the  employer  or  contractor  that  the  applicant  is  a

construction  worker  shall  be  produced  alongwith  the  application  for

19

registration.  In  case  such  a  certificate  is  not  available,  a  certificate

issued  by  the  registered  construction  workers  unions  or  a  certificate

issued  by  Labour  Officer  or  Assistant  Director,  Industrial  Safety  and

Health  of  the  concerned  area  or  by  the  Executive  Officer  of  the

Panchayat may also be considered.

(4)

Every building worker eligible to become a beneficiary to the Fund shall

submit  an  application  in  Form  V  to  the  Secretary  or  to  an  officer

authorized  by  him  in  this  behalf.  Every  such  application  shall  be

accompanied  by  the  documents  mentioned  in  this  Rule  and  a

registration fee of twenty five rupees.

(5)

Where the Secretary or an Officer authorized by him, is satisfied that

the  applicant  fulfils  the  conditions,  such  building  worker  shall  be

registered as a member.

(6)

Any person may within thirty days, file an appeal to the Board against

the  decision  taken  under  sub-Rule  (5)  and  the  decision  of  the  Board

thereon shall be final.

(7)     The  building  worker  shall  also  file  a  nomination  in  Form VI.  The

nomination  shall  stand  revised  in  the  name  of  the  spouse  on  his

acquiring  a  family  or  on  the  happening  of  any  legal  change  in  the

status of the family.

(8)

The  Secretary  or  other  officer  authorized  by  him  in  this  behalf  shall

issue  to  every  beneficiary  an  identity  card  with  a  photo  of  the

beneficiary  affixed  in  Form   VII  and  maintain  a  register  of  identity

cards so issued in Form  VIII.

29.

Contribution to the Fund Sec. 16

(1)

A beneficiary of the fund shall contribute to the Fund five rupees per

mensem. This contribution shall be remitted in advance once in three

months  in  any  of  the  banks  specified  by  the  Board  in  the  district  in

which the member resides.

(2)     If  a  beneficiary  commits  default  in  the  payment  of  contribution

continuously for a period of one year, he shall cease to be a beneficiary

of  the  fund.    However,  with  the  permission  of  the  Secretary  or  an

officer  authorized  by  him  in  this  behalf  the  membership  may  be

resumed  on  repayment  of  arrears  of  contribution  with  a  fine  of  two

20

rupees per month subject to the condition that such resumption shall

not be allowed more than twice.

30.

Duty of the employer to file returns Sec. 62

(I)

Every  employer  shall,  within  fifteen  days  from  the  commencement  of

these Rules send to the Secretary a consolidated return containing the

particulars  of  the  building  workers  entitled  to  be  registered  showing

their basic wages, allowances, and the amount being spent for the free

supply of food, if any.

(2)

Every employer shall, before the fifteenth day of every month send to

the  Secretary  or  any  other  officer  authorized  by  him  in  this  behalf  a

return  in  Form   IX  showing  the  details  of  the  workers  entitled  to  be

registered  as  well  as  those  who  left  the  service  during  the  preceding

month.

(3)     Every  employer  shall  furnish  to  the  Secretary  or  any  other  Officer

authorized by him in this behalf, in Form  X particulars regarding the

branches,  Directors,  Managers,  Occupiers,  Partners,  Person/persons

who has/have the ultimate control over the affairs of his establishment.

31.

Maintenance and production of records and registers Sec. 62

(I)

Every employer shall maintain a Register showing the particulars of the

building workers and a Register of contribution in such form as may be

directed by the Secretary or other officer authorized by him.

(2)     Every  employee  shall  whenever  the  Secretary  or  any  other  officer

authorized by him requires in person or by notice in writing to produce

the records in respect of the building worker, shall deliver such records

to the officer concerned in time and if the records are not returned he

shall issue a receipt for the records so retained by him.

in any existing Fund Sec. 62

(I)  If  a  worker  who  becomes  a  member  of  this  Fund,  the  concerned

authorities shall transfer such deposits in the name of that member of

this Fund.

(2)

The authority of the other welfare fund shall, furnish to the Secretary

or  any  other  officer  authorized  by  him  in  this  behalf  a  statement

21

showing  the  total  accumulation  in  the  credit  of  every  member  on  the

date of transfer under sub-Rule (I) and the amount of advance if any

taken by the member.

CHAPTER V

The Haryana Building and Other Construction Workers’ Welfare Board

33.    Definitions        62

(1)

In this chapter, unless the context otherwise requires,-

(a)     “Board”  means  the  Haryana  Building  and  Other  Construction

Workers’ Welfare Board constituted under section 18 of the Act.

(b)

“Contribution” means the sum of money payable to the fund by

the beneficiary;

(c)     “Family”    means  the  husband  or  wife,  minor  sons  including

physically handicapped ( who is unable to earn)  and unmarried

daughters  of  the  building  worker  and  the  parents  solely

dependant on him.

22

(d)     “Fund”  means  the  Haryana  Building  and  Other  Construction

Workers’  Welfare  Fund  constituted  by  the  Board  under  section

24 of the Act.

(e)

“Secretary”  means  the  Secretary  of  the  Board  appointed  under

section 19 of the Act;

(f)

“Year” means a financial year.

34.

Term of office of Chairperson and members of Board Sec 18

The  term  of  office  of  the  chairperson  and  the  members  of  the  Board

other  than  official  members,  shall  be  five  years  from  the  date  of  their

appointment.

35.

Filling up of casual vacancies Sec. 18

A  member  nominated  to  fill  a  casual  vacancy  shall  hold  office  for  the

remaining  period  of  the  term  of  office  of  the  member  in  whose  place  he  is

nominated.

36.

Meeting of the Board Sec. 20

The Board shall ordinarily meet once in three months;

Provided  that  the  chairperson  shall,  within  fifteen  days  of  the

receipt  of  a  requisition  in  writing  from  not  less  than  one  third  of  the

members of the Board, call a meeting thereof.

37.

Notice of the meeting and list of business Sec. 20

Notice intimating the date, time and venue of every meeting together

with  a  list  of  business  to  be  transacted  at  the  meeting,  shall  be  sent  by

registered post or by special messenger, to each member fifteen days before

the meeting:

Provided that when the chairperson calls a meeting for considering any

matter which in his opinion is urgent, notice of not less than three days shall

be deemed sufficient.

38.

Chairperson to preside at meeting Sec. 20

(1)

The chairperson shall preside over every meeting of the Board in which

he is present, and if, for any reason the chairperson is unable to attend

23

the meeting any member nominated by the Chairperson in this behalf

shall preside over the meeting.

(2)

When the chairperson is absent and no member has been nominated

by the chairperson under sub-Rule (1), the members present shall elect

one of them to preside over the meeting and the member so elected

shall  exercise  all  the  powers  of  the  Chairperson  in  conducting  the

meeting.

(3)

No business shall be transacted at any meeting of the Board unless, at

least,  six  members  are  present,  of  whom  one  shall  be  from  amongst

nominated members.

39.

Absent from the State Sec. 20

If any member leaves the state for a period of not less than six months

without  intimation  to  the  chairperson,  he  shall  be  deemed  to  have  resigned

from the Board.

40.

Transaction of business Sec. 20

Every question considered at a meeting of the Board shall be decided

by  a  majority  of  the  votes  of  the  members  present  and  voting  and  in  the

event  of  any  equal  votes,  the  chairperson  shall  have  and  exercise  a  casting

vote.

41.

Minutes of the meeting  Sec. 20

Every decision taken at a meeting of the Board shall be recorded in a

minutes  book  at  the  same  meeting  and  signed  by  the  chairperson.  The

minutes book shall be a permanent record.

42.

Fees and Allowance Sec. 18

(1)

Every  non  official  member  of  the  Board  shall  be  paid  a  sitting  fee  of

hundred rupees or amount as may be fixed by the State Government

from time to time, for attending a meeting of the Board. This fee shall

not be applicable for sub-committee meetings.

(2)

The  chairperson  shall  be  paid  a  sitting  fee  of  six  hundred  rupees  or

amount as may be fixed by the State Government from time to time,

for attending the meeting of the Board.

43.

Appointment of Sub-committee Sec. 22

24

The Board may appoint such sub-committees as it may deem fit for the

proper  discharge  of  its  duties  and  having  non-official  member  of  such  sub-

committee  shall  be  allowed  traveling  allowance  and  daily  allowance  for

attending the meeting of the sub-committee at such rates as are admissible

to Grade-1 of the State Government.

44.

Constitution of Sub-committee Sec. 22

(1)

The committee shall consist of the following persons, namely:-

(a)

The Chairperson of the Board;

(b)

One member representing the employers;

(c)

One member representing the building workers;

(d)

Two members representing the Government, not below the rank

of Deputy Labour Commissioner.

(2)     The  chairperson  of  the  Board  shall  be  the  chairperson  of  the  sub-

committee also. If the Chairperson is absent at any time the members

present shall elect one among them to preside over the meeting.

(3)

No  business  shall  be  transacted  at  a  meeting  of  the  sub-committee

unless at least three members of the committee are present of whom

one  shall  be  from  the  members  representing  employers  and  another

one shall be from the members representing building workers.

(4)

The term of the sub-committee shall be one year from the date of its

constitution:

Provided that the sub-committee shall continue in office until a

new committee is constituted:

Provided  further  that  in  no  case,  the  sub-committee  shall

continue  beyond  a  period  of  two  years  from  the  date  of  its  original

constitution.

(5)

The recommendation of the sub-committee shall be placed before the

Board for its decision.

45.

Powers, duties, and functions of Board Sec. 22

(1)

The Board shall be responsible for,-

(a)

all matters connected with the administration of the Fund;

(b)

laying down policies for the deposits of the amount of the Fund;

(c)

submission of annual budget to State Government for sanction;

25

(d)    submission  of  annual  report  to  State  Government  on  the

activities of the Board;

(e)

proper maintenance of accounts;

(f)

annual audit of accounts of the Board, in accordance  with

provisions of the Act;

(g)

collection of contribution to the Fund and other charges;

(h)

launching of prosecutions for and on behalf of the Board;

(i)

speedy settlement of claims and sanction of advances and other

benefits;

(j)

proper and timely recovery of any amount due to the  Board.

(2)     The  Board  shall  furnish  information  to  State  Government  on  such

matters as the State Government may refer to it, from time to time.

46.

Secretary to the Board Sec. 19

(1)

The Secretary to the Board shall be the Chief Executive Officer of the

Board.  The  secretary  to  the  Board  shall  exercise  full  administrative

control on the staff of the Board being its Chief Executive Officer.

(2)

The Secretary shall, with the approval of the Chairperson issue notice

to convene meetings of the Board and keep the record of minutes and

shall take necessary steps for carrying out the decisions of the Board.

47.

Appointment of Secretary and other officers Sec. 19

(1)

The  Labour  Commissioner  shall  be  the    Secretary  of  the  Board  and

any  other  gazetted  officers  may  be  appointed  as  one  or  more  Joint

Secretary by the Board who shall perform the duties under the control

and supervision of the Secretary of the Board as may be specified by

the Secretary.

(2)     The  Board  may,  with  the  prior  concurrence  of  the  Government,

appoint,-

(i)

as many officers of the Labour Department; and

(ii)

such other officers and employees of any other department  of

the Government, as it considers necessary to assist the Board in

the efficient discharge of its functions under the Act.

48.

Administrative and financial powers of Secretary Board Sec. 19

26

(1)     The  Secretary  of  the  Board  may,  without  reference  to  the  Board,

sanction  expenditure  and  contingencies,  supplies  and  services  and

purchase  of  articles,  refund  for  administering  the  Fund  subject  to  the

limits upto which he may be authorized to sanction expenditure on any

single item from time to time by the Board.

(2)

The secretary may also exercise such other administrative and financial

powers  other  than  those  specified  in  sub-Rule  (1)  above,  as  may  be

delegated to him from time to time by the Board.

(3)

The Board may, from time to time delegate, subject to such conditions

as  it  may  deem  fit,  administrative  and  financial  powers  to  any  other

officer  under  its  control  and  supervision  to  the  extent  considered

necessary for its efficient functioning.

4)

In the absence of holding of the meeting of the Board for any reason

whatsoever,  the  Chairman  shall  have  the  powers  to  approve  adhoc

budget  which  shall  be  deemed  to  be  the  budget  till  it  is  approved  by

the Board or not. This adhoc budget shall be signed by the Chairman,

Secretary and Joint Secretary, if any.

(5)

The Secretary of the Board shall have full power to hire a building for

the purpose of the Board on PWD assessed rates.

(6)

The  Secretary  of  the  Board  shall  have  full  powers  to  engage  services

on contract basis as per the requirement of the Board.

49.

The Haryana Building and Other Construction Workers’ Welfare Fund

Sec. 29

(1)

The Board may, as soon as may be after the coming into force of these

Rules,  constitute  a  Fund  by  the  name  “  The    Haryana  Building  and

Other  Construction  Workers’  Welfare  Fund”  in  accordance  with  the

provision of the Act and these Rules.

(2)

The Fund shall vest in and be administered by the Board.

(3)

There shall be credited to the Fund,-

(a)

grant or loan or advances if any, made by the  Government  of

India or by the Government or any local

(b)

the contribution paid by the beneficiaries under these  Rules;

(c)

all sums received by the Board from such other sources as may

be decided by the Central Government or the State Government.

27

50.

Maternity Benefit Sec. 62

The women employees who are beneficiary of the fund shall be given

Rs.  5000  each  as  maternity  benefit  during  the  period  of  maternity.    On  an

application made by her in Form XI with such other documents as may be

specified shall be submitted for this benefit:

Provided that this benefit shall not be allowed for more than twice.

51.

Eligibility for pension Sec. 62

A member of the fund who has been working as a building worker for

not  less  than  one  year  after  the  commencement  of  these  Rules  shall  on

completion  of  sixty  years  of  age  be  eligible  for  pension.    The  pension  will

become payable from the first day of the succeeding to the month in which

he completes sixty years of age.

52.

Procedure for payment of pension Sec. 62

(I)

An  application  for  pension  shall  be  submitted  in  Form XII  to  the

Secretary of the Board or the officer authorized by him for the purpose.

(2)

If in the opinion of the Secretary of the Board or the officer authorized

by  him  the  applicant  is  eligible  for  pension,  he  shall  sanction  pension

and send the pension sanctioning order to the applicant

Provided  that  no  application  shall  be  rejected  unless  the

applicant has been given an opportunity of being heard.

(3)     If  it  is  found  that  the  applicant  is  not  eligible  for  pension,  the

application shall be rejected, and the applicant informed accordingly.

(4)

The applicant may file an appeal before the Board against the decision

taken under sub-Rule (3) within sixty days from the date of the receipt

of the order;

Provided  that  the  Board  may  for  sufficient  reason  in  writing

condone the delay upto one year in filing the appeal.

(5)

The amount of pension shall be five hundred rupees per mensem.  An

increase  of  ten  rupees  shall  be  given  for  every  completed  year  of

service beyond five years.  The board may with the previous approval

of the Government revise the pension.

(6)

The pension sanctioning authority shall  maintain  a  register in Form

XIII.

28

53.

Advance for purchase of construction of house Sec. 62

(I)

The Board may on application by a member, sanction an amount not

exceeding  one  lakh  rupees  as  advance  for  the  outright  purchase  of  a

house  or  for  the  construction  of  house.    The  beneficiary  shall  along

with the application in Form XIV produce such documents as may be

specified by the Board.

(2)

No advance under sub-Rule (I) shall be sanctioned to those who do not

have  membership  in  the  fund  continuously  for  five  years  and  having

fifteen years service for superannuation.

(3)     A  completion  certificate  shall  be  submitted  to  the  secretary  of  the

Board  within  six  months  from  the  date  of  drawl  of  advance.    The

amount sanctioned as advance shall be recovered in equal installments

as may be fixed by the Board.

54.

Disability pension Sec. 62

(I)     The  Board  may  sanction  an  amount  of  three  hundred  rupees  per

mensem  as  disability  pension  to  a  beneficiary  who  is  permanently

disabled  due  to  paralysis,  leprosy,  cancer  ,  T  B,  accident  etc.    in

addition to the pension he will be eligible for an  exgratia payment of

not more than thirty thousand rupees depending upon the percentage

of  disability  and  subject  to  such  conditions  as  may  be  fixed  by  the

Board.

(2)

The application for disability pension and exgratia payment under sub

Rule  (  I)  shall  be  made  in  Form XV  with  such  certificate  and  other

documents as may be specified by the Board.

55.

Loan for purchase of Tools Sec. 62

An  amount  of  five  thousand  rupees  will  be  sanctioned  as  loan  to  the

members of the fund, for the purchase  of tools.  Those who have completed

three  years  membership  in  the  fund  and  those  who  remit  contribution

regularly  will  be  eligible  for  this  loan.    The  beneficiary  should  not  have

completed fifty five years of age.  The loan amount shall be recovered in not

more than sixty installments.  An application in Form XVI shall be made for

this loan with such other documents as may be specified by the Board.

56.

Payment of funeral assistant Sec. 62

29

The  Board  may  sanction  an  amount  of  five  thousand  rupees  to  the

nominees dependents of a deceased member, towards funeral expenses.  An

application in Form XVII shall be submitted for this benefit.

57.

Payment of Death Benefit Sec. 62

The Board may sanction an amount of seventy-five thousand rupees to

the  nominees/dependents  of  a  member  towards  death  benefit  in  case  of

death.  If the death is due to an accident, during the course of employment,

the nominee/dependents of the member shall be given one lakh twenty-five

thousand rupees towards death benefit.

58.

Application for Death Benefit Sec. 62

(I)

A nominee who is entitled to Death benefit under this Rule shall submit

to  the  Secretary  or  any  other  officer  authorized  under  this  Rule  shall

submit to the   Secretary  or any  other    officer  authorized by him an

application in Form XVIII. A certificate regarding the death/accident

death  issued  by  a  Government  doctor  not  below  the  rank  of  Medical

Officer   shall   be   produce   along   with   the   application   and   other

documents specified by the Board.

(2)

The  Secretary  or  the  officer  authorized  by  him  may  on  receipt  of  the

application  conduct  an  enquiry  with  regard  to  the  eligibility  of  the

applicant.

(3)

If  the  Secretary  or  the  officer  authorized  by  him  is  satisfied  that  the

person  who  has  applied  for  financial  assistance  is  entitled  for  such

benefit he may sanction the amount.

(4)

The  sanctioning  authority  shall  maintain  a  register  for  this  purpose  in

Form XIX.

(5)

A person aggrieved by any decision taken under sub Rule (3) may file

an appeal before the Board within sixty days from the date of receipt

of the order under that sub Rule and the decision of the Board thereon

shall be final

59.

Medical Assistant to the Beneficiary Sec. 62

The  Board  may  sanction  financial  assistance  to  the  beneficiaries  who

are  hospitalized  for  five  or  more  days  due  to  accident  or  any  disease.    The

financial  assistance  shall  be  two  hundred  rupees  for  the  first  five  days  and

30

twenty  rupees  each  for  the  remaining  days,  subject  to  a  maximum  one

thousand rupees.  This assistance shall also be given to the beneficiaries met

with accident and put in plaster at residence . If disability is resulted due to

accident,  the  worker  shall  be  eligible  for  a  financial  assistance  upto  a

maximum five thousand rupees depending upon the percentage of disability.

The  application  in  Form XX  or  XXI  shall  be  submitted  with  such  other

documents as may be specified by the Board.

60.

Financial Assistant for education Sec. 62

Children of the members shall be eligible for such financial assistance

as  may  be  determined  by  the  Board  for  such  courses  of  study  as  may  be

specified by the Board from time to time.   An application in Form XXII shall

be submitted with such documents and within such time as may be specified

by the Board.

61.

Financial Assistant for marriage Sec. 62

The  Building  workers  having  continuous  membership  for  three  years

shall  be  eligible  to  get  financial  assistance  of  two  thousand  rupees  for  the

marriage of their children.  A female member of this fund is also eligible for

this assistance for her own marriage.  This assistance shall be sanctioned for

the  marriage  of  two  children  of  the  beneficiary.    An  application  in  Form

XXIII  shall  be  submitted  along  with  such  other  documents  as  may  be

specified by the Board.

62.

Family Pension Sec. 62

In the event of death of a pensioner, family pension  shall be given to

the  surviving  spouse.    The  amount  of  pension  will  be  fifty  percent  of  the

pension  received  by  the  pensioner  or  one  hundred  rupees  which  ever  is

higher.    An  application  in  Form  XXIV  shall  be  submitted  with  such

documents  as  may  be  specified  by  the  Board  within  three  months  from  the

date of death of the pensioner.

63.

Recovery of Advances and loans Sec. 62

The Board shall have the power to stipulate the conditions for recovery

of loan and advances.

31

64.

Refund of the contribution of the deceased member Sec. 62

(I)

On the death of a member, the amount of contribution standing in his

credit shall be given to his nominee.  In the absence of a nominee the

amount shall be paid to his legal heirs in equal shares.

(2)

All  financial  benefits  under  these  Rules  other  than  death  benefit  and

medical    assistance for accidents shall become payable only after one

year of a person becoming member of the fund.

65.

Withdrawal to pay premium for Life Insurance Policy Sec. 62

(I)

Sanction can be accorded to a members of the fund for remitting the

premium  for  Life  Insurance  Policy  from  the  amount  standing  in  his

credit in the fund.  Withdrawal of amount for this purpose shall not be

allowed more than once in a year.

(2)

The full particulars of the policy shall be furnished to the Secretary of

the Board in such form as may be specified by him.

(3)     No  amount,  more  than  those  actually  required  for  remitting  the

premium, shall be sanctioned from the amount standing in this credit

of the member.

66.

Assignment of policy to the Fund Sec. 62

(I)

Within six months of the withdrawal of the amount the policy shall be

assigned  to  the  Secretary  of  the  Board  as  security  for  the  amount

withdrawn.

(2)     While  according  sanction  for  withdrawing  an  amount  for  remitting

premium  in  receipt  of  an  old  policy  the  secretary  of  the  Board  shall

ensure from the Life Insurance Corporation whether that policy is free

from any encumbrances.

(3)

No  changes  in  the  policy  on  transfer  to  another  policy  shall  be  made

without  the  previous  approval  of  the  Secretary  of  the  Board  and  the

particulars  regarding  the  changes  in  the  policy  or  the  transfer  to  the

new one shall be furnished to the Secretary of the Board in such form

as may be prescribed by him.

(4)     If  the  policy  is  not  so  assigned  and  entrusted,  the  member  shall

immediately  remit  to  the  Fund  any  amount  withdrawn  from  the  Fund

for the policy together with interest at such rate as may be fixed by the

Board in consultation with the Government.

32

67.

Returning of the policy Sec. 62

The  Board  shall  return  the  policy  in  the  following  circumstances,

namely -

(i)

on the member leaving service permanently on superannuation;

(ii)     on  leaving  service  permanently  due  to  physical  or  mental

disability;

(iii)

on the death of the member before he leaves the service;

(iv)

on the maturity of the policy assigned before the member leaves

service or on the member becoming entitled to receive payment

of the money in any other manner.

68.    Accounts Sec.

(I)     Excluding  the  administrative  expenses,  all  interest,  rent  and  other

income  realized  and  all  profits  or  losses,  if  any,  on  the  investment,

shall be credited or credited, as the case may be, as may be specified.

(2)

The secretary of the Board or any other officer authorized by him shall

submit  a  statement  to  the  Government  on  15th  Days  of  March  every

year or on such other date as the Government may specify an annual

report appending a classified statement of the assets of the Fund.

69.

Investment of amount Sec 24

All moneys belonging to the Fund may be invested in the Nationalized

Banks or Scheduled Banks or in the securities referred to in clauses (a) to (d)

of section 20 of the Indian Trusts Act, 1882 (2 of 1882).

70.

Utilization of Fund Sec. 24

The  Fund  shall  not,  without  the  previous  approval  of  Government  be

expended  for  any  purpose  other  than  those  mentioned  in  the  Act  and  the

rules.

71.

Expenditure from the Fund Sec. 24

(1)

All expenses for the administration of the fund, fees and allowances of

the  chairperson  and  members  of  the  Board  salaries,  leave  salaries,

joining  time  pay,  traveling  allowances,  compensatory  allowances,

charge allowances, pension contribution and other personnel expenses,

33

for  the  legitimate  needs  of  the  Board  and  the  stationery  expenses,

shall be met from the administrative account of the Fund.

(2)

The amounts incurred by the Government for the administration of the

Fund  shall  be  treated  as  a  loan  which  shall  be  repaid  from  the

administration account.

72.

Report regarding the functioning of the Board Sec. 27

A  report  on  the  functioning  of  the  Board  during  every  financial  year

shall be approved by the Board before the 15  day of June next year and be

submitted to the Government before the 31  day of July of that Year.

73.

Copies of registers and reports to be furnished  Sec. 62

The  Secretary  of  the  Board  shall  furnish  copies  of  the  registers  and

annual report of the fund to any employer or member of the fund on written

application and on payment of such fees as may be specified by the Board in

this behalf with the approval of the Government

74.

Recovery of arrears Sec. 62

If  any  amount  due  from  an  employer  or  a  member  is  in  arrears,  the

Secretary  of  the  Board  or  any  other  officer  authorized  by  him  in  this  behalf

shall,  after  ascertaining  the  amount  of  arrears,  issue  a  certificate  for  that

amount to the Collector of the District concerned.  On receipt of the certificate

the District Collector shall recovery the amount in the same manner as arrears

of public revenue due on land.

75.

Execution of Contracts Sec. 22

All  orders  and  other  instruments  shall  be  made  and  executed  in  the

name of the Board and shall be authenticated such persons as the Board may

specify.

34

CHAPTER –VI

(Part-I)

Hours Of Work, Welfare, Payment Of Wages, Registers and Records, Rest

Intervals and Weekly Off. Etc.

76.

Hours of work, interval of rest and spread over etc. Sec. 28

(1)

No  building  worker  employed  in  building  or  other  construction  work

shall be required or allowed to work  for more than nine hours a day or

forty eight hours a week.

(2)      No  building  worker  employed  in  building  or  other  construction  work

shall  be  required  or  allowed  to  work  continuously  for  more  than  five

hours unless he had an interval of rest of not less than half an hour.

(3)      The  working  day  of  a  building  worker  employed  in  building  or  other

construction work shall be so arranged that inclusive of the intervals of

rest, if any, shall not spread over more than twelve hours on any day.

(4)

When  a  building  worker  works  in  any  building  or  other  construction

work for more than nine hours on any day or for more than forty eight

hours in any week, he shall, in respect of overtime work, be entitled to

wages at double the ordinary rate of wages.

77.

Weekly rest, payment for work done on the day of rest at overtime

rate etc Sec. 28

(1)    Subject  to  the  provisions  of  these  Rules,  each  building  worker

employed in building and other construction  work shall be allowed  a

day  of  rest  every  week(hereinafter  referred  to  as  the  rest  day  which

shall ordinarily be Sunday, but the employer may fix any other day of

the week as the rest day;

Provided  that  the  building  worker  shall  be  informed  of  the  day

fixed  as  the  rest  day  and  of  any  subsequent  change  in  such  rest  day

before  the  change  is  effected,  by  display  of  a  notice  to  that  effect  in

the  place  of  employment  at  the  place  specified  by    the  Inspector

having jurisdiction in this behalf.

(2)

No  building  worker  employed  in  building  or  other  construction  work

shall  be  required  or  allowed  to  work  on  a  rest  day  unless  he  already

35

had or will have substituted rest day for whole day on one of the five

days immediately before or after such rest day;

Provided that no substitution  shall be made which results in a

building worker working in building worker working for more than ten

days consecutively without a rest day for a whole day.

(3)

Where  a  building  worker  employed  in  building    or  other  construction

work has worked on a rest day and has been given a substituted rest

day  on  any  one  of  the  five  days    before  or  after  the  rest  day,  as

provided  in  sub-Rule(1)  and  sub-Rule  (2)  such  rest  day  shall,  for  the

purpose  of  calculating    the  weekly  hours  of  work,  be  included  in  the

week in which such substituted rest day occurs.

(4)

A building worker employed in building or other construction work shall

be  granted  wages  for  a  rest  day,  calculated  at  the  rate  applicable  to

the day preceding such rest day and in case he has worked on a rest

day and has been given a substituted rest day he shall be paid wages

for such rest day on which he worked, at the overtime rate and wages

for  such  substituted  rest  day  at  the  rate  applicable  to  the  day

preceding such substitute rest day.

Explanation-I. For the purpose of this Rule ”preceding day”  means

the last day preceding a rest day or substituted rest day, as the case

may  be,  on  which  a  building  worker  had  worked  and  where  such

substituted  rest  day  falls  on  a  day  immediately  after  such  a  rest  day,

such “preceding day” means the last  day preceding such rest day on

which such building worker had worked.

Explanation-II, For the purposes of this Rule , “week” shall mean a

period of seven days beginning at mid-night on a Saturday night.

78.

Night Shifts Sec. 28

Where  a  building  worker  employed  in  building  or  other  construction

work works on a shift which extends beyond midnight,-

(a)

a  rest  day  for  the  purposes  of  Rule  77  shall  mean  a  period  of

twenty-four  consecutive    hours  beginning  from  the  time  when

such shift ends;

36

(b)

the hours after midnight during which such building worker  has

worked shall be counted towards  the previous day; and;

(c)

the  following  day  shall  be  deemed  to  be  the  period  of  twenty-

four hours  beginning from the time when such  shift ends.

79.

Application of provisions of this chapter to certain classes of building

workers Sec. 28.

(1)

The  provisions  of  this  chapter  shall  apply  to  the  classes  of  building

workers specified under clauses (a) to (d) of sub-section (2) of Section

28, of the Act subject to the following namely:-

(a)

no  building  worker  employed  in  building  or  other  construction

work shall be required or allowed to work continuously for more

than  fifteen  hours  a  day  inclusive  of  intervals  of  rest  or  sixty

hours in a week;

Provided that intervals of rest not less than half-an hour

are  given  after  every  five  hours  of  a  continuous  work  as  laid

down in sub-Rule (2) of Rule 76.

(b)

no building worker employed in building and other construction

work  shall  be  required  or  allowed  to  work  for  more  than

fourteen consecutive days unless a rest of twenty four hours is

given for rest to such worker;

(2)

Where the working hours in respect of a building worker employed in

building  or  other  construction  work  have  exceeded  the  hours  of  work

as laid down in sub-Rule (1) of Rule 76 or where such worker has been

deprived of a rest day due to application of sub-Rule (1 ) of this Rule,

such  worker  shall  be  paid  at  double  the  rate  of  normal  wages  in

respect of the work done in excess of such daily or weekly hours and

for work done on such rest-day.

CHAPTER –VI

(Part-II)

Welfare of Building Workers

80.

Latrine and urinal accommodation Sec. 33 and 34

Latrines or urinals, as the case may be, required to be provided under

section 33 of the Act shall be of the types as specified below, namely:-

37

(a)

every  latrine  shall  be  under  cover  and  so  partitioned  off  as  to

secure privacy, and shall have a proper door and fastenings;

(b)

(i)      where  both    male  and  female  building  workers  are

employed, there shall be displayed outside each block of

latrines  or  urinals  a  notice  containing  there  in  ”For  Men

only” or “ For  Women only” as the case may be, written

in  the  language  understood  by  the  majority  of  such

workers;

(ii)

such  notice  shall  also  bear  the  figure  of  a  man  or  of  a

woman, as the case may be;

(c)     every  latrine  or  urinal  shall  be  conveniently  situated  and

accessible to building workers at all times;

(d)

every  latrine  or  urinal  shall  be  adequately  lighted  and  shall  be

maintained in a clean and sanitary condition at all times;

(e)

every  latrine or urinal  other than those connected with a flush

sewage system shall comply with the requirements of the public

health authorities;

(f)

Water shall be provided by means of a tap or otherwise  so as

to  be  conveniently  accessible  in  or  nearby  every  latrine  or

urinals;

(g)

The walls, ceilings and partitions of every latrine or urinals  shall

be white washed or colour washed once in every period of four

months.

81.

Canteens Sec. 37

(1)

In  every  place  wherein  not  less    than  two  hundred  and  fifty  building

workers  are  ordinarily  employed,  the  employer  of  such  building

workers shall provided an adequate canteen in the manner as specified

in this  Rule for the use of such building workers.

(2)

The  canteen,  referred  to  in  sub-Rule(I)  shall  consist  of  a  dining  hall

with  furniture sufficient to accommodate building  workers using such

canteen, a kitchen, store-room, pantry and washing places separately

for building workers and for utensils.

(3)

(i)

The  canteen  referred  to  in  sub-Rule  (1)  shall  be  sufficiently

lighted at all times when any person has access to it;

38

(ii)     the  floor  of  such  canteen  shall  be  made  of  smooth  and

impervious  material  and  inside  walls  of  such  canteen  shall  be

lime-washed  or  colour-  washed  at  least    once  in  every  six

months;

Provided  that  such  inside  walls  of  the  kitchen  of  such

canteen shall be lime-washed once in every three months.

(4)

(i)

The  precincts  of  the  canteen  referred  to  in  sub-Rule  (1)

shall be maintained in a clean and sanitary condition;

(ii)

waste  water  from  such  canteen  shall  be  carried  away  in

suitable  covered  drains  and  shall  not  be  allowed  to

accumulate  in the surroundings of such canteen;

(iii)

suitable  arrangements    shall  be  made  for  the  collection

and disposal of garbage from such canteen.

(5)

Building of the canteen referred to in sub-Rule(1) shall be situated at

the distance not less than fifteen point two meters from any  latrine or

urinal or any source of dust, smoke or obnoxious fumes.

82.

Foodstuff to be served in the Canteen Sec. 37

The foodstuffs and other items to be served in the canteen, referred to

in sub-rule (1) of rule 81 shall be in conformity with the normal dietary habits

of the building workers.

83.

Serving of tea and snacks at the work place Sec. 37

At a building or other  construction work where a work place is situated

at  a  distance  of  more  than  zero  point  two  kilo  meters  from  the  canteen

provided  under  sub-Rule  (1)  of  Rule  81  arrangement  shall  be  made  by  the

employer employing building workers at such place for serving tea and light

refreshment to such building workers at such place.

84.

Charges for foodstuff Sec. 37

(1)

The  charges  for  food  stuffs,  beverages  and  other  items  served  in  the

canteen provided under sub-Rule (1) of Rule 81 shall be based on “ no

profit  no  loss”  and  the  price  list  of  such  items  shall  be  conspicuously

displayed  in such canteen.

39

(2)     In  arriving  at  the  prices  of  items  referred  to  in  sub-Rule  (1),  the

following shall not be taken into consideration as expenditure, namely-

(a)

the rent for the land and building of such canteen;

(b)

the depreciation and maintenance charges for the building and

equipment provided in such canteen;

(c)

The  cost  of  purchase,  repairs  and  replacement  of  equipments

including  furniture,  crockery,  cutlery,  utensils  and  uniforms

provided to the employees of such canteen;

(d)

the  water  charges  and  other  charges  incurred  for  lighting  and

ventilation of such canteen; and

(e)

the interest on the amounts spent for providing and maintaining

furniture and other equipment for such canteen.

CHAPTER –VI

(Part-III)

Notice, Registers, Records And Collection Of Statistics.

85.

Notice of wages periods, etc. Sec. 30

(I)

Every employer shall cause to display at the conspicuous place of the

work  place  of  an  establishment  under  his  control,  notice  showing  the

rates of wages of the building workers working in such establishment,

hours of work of such worker, their wage periods, date of payment of

such wages, names and addresses of the Inspector having jurisdiction

to  such  establishment  and  date  of  payment  of  unpaid  wages  to  such

workers, in English. Hindi and in the local language understood by the

majority of such building workers.

(2)

A  copy  of  the  notice  referred  to  in  sub-Rule  (1)  shall  be  sent  to  the

Inspector having jurisdiction and whenever any change occurs relating

to facts contained in such notice, such change shall be communicated

by the employer  to such inspector.

86.

Notice of commencement and completion Sec, 46

(1)

Every  employer  shall  ,  at  least  thirty  days  before  the  commencement

of any  building or other construction work under his control, send or

cause to be sent to the Inspector having jurisdiction , a written notice

intimating the actual date of the commencement ,the probable date of

completion and other such particulars as referred to in sub-section (1)

40

of section 46 of the Act relating to such building or other construction

work in Form -IV, annexed  to these Rules.

(2)

Where  any  change  occurs  in  any  of  the  particulars  furnished  under

sub-Rule (1) the employer shall intimate such change to the Inspector

having jurisdiction within two days of such change.

(3)

Nothing contained in sub-Rule (1) shall apply in case of such class of

building  or  other  construction  work  as  the  State  Government  may  by

notification specify to be emergent work.

87.

Register of person employed as building workers Sec. 38

Every   employer   shall   maintain   in   respect   of   each   registered

establishment, where he employs building workers, a register in Form-XXV,

annexed to these Rules.

88.    Muster  roll,  wages  register,  deduction  register,  overtime  register

and  issue  of  wages  books  and  service  certificate  Sec.  30  read  with

Sec.62

(1)

Every  employer  shall,  in  respect  of  each  work  on  which  he  employs

building workers, maintain-

(a)

Muster  roll  and  register  of  wages  in  Form XXVI  and  Form-

XXVII, respectively, annexed to these Rules;

Provided  that  a  combined  register  of  wage-cum-muster

roll   in   Form-XXVIII,   annexed   to   these   Rules   shall   be

maintained  by  the  employer  where  the  wage  period  for  such

building worker is a fortnight or less;

(b)

a register of deductions for damage or loss, register of fines and

register  of  advances  in  Form XXIX, Form XXX and Form

XXXI respectively, annexed to these Rules:

(c)

a register of overtime in Form XXXII annexed to these Rules,

for recording therein the number of hours of and the wages paid

for, overtime work, if any.

(2)

Every  employer  shall,  in  respect  of  each  work  on  which  he  engages

building workers,-

41

(a)

issue where the wage period is one week or more, wage book

to  each  of  such  building  worker  in  Form XXXIII  annexed  to

these  Rules  to  such  building  workers  in  which  entries  shall  be

made at least a day prior to the disbursement of wages to them

;

(b)

issue  a  service  certificate  to  each  of  such  building  worker  in

Form XXXIV annexed to these Rules to such  building workers

on termination of his service  on account of completion of such

work or for any other reason;

(c)     obtain  signature  or  thumb  impression  of  each  such  building

worker against entries relating to him on the register of wages

or  muster  roll-cum-wages  register.  as  the  case  may  be,  and

such  entries  shall  be  authenticated  by  the  employer  or  his

authorized representative.

(3)

In  respect  of  an  establishment    to  which  the  Payment  of  Wages  Act,

1936  (  4  of  1936),  or  Minimum  Wages  Act,  1948(11  of  1948)  or  the

Contract  Labour  (Regulation  and  Abolition)  Act,  1970  (37  of  1970),

applies  the  following  registers  and  records  required  to  be  maintained

by an employer under any of such Acts or the Rules made thereunder,

shall  be  deemed  to  be  the  registers  and  records  maintained  by  the

employer under these Rules, namely:-

(a)      Muster

(b)

Register of wages;

(c)

register of deductions;

(d)

register of overtime;

(e)

register of fines;

(f)

register of advances; and

(g)

combined register of wages-cum-muster roll.

(4)

Notwithstanding anything contained in these Rules, where a combined

or alternative form, in lieu of any of form specified under these Rules,

is sought to be used by an employer to avoid duplication of work for

compliance  with  the  provisions  of  any  other  Act  or  the  Rules  framed

there-under  or  for  administrative  convenience,  such  combined  or

alternative  form  may  be  used  with  the  prior  approval  of  the  State

Government.

42

(5)

Every employer shall, display at the conspicuous place of the work site

where he employs building workers, an abstract of the Act and these

Rules  in  English  and  in  Hindi  and  in  a  language  understood  by  the

majority of such building workers.

(6)     Every  employer  shall  ensure  that  the  registers  and  other  records

required  to  be  maintained  under  the  Act    or  these    Rules,  are

maintained  complete  and  up-to  date  and  unless    otherwise  provided

for, are kept at an office or the  nearest convenient building within the

precincts of the concerned workplace.

(7)     The  registers  and  other  records  relating  to  an  establishment  and

required  to  be  maintained  under  the  Act  or  these  Rules,  shall  be

maintained legibly in English and in Hindi or in a language understood

by   the   majority   of   the   building   workers   employed   in   such

establishment.

(8)     Every  register  or  other  record  referred  to  in  sub-rule  (7)  shall  be

preserved  by  the  employer,  with  whom  such  register  or  other  record

belongs, in original for a period of three calendar years from the date

of last entry therein.

(9)

Every  register,  record  or  notice  maintained  under  the  Act,  or  these

Rules  shall  be  produced  or  caused  to  produced  by  the  employer

concerned  on  demand  before  the  Inspector  or  any  other  authority

under the Act or any other person authorized by the State Government

for such purpose.

(10)    In  case,  where  during  a  wage  period,  no  deduction  has  been  made

from  the  wage  of  a  building  worker  or  no  fine  has  been  imposed  on

such building worker or no overtime work has been performed by such

building worker or no payment has been made for overtime work  to

such  building  worker,  a  “nil”  entry  shall  be  made  against  such  wage

period  at  the  appropriate  place  in  the  relevant  register  maintained  in

Form XXIX, XXX, XXXI, XXXII as the case may be.

89.

Returns Sec. 62

Every  employer  of  a  registered  establishment  shall  send  annually  a

return relating to such establishment in duplicate in Form –XXXV annexed to

these Rules to the registering officer having jurisdiction so as to reach him not

43

later than the fifteenth February following the end of each calendar year with

a copy to the Inspector having jurisdiction.

CHAPTER VII 
		(Part-I)

Safety and Health General Provisions

90.

Excise Noise, Vibration etc, Sec. 40

An employer shall ensure at a construction site of a building or other

construction  work  that  adequate  measures  are  taken  to  protect  building

workers  against  the  harmful  effects  of  excessive  noise  or  vibration  at  such

construction site and the noise level in no case exceeds the limits laid down in

Schedule-I  annexed to these rules.

91.

Fire Protection Sec. 40

An employer shall ensure at a construction site of a building or other

construction work that—

(a)

such construction site is provided with—

(i)

fire extinguishing equipment sufficient to extinguish   any

probable fire at such construction site;

(ii)

an  adequate  water  supply  at  ample  pressure  as  per  national

standards;

(iii)     number   of   trained   persons   required   to   operate   the   fire

extinguishing equipment provided under sub-clause (i) of clause

(a) is properly maintained and inspected at regular intervals of

not  less  than  once  in  a  year  by  the  responsible  person  and  a

record of such inspections is maintained  ; and

44

(b)

in  case  of  every  launch  or  boat  or  other  craft  used  for  transport  of

building  workers  and  the  cabin  of  every  lifting  appliance  including

mobile   crane,   adequate   number   of   portable   fire   extinguishing

equipment of suitable type shall be provided at each of such launch or

boat or craft or lifting appliance.

92.

Emergency Action Plans Sec. 40

An employer shall ensure at a construction site of a building or other

construction  work  that  in  case  more  than  five  hundred  building  workers  are

employed  at  such  construction  site  emergency  action  plan  to  handle  the

emergencies like-

(a)

fire and explosion;

(b)

collapse of lifting appliances and transport equipment;

(c)

collapse of building, sheds or structures etc.;

(d)

gas leakage or spillage or dangerous goods or chemicals;

(e)

drowning of building workers, sinking of vesels; and

(f)

land  slides  getting  building  worker  buried,  floods,  storms,  and  other

natural  calamities,  is  prepared  and  submitted  for  the  approval  of  the

Director General.

93.

Fencing of Motors, etc Sec. 40

An employer shall ensure at a construction site of a building or other

construction work that—

(a)

all motors, cogwheels, chains, and friction gearing, flywheels, shafting,

dangerous and moving parts of machinery ( whether or not driven by

mechanical power) and steam pipes are securely fenced or lagged;

(b)

the fencing of dangerous parts of machinery is not removed while such

machinery is in motion or in use;

(c)

no part of any machinery which is in motion and which is not securely

fenced  is  examined,  lubricated,  adjusted  or  repaired  except  by  a

person skilled for such examination, lubrication, adjustment or repairs;

(d)

machine parts are cleaned when such machine is stopped;

(e)

when a machine is stopped for servicing or repairs, adequate measures

are taken to ensure that such machine does not re-start inadvertently.

94.

Lifting and carrying of excessive weights Sec. 40

45

An employer shall ensure at a construction site of a building or other

construction work that,--

(a)

no building worker lifts by hand or carries overhead or over his back or

shoulders  any  material  article,  tool  or  appliances  exceeding  in  weight

the maximum limits set out in the following Table:

Person                          Maximum

Adult-man                             55kg

Adult-woman                         30kg

Adolescent-male                     30kg

Adolescent-female

20kg

Unless  aided  by  any  other  building  worker  or  a  mechanical

device.

(b)

no  building  worker  aided  by  other  building  workers,  lift  by  hand  or

carry  overhead  or  over  their  back  or  shoulders,  any  material,  article,

tool or appliance exceeding in weight the sum total of maximum limits

set  out  for  each  building  worker  separately  under  clause  (a)  unless

aided by a mechanical device.

95.

Health and Safety Policy Sec. 40

(1)

(a)

Every  establishment  employing  fifty  or  more  building  workers

shall  prepare  a  written  statement  of  policy  in  respect  of  safety

and  health  of  building  workers  and  submit  the  same  for  the

approval of the Chief Inspector.

(b)

the  policy  referred  to  in  clause  (a)  shall  contain  the  following,

namely:-

(i)

the  intentions  and  commitments  of  the  establishment

regarding health, safety and environmental protection of

building workers;

(ii)     organizational

referred  to  in  clause  (a)  specifying  the  responsibility  at

different levels of hierarchy;

(iii)

responsibilities of the principal employer, contractor, sub-

contractor, transporter or other   agencies involved in the

building or other construction work;

46

(iv)

techniques and methods for assessment of risk to safety,

health and environmental and remedial  measures

therefore;

(v)

arrangements  for  training  of  building  workers,  trainers,

supervisors or other persons engaged in the construction

work;

(vi)

other  arrangements  for  making  the  policy  referred  to  in

clause (a), effective;

(c)

the  intention  and  commitment  referred  to  in  sub  clause  (i)  of

clause  (b)  shall  be  taken  into  account  in  making  decisions

relating   to   plant,   machinery,   equipment,   materials   and

placement of building workers.

(2)

A copy of the policy referred to in clause(a) of sub-rule (1), signed by

an authorized signatory shall be sent to the Chief Inspector.

(3)

The  establishment  shall  revise  the  policy  referred  to  in  clause  (a)  of

sub-rule(1)  as  often  as  necessary  under  the  following  circumstances,

namely:-

(i)

whenever  any  expansion  or  modification  having  implication  on

safety  and  health  of  the  building  workers  is  made  in  such,

building or other construction work; or

(ii)     whenever   any   new   building   or   other   construction   work,

substances,   articles   or   techniques   are   introduced   having

implication on health and safety of building workers.

(4)

A copy of the policy referred to in sub-clause (a) of sub-rule(1) shall be

displayed  at  the  conspicuous  places  in  Hindi  and  local  language

understood by the majority of building workers at a construction site.

harmful environment Sec. 40

An employer shall ensure at a construction site of a building or other

construction work that,-

(a)

When an internal combustion engine exhausts into a confined space or

excavation  or  tunnel  or  any  other  work  place  where  neither  natural

ventilation  nor  artificial  ventilation  system  is  adequate  to  keep  the

carbon  monoxide  content  of  the  atmosphere  below  fifty  parts  per

million, adequate and suitable measures are taken at such work place

in order to avoid exposure of building workers to health hazards.

47

(b)

No  building  worker  is  allowed  to  enter  any  confined  space  or  tank  or

trench  or  excavation  wherein  there  is  given  off  any  dust,  fumes  or

other  impurities  of  such  nature  and  to  such  extent  as  is  likely  to  be

injurious  or  offensive  to  the  building  worker  or  in  which  explosives,

poisonous, noxious or gaseous material or other harmful articles have

been  carried  or  stored  or  in  which  dry  ice  has  been  used  as  a

refrigerant,  or  which  has  been  fumigated  or  in  which  there  is  a

possibility  of  oxygen  deficiency,  unless  all  practical  steps  have  been

taken  to  remove  such  dust,  fumes,  or  other  impurities  and  dangers

which may be present and to prevent any further ingress thereof, and

such  work  place  or  tank  or  trench  or  excavation  is  certified  by  the

responsible  person  to  be  safe  and  fit  for  the  entry  of  such  building

workers.

97.

Overhead Protection Sec. 40

(1)

The  employer  shall  ensure  at  the  building  or  other  construction  work

that  overhead  protection  is  erected  along  the  periphery  of  every

building under construction which shall be of fifteen meters or more in

height when completed.

(2)

Overhead  protection  referred  to  in  sub-rule  (1)  shall  not  be  less  than

two  meters  wide  and  shall  be  erected  at  a  height  not  more  than  five

meters  above  the  base  of  the  building  and  the  outer  edge  of  such

overhead protection shall be one hundred fifty milli meters higher than

the inner edge thereof or shall be erected at an angle of not more than

twenty degrees to its horizontal sloping into the building.

(3)

The employer shall ensure at the building and other construction work

that  any  area  exposed  to  risk  of  falling  material,  articles  or  objects  is

roped  off  or  cordoned  off  or  otherwise  suitably  guarded  from

inadvertent  entry  of  persons  other  than  building  workers  at  work  in

such area.

98.

Slipping, tripping, cutting, drowning and falling hazards Sec. 40

(1)

Passageways,  platform  and  other  places  of  construction  work  at  the

building or other construction work shall be kept by the employer free

from accumulations of dust, debris or similar material and from other

obstructions that may cause tripping.

48

(2)

Any  sharp  projections  or  protruding  nails  or  similar  projections  which

may  cause  any  cutting  hazard  to  a  building  worker  at  the  building  or

other  construction  work  shall  be  removed  or  otherwise  made  safe  by

taking suitable measures by the employer.

(3)

No  employer  shall  allow  any  building  worker  at  building  or  other

construction  work  to  use  the  passageway,  or  a  scaffold,  platform  or

any  other  elevated  working  surface  which  is  in  a  slippery  and

dangerous  condition  and  shall  ensure  that  water,  grease,  oil  or  other

similar substances which may cause the surface slippery, be removed

or  sanded,  saw  dusted  or  covered  with  suitable  material  to  make  it

safe from slipping hazard at a building or other construction work.

(4)

Wherever building workers at a building or other construction work are

exposed to the hazard of falling into water, they shall be provided by

the  employer  with  adequate  equipment  for  saving  themselves  from

drowning  and  rescuing  from  such  hazard  and  if  the  Chief  Inspector

considers  necessary,  well-equipped  boat  or  launch  manned  with

trained personnel shall be provided by the employer at the site of such

work.

(5)

Every  open  side  or  opening  into  or  through  which  a  building  worker,

vehicle or lifting appliance or other equipments may fall at a building or

other  construction  work  shall  be  covered  or  guarded  suitably  by  the

employer to prevent such fall except where free accesses necessary by

reasons of the nature of the work.

(6)

Wherever building workers at a building or other construction work are

exposed to the hazards of falling from height while employed on such

work, they shall be provided by the employer with adequate equipment

or  means  for  saving  them  from  such  hazards.  Such  equipment  or

means shall be in accordance with the national standards.

(7)

Whenever there is a possibility of falling of any material, equipment or

building  worker  at  a  construction  site  relating  to  a  building  or  other

construction work, adequate and suitable safety net shall be provided

by employer in accordance with the national standards.

99.

Dust, gases, fumes etc. Sec. 40

An  employer  shall  prevent  concentration  of  dust,  gases  or  fumes  by

providing suitable means to control their concentration within the permissible

49

limit  so  that  they  may  not  cause  injury  or  pose  health  hazard  to  a  building

worker at a building or other construction work.

100.  Corrosive substances Sec. 40

The  employer  shall  ensure  that  corrosive  substances,  including  alkalis

and acids, shall be stored and used by a person dealing with such substances

at  a  building  or  other  construction  work  in  such  a  manner  that  it  does  not

endanger  the  building  worker  and  suitable  protective  equipment  shall  be

provided by the employer to a building worker during handling or use of such

substances at a building or other construction work and in case of spillage of

such  substances  on  the  building  worker,  immediate  remedial  measures  shall

be taken by the employer.

101.  Eye Protection Sec. 40

Suitable  protective  equipment  for  the  protection  of  eyes  shall  be

provided  by  an  employer  and  used  by  the  building  worker  engaged  in

operations like welding, cutting, chipping, grinding or similar operations which

may cause hazard to his eyes at a building or other construction work.

102.  Head Protection and other protection Sec. 40

(1)

Every  building  worker  required  to  pass  through  or  work  within  the

areas as building or other construction work where there is hazard of

his being struck by falling objects or materials shall be provided by the

employer  with  Safety  hamlets  of  type  and  tested  in  accordance  with

the national standards.

(2)

Every building worker required to work in water or in wet concrete or

in other similar work at a building or other construction work, shall be

provided with suitable water-proof boots by the employer.

(3)

Every  building  worker  required  to  work  in  rain  or  in  similar  wet

condition at building or other construction work, shall be provided with

water-proof coat with hat by the employer.

(4)

Every building worker required to use or handle alkalies, acid or other

similar  corrosive  substances  at  a  building  or  other  construction  work

shall  be  provided  with  appropriate  protective  equipment  by  an

employer, in accordance with the national standards.

50

(5)

Every  building  worker  engaged  in  handling  sharp  objects  or  materials

at a building or other construction work which may cause hand injury,

shall  be  provided  with  suitable  hand-gloves  by  the  employer,  in

accordance with the national standards.

103.  Electrical hazards Sec. 40

(1)

Before commencement of any building or other construction work, the

employer  shall  take  adequate  measures  to  prevent  any  worker  from

coming   into   physical   contact   with   any   electrical   equipment   or

apparatus, machines or live electrical circuit which may cause electrical

hazard  during  the  course  of  his  employment  at  a  building  or  other

construction work.

(2)

The  employer  shall  display  and  maintain  suitable  warning  signs  at

conspicuous  places  at  a  building  or  other  construction  work  in  Hindi

and  in  a  local  language  understood  by  the  majority  of  the  building

workers.

(3)

In workplaces at a building or other construction work where the exact

location of underground electric power line is not known, the building

workers using jack hammers, crow bars or other hand tools which may

come  in  contact  with  a  live  electrical  line,  shall  be  provided  by  the

employer with insulated protective  gloves and foot-wear of the type in

accordance with the national standards.

(4)

The employer shall ensure that, as far as practicable, no wiring, which

may  come  in  contact  with  water  or  which  may  be  mechanically

damaged, is left on ground or floor, at a building or other construction

work.

(5)

The  employer  shall  ensure  that  all  electrical  appliances  and  current

carrying  equipment  used  at  a  building  or  other  construction  work  are

made of sound material and are properly and adequately earthed.

(6)

The employer shall ensure that all temporary electrical installations at a

building  or  other  construction  work  are  provided  with  earth-leakage

circuit breakers.

(7)

The  employer  shall  ensure  that  all  electrical  installations  at  a  building

or other construction work comply with the requirements of any law for

the time being in force.

51

104.  Vehicular Traffic Sec. 40

(1)

Whenever any building or other construction work is being carried on,

or is located in close proximity to a road or any other place where any

vehicular  traffic  may  cause  danger  to  building  workers,  the  employer

shall ensure that such building or other construction work is barricaded

and  suitable  warning  signs  and  lights  displayed  or  erected  to  prevent

such danger and if necessary, he may make a request in writing to the

concerned authorities to control such traffic.

(2)

The employer shall ensure that all vehicles used at construction site of

a building or other construction work comply with the requirements of

the  Motor  Vehicles  Act,  1988  (  59  of  1988),  and  the  rules  made

thereunder.

(3)

The  employer  shall  ensure  that  a  driver  of  a  vehicle  of  any  class  or

description  operating  at  a  construction  site  of  a  building  or  other

construction work holds a valid driving licence under the Motor Vehicles

Act, 1988 ( 59 of 1988).

105.  Stability structures Sec. 40

The employer shall ensure that no wall, chimney or other structure or

part of a structure is left unguarded in such condition that it may fall, collapse

or  weaken  due  to  wind  pressure,  vibration  or  due  to  any  other  reason  at  a

site of a building or other construction work.

106.  Illumination pf passageways etc Sec. 40

The  employer  shall  ensure  that  illumination  sufficient  for  maintaining

safe  working  conditions  at  a  site  of  a  building  or  other  construction  work  is

provided  where  building  workers  are  required  to  work  or  pass  and  for

passageways  stairways  and  landing,  such  illumination  is  not  less  than  that

provided in the relevant national standards.

107.  Stacking of materials Sec. 40

The employer shall ensure, at a construction site of a building or other

construction work that-

(a)

all  building  materials  are  stored  or  stacked  in  a  safe  and  orderly

manner to avoid obstruction of any passageway or place of work;

52

(b)

material  piles  are  stored  or  stacked  in  such  a  manner  as  to  ensure

stability;

(c)

material of equipment is not stored upon any floor or platform in such

quantity as to exceed its safe carrying capacity;

(d)

material or equipment is not stored or placed so close to any edge or a

floor  or  platform  as  to  endanger  the  safety  of  persons  below  or

working in the vicinity.

108.  Disposal of debris Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that--

(a)

debris are handled and disposed of by a method which does not cause

danger to the safety of a person;

(b) 
(c)

debris are not allowed to accumulate so as to constitute a hazard;

debris  are  kept  sufficiently  moist  to  bring  down  the  dust  within  the

permissible limit;

(d)

debris  are  not  thrown  inside  or  outside  from  any  height  of  such

building or other construction work;

(e)

on  completion  of  work,  left  over  building  material,  article  or  other

substance  or  debris  are  disposed  of  as  soon  as  possible  to  avoid  any

hazard to any traffic or person.

109.  Numbering and marking of floors Sec. 40

The employer shall ensure that each floor or level of a building or other

construction work is appropriately numbered or marked at the landing of such

floor or level.

110.  Use of Safety helmets and shoes Sec. 40

The  employer  shall  ensure  that  all  persons  ;  who  are  performing  any

work or services at a building or other construction work, wear safety shoes

and helmets conforming to the national standards.

CHAPTER-VII

(Part-II)

Medical Facilities

53

111.  Medical examination of building workers, etc Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that –

(a)

(i)

a building worker who is employed for a work involving such risk

or  hazards  inherent  in  such  work  as  the  Chief  Inspector

considers  appropriate  for  the  periodical  medical  examination  of

such  workers,  is  medically  examined  at  such  intervals  as  the

Chief Inspector may direct from time to time;

(ii)     Every  operator  of  a  crane,  winch  or  other  lifting  appliance,

transport  equipment  or  vehicle,  is  medically  examined  before

employing   such   operator   and   again   periodically,   at   such

intervals at the Chief Inspector may direct from time to time;

(iii)

the  medical  examination  referred  to  in  sub-clause  (i)  and  sub-

clause(ii) is in accordance with Schedule-II, annexed to these

rules  and  is  conducted  by  such  medical  officers  or  at  such

hospitals  as  are  approved  by  the  State  Government  for  the

purpose from time to time;

(iv)    in  case  of  a  building  worker  who  is  exposed  to  special

occupational  health  hazard  owing  to  job  or  work  assigned  to

such worker, the periodical medical examination referred to  in

sub-clause(i) or sub-clause(ii) includes such special investigation

as may be deemed necessary by the construction medical officer

examining   such   building   worker   for   the   diagnosis   of   a

occupational disease ;

(b)

no building worker is charged for the medical examination referred to

in  sub-section(i)  or  sub  clause  (ii)  of  clause(a)  and  the  cost  of  such

examination  is  borne  by  the  employer  employing  such  building

workers.

(c)     certificate  of  medical  examination  referred  to  in  sub-clause  (I)  or

sub(ii) of clause(a)  is issued in Form-XXXVI annexed to these rules;

(d)

the record of the medical examination referred to in sub-clause (I) or

sub-clause(ii) of clause(a) of every building worker employed by him is

maintained in a register in Form-XXXVII annexed to these rules and

such  register  shall  be  made  available  to  the  Inspector  having

jurisdiction, on demand;

54

(e)     in  case  a  construction  medical  officer  examining  a  building  worker

under  sub-clause  (I)  or  sub  clause  (ii)  of  clause  (a)  is  of  the  opinion

that  such  building  worker  so  examined  is  required  to  be  taken  away

from the building or other construction work at which he is  employed

for health protection, such medical officer shall inform the employer of

such building worker accordingly and such employer shall inform such

opinion to the Board where such worker is registered as a beneficiary.

112.  Duties of construction medical officers Sec. 40

(1)

The medical examination referred to in sub-clause (i) or sub clause(ii)

of clause(a) of rule 111 shall be carried out by a construction medical

officer.

(2)

The duties and responsibilities of such construction medical officer shall

be as given below, namely-

(a)

medical examination of building workers;

(b)

first-aid care including emergency medical treatment;

(c)      notification   of   occupational   diseases   to   the   concerned

authorities in accordance with these rules;

(d)

immunization services;

(e)

medical record upkeep and maintenance;

(f)

health education including advisory services on family planning,

personal hygiene, environmental sanitation and safety;

(g)    referral

113.  Occupational health centers Sec. 40

The employer shall ensure at a construction  site of a building or other

construction  work  involving  hazardous  processes  specified  under  Schedule-

III annexed to these rules that-

(a)     an  occupational  health  centre,  mobile  or  static,  is  provided  and

maintained in good order at such site;

(b)

services  and  facilities  as  per  the  scale  laid  down  in  Schedule -IV,

annexed to these rules are provided at the occupational health centre

referred to in clause(a) ;

(c)

a construction medical officer appointed at a occupational health enter

possesses  the  qualification  as  laid  down  in  Schedule -V,  annexed  to

these rules.

55

114.  Ambulance Rooms Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that -

(a)

in case five hundred or less workers are employed at such construction

site  there  is  an  ambulance  room  at  such  construction  site  or  an

arrangement with a nearby hospital for providing an ambulance room

and such ambulance room is in the charge of a qualified nurse and the

service  of    such  ambulance  room  is  available  to  building  worker

employed at such construction site at every time when he is at work;

(b)

in case more than five hundred building workers are employed at such

construction   site   there   is   an   ambulance   room   with   effective

communication system and such ambulance room is in the charge  of a

qualified nurse and the service of such ambulance room is available to

a  building  worker  employed  at  such  construction  site  at  every  time

when he is at work, and such ambulance room is in overall charge of a

construction medical officer;

(c)

an ambulance room referred to in clause (a) or clause (b) is equipped

with the articles specified is Schedule-VI, annexed to these rules;

(d)

record of all cases of accidents and sickness treated at the ambulance

room referred to in clause(a) or clause(b) is maintained and produced

to the inspector having jurisdiction on demand.

115.  Ambulance Van Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that an ambulance van is provided at such construction site

or  an  arrangement  is  made  with  a  nearby    hospital  for  providing  such

ambulance van for transportation of serious cases of accident or sickness of

the  building  workers  to  the  hospital    promptly    and  such  ambulance  van  is

maintained in good repair and is equipped with standard facilities specified in

Schedule-VII annexed to these rules.

116.  Stretches Sec.

The employer shall ensure at a construction site of a building or other

construction  work  that  sufficient  number  of  stretchers  is  provided  at  such

construction site so as to be readily  available in an emergency.

56

117.  Occupational health services for building workers

(1)

The employer shall ensure at a construction site of a building or other

construction work, where more than five hundred building workers are

employed that-

(a)

a  special  medical  service  or  an  occupational  health  service  is

available at such construction site at all times and such service

shall-

(i)

provide first-aid and emergency treatment;

(ii)     conduct  special  medical  examination  for  occupational

hazards   to   such   building   workers   before   their

employment  and  thereafter  at  such  intervals  as  may  be

specified by the Chief Inspector from time to time;

(iii)     conduct  training  of  first-aid  personnel  of  such  medical

service;

(iv)     render  advice  to  such  employer  on  conditions  of  work

and improvement required to avoid hazards to the health

of such building worker;

promote   health   education,   including   family   welfare

among such building workers;

(vi)     co-operate  with  the  Inspector  having  jurisdiction  in  the

detection,  measurement  and  evaluation  of  chemical,

physical  or  biological  factors  suspected  of  being  harmful

to such building workers;

(vii)   undertake  immunization  for  all  such  building  workers

against  tetanus,  typhoid,  cholera  and  other  infectious

diseases;

(b)

the special medical service referred to in clause (a) collaborates

with the Labour department or any other concerned department

or service of the State Government in matters of treatment, job

placement,  accident  prevention  and  welfare  of  such  building

workers;

(c)

the special medical service referred to in clause (a) is headed by

a  construction  medical  officer  and  is  provided  with  adequate

staff, laboratory and other equipments;

57

(d)     the  premises  of  the  special  medical  service  referred  to  in

clause(a)  are  conveniently  accessible,  comprise  at  least  a

waiting room a consulting room a treatment room, a laboratory

and suitable accommodation for nurses and other staff of such

service;

(e)

the  special  medical  service  referred  to  in  clause  a)  maintains

records pertaining to its activities referred to in sub-clause(i) to

(vii) of  clause(a) and sends to the Chief Inspector once in every

three months, information in writing on-

(i)

the state of health of such building workers ;and

(ii)

the nature and causes of occupational injuries or disease

suffered  by  any  of  such  building  workers,  treatment

provided to such worker and measures taken to prevent

recurrence of such injuries or disease.

118.  Notice of poisoning or occupational diseases Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that-

(a)

when a building  worker contracts any disease specified in Schedule-

VIII  annexed  to  these  rules,  a  notice  in  Form-XXXVIII  annexed  to

these rules is sent without delay to the Inspector, having jurisdiction,

and  to  the  Board  with  which  such  building  worker  is  registered  as  a

beneficiary ;

(b)

If any medical practitioner or construction medical officer attends on a

building  worker  suffering  from  any  disease  referred  to  in  clause  (a)

such   medical   practitioner   or   construction   medical   officer   sends

information  regarding  the  name  and  full  particulars  of  such  building

worker  and  the  disease  suffered  by  him,  to  the  Chief  Inspector

without delay.

119.  First aid boxes Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that-

(a)

sufficient  number  of  first  aid  boxes  or  cupboards  are  provided  and

maintained for providing first aid to the building workers;

58

(b)

every  first-aid  box  or  cupboard  is  distinctly  marked  “First-Aid”  and  is

equipped with the articles specified in Schedule-IX annexed to these

rules;

(c)

nothing except appliances or requisites for first-aid is kept in a first-aid

box or cupboard and such box or cupboard is so kept as to protect it

against  contamination  by  dust  or  other  foreign  matter  and  against

penetration of moisture and such box or cupboard is kept in the charge

of  a  person  trained  in  first-aid  and  is  always  readily  available  during

working hours.

120.  Emergency care services or emergency treatment Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that-

(a)

essential life saving aides and appliances required to handle-

(i)

head injuries and spinal injuries;

(ii)     bleeding;

(iii)

fractures and dislocations of bones and joints;

(iv)    crush

(v)

shock including electric shock;

(vi)

dehydration due to any cause;

vii)

snake bite, insect bite, scorpion and bee stings;

(viii)

burns including chemical burn;

(ix)

bends or divers paralysis;

(x)    other    surgical,    gynecological,    obstetric,    or    pediatric

emergencies;

(xi)    drowning

(xii)

sunstroke and frost bite to building workers,

are  provided  and  properly  maintained  under  the  supervision  of  a

construction medical officer ;

(b)

the essential life saving aids for any emergent situation referred to in

sub-clause (i) to (xii) of clause (a) are provided to an injured or a sick

building  worker  during  his  transportation  from  such  building  site  to  a

hospital  and  till  such  building  worker  is  attended  by  a  doctor  in  such

hospital;

(c)     any  other  equipment  or  facilities  required  for  emergency  care  or

treatment to the building workers arising from special local conditions

59

and  construction  processes  of  such  building  site,  as  specified  by  the

State Government from time to time, are provided.

CHAPTER-VII

(Part-III)

Lifting Appliances And Gear

121.  Construction and maintenance of lifting appliances Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that -

(a)

all  lifting  appliances,  including  their  parts  and  working  gear,  whether

fixed or moveable and any plant or gear used in anchoring or fixing of

such appliances, are—

(i)

of sound construction, sound material, and of adequate strength

to serve the purpose for which these are to be used and all such

appliances shall be free form patent defects; and

(ii)

maintained in good repair and working condition.

(b)

(i)

every  drum  or  pulley  around  which  the  rope  of  any  lifting

appliance  is  carried,  is  of  adequate  diameter  and  sound

construction in relation to such rope;

(ii)     any  rope  which  terminates  at  the  winding  drum  of  a  lifting

appliance  is  securely  attached  to  such  drum  and  at  least  three

dead turns of such rope remain on such drum in every operating

position of such lifting appliance;

(iii)

the  flange  of  a  drum  projects  twice  the  rope  diameter  beyond

the last layer of such rope and if such projection is not available,

other  measures  like  anti-slackness  guards  shall  be  provided  to

prevent such rope from coming of such drum;

(c)

Every  lifting  appliance  is  provided  with  adequate  and  efficient  brakes

which-

(i)

are capable of preventing fall of a suspended load (including any

test  load)  and  of  effectively  controlling  such  load  while  it  is

being lowered;

(ii)

act without shock;

(iii)

have shoes that can be easily removed for running; and

60

(iv)    are  provided  with  simple  and  easily  accessible  means  of

adjustment:

Provided  that  nothing  contained  in  this  clause  shall  apply  to

steamwinch  which  can  be  operated  as  safely  as  with  brakes  as

provided in accordance with this clause.

(d)

Controls of every lifting appliance-

(i)

are so situated that the driver of such appliance at his stand or

seat has ample room for operating and has an unrestricted view

of  building    or  other  construction  work,  as  far  as  practicable,

and  that  he  remains  clear  of  the  load  and  ropes,  and  that  no

load passes over him;

(ii)

are positioned with due regard to ergonometric considerations

for proper operations of such appliance;

(iii)

are so located that the driver of such appliance remains above

the height of the heel block during the whole operation of such

appliance;

(iv)

have  upon  them  or  adjacent  to  them  clear  markings  to

indicate their purpose and mode of operations;

(v)

are provided, where necessary, with a suitable locking device

to prevent accidental movement or displacement;

(vi)

move,  as  far  as  practicable,  in  the  direction  of  the  resultant

load movement; and

(vii)

whenever  automatic  brakes  are  provided,  automatically  come

to the neutral position in case of power failure.

122.  Test and periodical examination of lifting appliances Sec. 40

The  employer  shall  ensure  at  construction  site  of  a  building  or  other

construction work that,-

(a)

all lifting appliances including all parts and gears thereof, whether fixed

or moveable, are tested and examined by a competent person before

being  taken  into  use  for  the  first  time  or  after  it  has  undergone  any

alterations  or  repairs  liable  to  affect  its  strength  or  stability  or  after

erection  on  a  construction  site  and  also  once  at  least  in  every  five

years, in the manner specified in Schedule-X annexed to these rules;

(b)

all  lifting  appliances  are  thoroughly  examined  by  a  competent  person

once at least in every twelve months and where the competent person

61

making  such  examination  forms  the  opinion  that  the  lifting  appliance

cannot  continue  to  function  safely,  he  shall  forthwith  give  notice  in

writing of his opinion to the owner of the lifting appliance.

Explanation.-   For  the  purpose  of  this  rule,  through  examination

means  a  visual  examination,  supplemented,  if  necessary,  by  other

means such as hammer test, carried out as carefully as the conditions

permit, in order to arrive at a reliable conclusion as to the safety of the

parts examined; and , if necessary, for such examination, parts of the

lifting appliance and gear, shall be dismantled.

123.  Automatic safe lead indicators  Sec. 40

(1)

The employer shall ensure at a construction site of a building or other

construction work that—

(i)

every crane, if so constructed that the safe working load may be

varied by raising or lowering of the jib or otherwise, is attached

with an automatic indicator of safe working loads which gives a

warning  to  the  operator  wherever  the  load  exceeds  the  safe

working load;

(ii)

cut-out  is  provided  which  automatically  arrests  the  movements

of  the  lifting  parts  of  every  crane  if  the  load  exceeds  the  safe

working load, wherever possible;

(2)

The provisions of sub-rule (1) apply, except where it is not possible to

install  an  automatic  safe  load  indicator,  in  which  case,  provision  of  a

table showing the safe working loads at the corresponding inclinations

or radi of the jib on the crane shall be considered sufficient.

124.  Installation

(1)

The employer shall ensure at a construction site of a building or other

construction work that—

(a)

fixed lifting appliances are installed--

(i)

by competent persons;

(ii)

in a manner that such appliances cannot be displaced by

the loan,  vibration or other influences;

(iii)

in  a  manner  that  the  operator  of  such  appliances  is  not

exposed to danger from loads, ropes or drums; and ;

62

(iv)     in  a  manner  that  the  operator  can  either  see  over  the

zone  of  operation  or  communicate  with  all  loading  and

unloading  points  by  signal,  or  other  communication

system;

(b)

adequate clearance is provided between parts or loads of lifting

appliances and--

(i)

the fixed objects such as walls and posts; or

(ii)     electrical conductors;

(c)

the  lifting  appliances,  when  exposed  to  wind  loading  are  given

sufficient  additional  strength,  stability  and  rigidity  to  withstand

such loading safely;

(d)

no structural alterations or repairs are made on any part of the

lifting  appliances  that  affect  the  safety  of  such  appliances

without  obtaining  the  opinion  of  the  competent  person  to  this

effect.

125.  Winches Sec.

The employer shall ensure at a construction site of a building or other

construction work that.--

(a)

(i)

winches  are  not  used  if  control  levers  operate  with  excessive

friction or play;

(ii)

double  gear  winches  are  not  used  unless  a  positive  means  of

locking the gear shift is provided;

(iii)

there  is  no  load  other  than  the  fall  and  the  book  assembly  on

the winch while changing gears on a two gear winch ;

(iv)    adequate  protection  is  provided  to  winch  operator  against

abnormal weather ;

(v)

temporary seats or shelter for winch operators which may pose

hazard to the winch operator or any other building workers are

not allowed to be used;

(b)

in use of every steam winch--

(i)

measures  are  taken  to  prevent  excaping  steam  from  obscuring

any  part  of  the  construction  site  or  other  work  place  or  from

otherwise hindering or injuring any building worker;

63

(ii)

extension  control  levers  which  tend  to  fall  of  their  own  weight

are counter balanced;

(iii)     winch  operators  are  not  permitted  to  use  the  winch  control

extension levers except for short handles on wheel type controls

and  that  such  levers  are  of  adequate  strength,  secure  and

fastened  with  metal  connections  at  the  fulcrum  and  at  the

permanent control lever;

(c)

in use of every electric winch, a building or other construction worker is

not permitted to transfer, alter or adjust electric control circuits in case

of any defect in such winch.

(d)

electric winches are not used for building work where—

(i)

the electro- magnetic brake is unable to hold the load; or

(ii)

one or more control points, either hoisting or lowering, are not

operating properly.

126.  Bukets Sec. 40

The employer shall ensure at a construction site of a building or other

construction  work  that  tip-up  buckets  are  equipped  with  a  device  that

effectively prevents accidental-tipping.

127.  Identification and marking of safe working load Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that--

(a)

every  lifting  appliance  and  loose  gear  is  clearly  marked  for  its  safe

working load and identification by stamping or other suitable means;

(b)

(i)

every derrick ( other than derrick crane) is clearly marked for its

safe  working  load  when  such  derrick  is  used  either  in  single

purchase with a lower block or in union purchases in all possible

block positions;

(ii)

the lowest angle to the horizontal, to which the derrick may be

used, is legibly marked;

(c)

every lifting appliance having more than one working load is fitted with

effective means to enable the operator to determine safe working load

at each point under all condition of use;

64

(d)

means  to  ascertain  the  safe  working  load  for  lifting  gears  under  such

conditions in which such gears may be used are provided to enable a

worker using such gears and such means shall consist of—

(i)

marking  the  safe  working  load  in  plain  figures  or  letters  upon

the  sling  or  upon  a  table  or  ring  of  durable  material  attached

securely thereto in case of chain slings; and

(ii)     either  the  means  specified  in  sub-clause  (i)  or  notices  so

exhibited  as  can  be  easily  read  by  any  concerned  building

worker stating the safe working load for the various sizes of the

wire rope slings used in case of wire rope slings.

128.  Loading of lifting appliance and lifting gears Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that--

(a)

no lifting appliance, lifting gear or wire rope is used in an unsafe way

and in such a manner as to involve risk to life of building workers, and

that  they  are  not  loaded  beyond  their  safe  working  load  except  for

testing  purposes  under  the  direction  of  a  competent  person  in  the

manner as specified in Schedule -X annexed to these rules.

(b)     no  lifting  appliance,  lifting  gear  or  any  other  material  handling

appliance is used, if—

(i)

the  Inspector  having  jurisdiction  is  not  satisfied  with  reference

to  a  certificate  of  test  or  examination  or  to  an  authenticated

record maintained as provided under these rules; and

(ii)

in  the  view  of  such  Inspector,  the  lifting  appliance,  lifting  gear

or  any  other  material  handling  appliance  is  not  safe  for  use  in

building or other construction work;

(iii)

no  pulley  block  is  used  in  building  or  other  construction  work

unless  the  safe  working  load  and  its  identification  are  clearly

marked on such block.

129.  Operator’s cab or cabin Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that--

(a)

the  operator  of  every  lifting  machine  in  outdoor  service  is  provided

with a cab or cabin which—

65

(i)

is made of fire resistant material;

(ii)

has a suitable seat, a foot rest and protection from vibration;

(iii)

affords the operator an adequate view of the area of operation.

(iv)

affords the necessary access to working parts in cab;

(v)

affords the operator adequate protection against the weather;

(vi)

is adequately ventilated; and

(vii)

is provided with a suitable fire extinguisher.

130.  Operation of lifting appliance  Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that--

(a)     every  crane  driver  or  lifting  operator  possess  adequate  skill  and

training in the operation of the particular lifting appliance;

(b)

no  person  under  eighteen  years  of  age  is  in  control  of  any  lifting

appliance, scaffold winch, or to give signals to the operator;

(c)

precaution is taken by the trained operator to prevent lifting appliance

from being set in motion;

(d)

the operation of lifting appliance is governed by signals, in conformity

with the relevant national standards;

(e)

the  lifting  appliance  operator’s  attention  is  not  distracted  while  he  is

working;

(f)

no  crane,  hoist,  winch  or  other  lifting  appliance  or  any  part  of  such

crane,  hoist,  winch  or  other  lifting  appliance  is,  except  for  testing

purposes, loaded beyond the safe working load;

(g)

during the hoisting operations effective precaution is taken to prevent

any  person  from  standing  or  passing  under  the  load  in  such

operations;

(h)

operator does not leave lifting appliance unattended while power is on

or load is suspended to such appliance;

(i) 
(j)

no person rides on a suspended load or on any lifting appliance;

every part of a load in course of being hoisted or lowered is adequately

suspended and supported to prevent danger;

(k)

every receptacle used for hoisting bricks, tiles, slates or other material

is suitably enclosed as to prevent the fall of any such materials;

66

(l)

the  hoisting  platform  is  enclosed  when  loose  materials  or  loaded

wheel-barrows  are  placed  directly  on  such  platform  or  lowering,  such

materials or wheel-barrows;

(m)

no  material  is  raised,  lowered  or  slowed  with  any  lifting  appliance  in

such a way as to cause sudden jerks to such appliance;

(n)

in hoisting a barrow , any wheel of such barrow is not used as a means

of support unless adequate steps are taken to prevent the axle of such

wheel from slipping out of its bearing;

(o)

long  objects  like  planks  or  girders  are  provided  with  a  tag  line  to

prevent  any  possibility  of  danger  while  raising  or  lowering  such

objects;

(p)

during  the  process  of  landing  of  material,  a  building  worker  is  not

permitted to lean out into empty space for finding out the loading and

unloading of such material;

(q)

the hoisting of loads at places where there is regular flow of traffic is

carried  out  in  an  enclosed  space,  or  in  case  such  hoisting  is

impracticable  in  enclosed  space  measures  are  taken  to  hold  up  or

divert the traffic during the time of such hoisting;

(r)

adequate  steps  are  taken  to  prevent  a  load,  in  the  course  of  being

hoisted or lowered from coming into contact with any object to avoid

any displacement of such load;

(s)

appliances are provided and used for guiding heavy loads when raising

or lowering heavy loads to avoid crushing of hands of building workers

during such raising or lowering of loads.

131.  Hoists

The employer shall ensure at a construction site of a building or other

construction work that—

(a) 
(b)

hoist towers are designed according to relevant national standards;

hoist shafts are provided with right panels or other adequate fencing--

(i)

at the ground level on all sides of such shafts; and

(ii)

at all other levels on all sides of the access to such shafts;

(c)

the  walls  of  hoist  shafts,  except  at  approaches,  extend  at  least  two

meters above the floor or platform of access to such shafts;

(d)

approaches to a hoist are provided with gates which are--

(i)

girded to maintain visibility;

67

(ii)

at least of two meters height; and

(iii)

equipped  with  a  device  which  requires  such  gate  to  be  closed

before  the  platform  of  such  hoist  can  leave  the  landing  and

prevents the gate from being opened unless such platform, is at

the landing;

(e)

approaches to a hoist are adequately lit;

(f)

the guides of hoist platforms offer sufficient resistance to bending and,

to bucking, in the case of jamming, by providing a safety catch;

(g)

overhead  beams  and  their  supports  are  capable  of  holding  the  total

maximum  live  and  dead  loads  that  such  beams  and  supports  will  be

required to carry, with a safety factor of at least five;

(h)

a clear space is provided-

(i)

above the highest stopping place of a cage or platform to allow

sufficient unobstructed travel of such cage or platform in case of

over winding; and

(ii)

below the lowest stopping place of such cage or platform;

(i)

adequate covering is provided above the top of hoist shafts to prevent

materials from falling into such shafts;

(j)

outdoor  hoist  towers  are  erected  on  adequately  firm  foundations  and

are securely braced, guyed and anchored;

(k)

a ladderway extends from the bottom to the top of every outdoor hoist

tower  in  case  no  other  ladder  way  exists  within  easy  reach  and  such

ladderway comply with the relevant national standards;

(l)

the rated capacity of a hoisting engine is at least one and a half times

the maximum load that such engine will be required to move;

(m)

all gearing on a hoisting engine is securely enclosed;

(n)     steam  piping  of  a  hoisting  engine  is  adequately  protected  against

accidental contact of such piping with a building worker;

(o)

electrical equipment of a hoisting engine is effectively earthed;

(p)

a  hoist  is  provided  with  suitable  devices  to  stop  a  hoisting  engine  as

soon as the platform of such hoist reaches its highest stopping place;

(q)

a hoisting engine is protected by a suitable cover against weather and

falling objects;

(r)     a  hoisting  engine  set  up  in  a  public  thoroughfare  is  completely

enclosed;

68

(s)

all  exhaust  steam  pipes  discharge  steam  in  such  a  manner  that  the

steam  so  discharged  does  not  scald  any  person  or  obstruct  the

operator’s view;

(t)

the motion of a hoist is not reversed without first bringing it to rest to

avoid any harm from such reverse motion;

(u)

a  hoist,  not  designed  for  the  conveyance  of  persons,  is  not  set  in

motion from the platform of such hoist;

(v)

pawls and ratchet wheels of a hoist, requiring disengagement of such

pawls  from  such  ratchet  wheels,  before  the  platform  of  such  hoist  is

lowered, are not used;

(w)

a platform of a hoist is capable of supporting such maximum load, that

such platform may carry, with a safety factor of at least three;

(x)

a platform of a hoist is equipped  with suitable safety gear which can

hold  such  platform  with  its  maximum  load  in  case  its  hoisting  rope

breaks;

(y)

on  platform  of  a  hoist,  the  wheelbarrows  or  truck  are  efficiently

blocked in a safe position;

(z)

a  cage  of  a  hoist  or  a  platform,  where  the  building  workers  are

required to enter into such cage or to go on such platform at landing

level, is provided with a locking arrangement to prevent such cage or

platform from moving during the time a worker enters or leaves such

cage or platform;

(za)    the  sides  of  a  platform  of  a  hoist  which,  are  not  used  for  loading  or

unloading, are provided with toe-board and enclosures of a wire mesh

or  any  other  suitable  means  to  prevent  the  fall  of  any  part  of  a  load

from such platform;

(zb)    a  platform  of  a  hoist,  which  has  any  probability  of  falling  and  part  of

load from it, is provided with an adequate covering with such fall;

(zc)

the counter-weights of a hoist consisting of an assemblage of several

parts  are  so  constructed  that  such  parts  are  rigidly    connected

together;

(zd)

the counter-weights of a hoist run between guides;

(ze)

at every level of work and building worker are provided with adequate

platforms for performing such work;

(zf)

a legible notice in Hindi as well as in a local language is displayed at—

69

(i)

a  conspicuous  place  of  the  platforms  of  a  hoist  and  that  such

notice  states  the  maximum  carrying  capacity  of  such  hoist  in

kilograms;

(ii)

a conspicuous place on the hoisting engine and that such notice

states maximum lifting capacity of such hoist in kilograms;

(iii)

a conspicuous place on a hoist authorized and certified for  the

conveyance  of  the  person  on  the  platform  or  cage  and  such

notice states the maximum number of persons to be carried on

such hoist at one time;

(iv)    a  conspicuous  place  on  a  hoist  carrying  goods  and  other

materials and such notice states that such hoist is not meant for

carriage of persons.

132.  Fencing of means of access to lifting appliances Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that--

(a) 
(b)

safe means of access is provided to every part of a lifting appliance;

the  operator’s  platform  on  every  crane  or  tip  driven  by  mechanical

power  is  securely  fenced  and  is  provided  with  safe  means  of  access

and where access to such platform is by a ladder,-

(i)

the sides of such ladder extend to a reasonable height beyond

such  platforms  or  some  other  suitable  handhold  is  provided  in

lieu  thereof  to  prevent  any  falling  of  persons  from  such

platforms;

(ii)

the  handling  place  on  such  platform  is  maintained  free  from

obstruction and slipping; and

(iii)

in case the height of such ladder exceeds six meters, the resting

platform are provided on such ladder at every six metres of its

height  and  where  the  distance  between  last  platform  so

provided  and  the  top  end  of  such  ladder  is  more  than  two

meters then on such top end.

133.  Rigging of derricks Sec. 40

The employer shall ensure at a construction site of a building or other

construction  work  that  every  derrick  has  current  and  relevant  rigging  plans

70

and any other information necessary for the safe rigging of such derricks and

its gear.

134.  Securing of derrick foot Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that appropriate measures are taken to prevent the foot of

a derrick being lifted out of its socket or support.

135.  Construction and maintenance of lifting gears Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that--

(a)

every lifting gear is--

(i)

of  good  design  and  construction,  sound  material  and  adequate

strength to perform the work for which it is used;

(ii)

free from patent defects; and

(iii)

properly maintained in good repair and working order;

(b)

components  of  the  loose  gear,  at  the  time  of  its  use,  are  renewed  if

one  of  its  dimensions  at  any  point  has  decreased  by  ten  per  cent  or

more by user;

(c)

a  chain  is  withdrawn  from  use  when  it  is  stretched  and  increased  in

length which exceeds five per cent of its length or when a link of such

chain is deformed or is otherwise damaged or raised scrafs of defective

welds is appeared on it;

(d)

rings, hooks, swivels and end links attached to a chain are of the same

material as that of such chain;

(e)

the  voltage  of  electric  supply  to  any  magnetic  lifting  device  does  not

fluctuate by more than plus ten per cent or minus ten per cent.

136.  Test and periodical examination of lifting gears Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that—

(a)

a  lifting  gear  is  initially  tested  for  the  manufacturer  by  a  competent

person, in a manner specified in Schedule-X annexed to these rules

before taking into use or after undergoing any substantive alterations

which  renders  its  any  part  liable  to  affect  its  safety  and  such  gear

71

alters such test shall subsequently be retested for the use of its owner

at least once in every five years.

(b)

a  lifting  gear  in  use  is  thoroughly  examined  once  at  least  in  every

twelve  months  by  a  competent  person  in  Form-XXXIX  annexed  to

these rules ;

(c)

a chain in use is thoroughly examined once at least every month by a

responsible person for its use;

(d)     certificates  of  initial  and  periodical  tests  and  examinations  of  loose

gears  under  these  rules  are  obtained  in  Form-XL  annexed  to  these

rules;

137.  Ropes Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that--

(a)

no rope is used for building or other construction work unless--

(i)

it is of good quality and free from patent defects; and

(ii)

in the case of wire rope, it has been tested and examined by a

competent  person  in  the  manner  specified  in  Schedule-X

annexed to these rules;

(b)

every wire rope of lifting appliance or lifting gear used for building or

other construction work is inspected by a responsible person for such

use, once at least in every three months:

Provided that after any such wire is broken in such rope, it shall

thereafter be inspected once at least in every month by the responsible

person.

(c)

no  wire  rope  is  used  for  building  or  other  construction  work  if  in  any

length  of  eight  diameters  of  such  wires,  the  total  number  of  visible

broken wires exceed ten per cent of the total number of wires in such

rope,  or  such    rope  shows  sign  of  excessive  wear,  corrosion  or  other

defects which in the opinion of the person who inspects it or Inspector,

having jurisdiction, is unfit for use.

(d)

eye splices and loops of ropes for the attachment of hooks, rings and

other such parts to wire ropes are made with suitable thimble.

(e)

a  thimble  or  loop  splice  made  in  any  wire  rope  sling  conforms  to  the

following standards, namely:-

72

(i)

wire rope sling shall have at least three tucks with full strand of

rope and two tucks with one-half of the wires cut out of each of

such strand in all cases, such strands shall be tucked against the

lay of the rope;

(ii)

protruding end of such strands in any splice of wire rope slings

shall be covered or treated so as to leave no sharp points;

(iii)

a fiber rope or a rope sling shall have at least four tucks; tail of

such tuck being whipped in a suitable manner; and

(iv)

a synthetic fiber rope or rope sling shall have at least four tucks

with full strand followed by further tuck with one half filaments

cut out of each of such strand and final tuck with one-half of the

remaining  filaments  cut-out  from  such  strands.  Any  portion  of

the  splices  containing  such  tucks,  with  reduced  number  of

filaments, shall be securely covered with suitable tape or other

materials:

Provided  that  nothing  contained  in  this  sub-clause  shall

apply  where  any  other  form  of  splice,  which  may  be  shown  to

be as efficient as the splice with above standards, is used.

138.  Heat treatment of lifting gears Sec. 40 and 42

The employer shall ensure at a construction site of a building or other

construction work that--

(a)

all  chains  other  than  bridle  chains  attached  to  derricks  and  all  rings,

books, shakles and swivels used in hoisting or lowering of such derricks

are effectively annealed under supervision of a competent person and

at the following intervals, namely :-

(i)

such  chains,  rings,  hooks,  shackles  and  swivels  which  are  not

more  than  twelve  and  a  half  millimeter  of  length  are  so

annealed at least once in every six months; and

(ii)

all other such chains, rings, hooks, shackles and swviles are so

annealed at least once in every twelve months:

Provided that such annealing as referred to in sub-clause

(i)  and  sub-clause  (ii)  shall  not  be  required  if  the  Inspector,

having  jurisdiction,  after  obtaining  the  approval  of  the  Chief

Inspector,  directs  that  such  chains,  rings,  hooks,  shackles  and

73

swivels  undergo  some  other  treatment  and  in  such  cases  the

treatment directed by such Inspector shall be followed:

Provided further that in case of such chains, rings hooks,

shackles  and  swivels  used  solely  on  such  derricks  and  other

hoisting appliances which are worked by hand, the provisions of

sub-clause(i) and sub-clause(ii), as the case may be, shall apply

as if for the period of six months and twelve months the periods

of  twelve  months  and  two  years  have  respectively  been

substituted therein :

Provided further that in case where the Inspector, having

jurisdiction,  is  of  the  opinion  that  owing  to  the  size,  design

material  or  frequency  of  use  of  any  such  chains,  rings,  hooks,

shackles  and  swivels,  the  requirement  of  this  clause  for

annealing is not necessary for the protection of building worker,

he  may  after  obtaining  the  approval  of  the  Chief  Inspector,

certify in writing to such employer that subject to the conditions

specified   in   such   certification,   such   chains,   rings,   hooks,

shackles  and  swivels  are  exempted  from  such  annealing  and

thereafter the provision of this clause shall apply subject to such

exemption:

Provided further that this clause shall not apply to--

(i)

pitched chains, working on sprocket or sprocketed

wheels;

(ii)

rings,  hooks  and  swivels  permanently  attached  to

pitched   chains,   pulley   blocks   or   weighing

machines; and

(iii)     hooks  and  swivels  having  ball  bearings  or  other

case hardened parts;

(b)

a  chain  or  a  loose  gear  made  of  high  tensile  steel  or  alloy  steel  is

plainly marked with a mark indicating that it is so made;

(c)

no  chain  or  loose  gear  made  of  high  tensile  steel  or  alloy  steel  is

subjected to any form of heat treatment except where such treatment

74

is necessary for the purpose of repair of such chain or loose gear and

that such repair is made under the direction of the competent person.

(d)

that the wrought iron gear, the past history of which is not traceable, is

suspected of being heat treated at incorrect temperature, is normalized

before using it on any building or other construction work.

139.   Certificate  is  to  be  issued  after  actual  testing  and  examination  etc

Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that a competent person issues a certificate for the purpose

of rule 122, rule 128, rule 137 and rule 138 only after actual testing or, as the

case may be, examination of the apparatus specified in the said rules.

140.  Register of periodical test, examination and certificates Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that--

(a)

a  register  in  Form-XLI,  annexed  to  these  rules  is  maintained  and

particulars  of  such  test  and  examination  of  lifting  appliances,  lifting

gears and heat treatment as required under rule 121, rule 128 and rule

138 are entered in such register;

(b)     certificate  in  respect  of  the  following  is  obtained  from  competent

person in the forms as mentioned below, namely:-

(i)

in case of initial and periodical test and examination under rule

122 and rule 138 for--

(a)

winches,  derricks  and  their  accessory  gears  in  Form-

XLII annexed to these rules;

(b)

cranes  or  hoists  and  their  accessory  gears  in  Form-

XLIII annexed to these rules;

(ii)

in  case  of  test,  examination  and  re-examination  of  loose  gears

under  clause  (d)  of  rule  136  in  Form-XL  annexed  to  these

rules;

(iii)

in case of test and examination of wire ropes under rule 128 in

Form-XLIV annexed to these rules;

(iv)

in case of heat treatment and examination of loose gears under

rule 138 in Form-XLV annexed to these rules;

75

(v)

in case of annual thorough examination of the loose gears under

clause (b) of rule 136 except where required particulars of such

exemption  have  been  enclosed  in  the  register  referred  to  in

clause  (a),  in  Form-XLI,  annexed  to  these  rules,  and  such

certificates are attached to the register referred to in clause (a);

(c)

the register referred to in clause (a) and the certificates referred to in

clause (b) attached to such register are—

(i)     kept  at  such  construction  site  in  case  such  register  and

certificate relate to lifting appliances, loose gear and wire ropes;

(ii)

produced  on  demand  before  an  Inspector  having  jurisdiction;

and

(iii)

retained  for  at  least  five  years  after  the  date  of  the  last  entry

made in such register;

(d)     no  lifting  appliance  or  lifting  gear  in  respect  of  which  an  entry  is

required to be made in register referred to in clause (a) and certificate

of test and examination are required to be attached in such register in

the  manner  as  specified  in  clause  (a)  or  clause  (b),  as  the  case  may

be, is used for building or other construction work unless the required

entries have been made in such register and certificates.

141.  Vacuum and magnetic lifting gears Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that--

(a)

no vacuum  lifting gear, magnetic lifting gear or any other lifting gear

where the load on it is held by adhesive power, is used while workers

are performing operations beneath such gear;

(b)     a  magnetic  lifting  gear  used  in  connection  with  building  or  other

construction work is provided with an alternative supply of power, such

as batteries, which may come into operation immediately in the event

of failure of the main power supply;

(c)

no  building  worker  shall  work  within  the  swinging  zone  of  the  lifting

gear  or  load  or  building  or  other  construction  material  suspended  to

such lifting gear.

142.  Knotting of chains and wire ropes Sec. 40

76

The employer shall ensure at a construction site of a building or other

construction  work  that  no  chain  or  wire  rope  with  a  knot  in  it  is  used  in

building or other construction work.

143.  Carrying of persons by means of lifting appliance Sec. 40

(1)

The employer shall ensure at a construction site of a building or other

construction work that no building worker is raised, lowered or carried

by a power driven lifting appliance except--

(a)

on the driver’s platform in the cage of a crane; or

(b)

on a hoist; or

(c)

on an approved suspended scaffold:

Provided  that  a  building  worker  may  be  raised,  lowered

or carried by a power driven lifting appliance—

(i)      in  circumstances  where  the  use  of  a  hoist  or  of  a

suspended scaffold is not reasonably practicable and the

requirements of sub-rule (2) are complied with; or

(ii)

on  an  aerial  cableway  or  aerial  ropeway  in  case  where

the   requirements of sub-rule(2) are complied with.

(2)

The  requirements  referred  to  in  proviso  to  sub-rule(I)  are  as  below,

namely:-

(i)

that  the  appliance  referred  to  in  such  proviso  can  be  operated

from one position only;

(ii)

that any winch used in connection with the appliance referred to

in such proviso comply with the requirements of rule 125.

(iii)

that  no  person  shall  be  carried  by  the  appliance  referred  to  in

such proviso except--

(a)

in a chair or cage, or

(b)     in  a  skip  or  other  receptacle  at  least  three  feet  deep

which  is  suitable  for  safe  carriage  of  a  person  and  any

such    chair,  cage,  skip  or  other  receptacle  is  made  of

good  construction  sound  material,  and  has  adequate

strength  and  is  properly  maintained  with  suitable  means

to prevent any occupant therein from falling out of it and

is  free  from  any  material  or  tools  which  may  interfere

with  the  handhold  or  foothold  of  such  occupant  or

otherwise endanger him; and

77

(iv)

that suitable measures shall be taken to prevent the chair, cage,

skip  or  other  receptacle  from  spinning  or  tipping  in  a  manner

dangerous to any occupant therein.

144.  Hoists carrying persons Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that--

(a)

no building worker is carried by a hoist unless it is provided with a cage

which--

(i)

is  so  constructed  as  to  prevent,  when  its  gates  are  shut,  any

building  worker  carried  by  such  hoist  from  falling  out  of  it  or

from  being  trapped  between  any  part  of  such  cage  and  any

fixed structure or other moving part of such hoist or from being

struck  by  articles  or  materials  falling  down  the  hoistway  on

which such hoist is moving; and

(ii)

is fitted on each of its side from which, access is provided to a

landing  placed  with  a  gate  which  has  efficient  interlocking  or

other  devices  to  secure  so  that  such  gate  cannot  be  opened

except when such cage is at a landing place and that such cage

cannot  be  moved  away  from  any  such  place  until  such  gate  is

closed;

(b)

every  gate  in  the  hoistway  enclosure  of  such  hoist  used  for  carrying

persons is fitted with efficient inter-locking or other devices to secure

so that gate cannot be opened except when the cage of such gate is at

the landing place, and that such cage cannot be moved away from the

landing place until such gate is closed;

(c)

in  every  hoist  used  for  carrying  building  workers  these  are  provided

suitable and efficient automatic devices to ensure that the cage of such

hoist  comes  to  rest  at  a  point  above  the  lowest  point  to  which  such

cage may travel.

145.  Attachment of loads Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that—

(a)

when a sling is used to hoist long materials, a lifting beam is used to

space the sling legs for proper balance and when a load is suspended

78

at two or more points with slings, the eyes of the lifting legs of such

slings are shackled together and such shackle or eyes of the shackled

slings  are  placed  on  the  hook  or  the  eyes  of  such  lifting  legs  are

shackled  directly  to  the  hoisting  block,  ball  or  balance  beam,  as  the

case may be;

(b)

every container or receptacle used for raising or lowering stone, bricks,

tiles, slates or other similar objects is so enclosed with the hoist as to

prevent the fall of such objects;

(c)

a  loaded  wheel  barrow  placed  directly  on  a  platform  of  a  hoist  for

raising  or  lowering  of  such  wheel  barrows  is  so  secured  that  such

wheel barrows cannot move and such platform is enclosed to prevent

the fall of the contents kept in such wheel barrows;

(d)

landings of a hoist are so designed and arranged that building workers

on such hoist are not required to lean out into empty space for loading

and unloading any material from such hoist.

146.  Tower Cranes Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that--

(a)

no  person  other  than  the  operator  trained  and  capable  to  work  at

heights are employed to operate tower cranes;

(b)     the  ground  on  which  a  tower  crane  stands  has  adequate  bearing

capacity;

(c)

bases  for  tower  cranes  and  trucks  for  rail-mounted  tower  cranes  are

firm  and  leveled  and  such  cranes  are  erected  at  a  reasonably  safe

distance  from  excavations  and  are  operated  within  gradient  limits  as

specified by the manufacturer of such cranes;

(d)     tower  cranes  are  sited  where  there  is  a  clear  space  available  for

erection, operation and dismantling of such cranes;

(e)

tower cranes are sited in such a way that the loads  on such cranes are

not   handled   over   any   occupied   premises,   public   thoroughfares,

railways or near power cables, other than construction works for which

such cranes are used;

79

(f)

where two or more tower cranes are sited and operated every care is

taken to ensure positive and proper communication between operators

of such cranes to avoid any danger or dangerous occurrences;

(g)

tower  cranes  are  used  for  loading  magnet  or  demolition  ball  service,

piling  operation  or  other  similar  operations  which  could  impose

excessive load stress on the crane structure of such cranes;

(h)

the  instructions  of  the  manufacturer  of  a  tower  crane  and  standard

safe  practices  regarding  such  crane  are  followed  while  operating  or

using such crane.

147.   Qualification  of  operator  of  lifting  winches  and  of  signaler,  etc  Sec.

40

The employer shall ensure at a construction site of a building or other

construction  work  that  no  person  is  employed  to  drive  or  operate  a  lifting

appliance whether driven by mechanical power or otherwise or to give signals

to  drive  or  operator  of  such  lifting  appliance  or  to  work  as  a  operator  of  a

rigger or derricks unless he--

(i)

is above eighteen years of age:

(ii)

is sufficiently competent and reliable;

(iii)

possesses the knowledge of the inherent risks involved in the operation

of lifting appliances; and

(iv)    is   medically   examined   periodically   as   specified   in   Schedule-II

annexed to these rules.

CHAPTER-VII

(Part-IV)

Runways And Ramps

148.  Use of runways and ramps by building works Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that--

(a)

runway or ramp provided for use by building workers is not less than

four hundred and thirty millimeters in width and is constructed of not

less than twenty five millimeter thick planking or any other material of

adequate   strength   to   withstand   the   required   load   supported

substantially in relation to the span and braced of such runway or ramp

80

and design and construction of such runway or ramp is in accordance

with the relevant national standards;

(b)

every  runway  of  ramp  provided  for  use  of  building  workers  located

more  than  three  meters  above  the  floor  or  ground  is  on  open  sides

provided with a guard rail of adequate strength and height of not less

than one thousand millimeters.

149.  Use by Vehicles Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that--

(a)

all  runways  and  ramps  are  of  sound  construction  strength  and  are

securely braced and supported;

(b)

every runway or ramp for the use of transport equipment like trailers,

trucks  or  heavier  vehicles  has  a  width  of  not  less  than  three  point

seven meters and is provided with timber curbs or any other material

of  adequate  strength  with  not  less  than  two  hundred  millimeters  by

two hundred millimeters in width placed parallel to, and secured to, the

sides of such runway or ramp and such runways or ramps are designed

in accordance with the relevant national standards.

150.  Slope of Ramps Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that every ramp has a slope not exceeding one in four and

the total rise of a continuous ramp used by building workers carrying material

or  using  wheel-barrows  does  not  exceed  three  point  seven  meters,  unless

broken by horizontal landing of at least one point two meters in length or as

provided in accordance with the relevant national standards.

151.  Use by wheel barrows, etc Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that--

(a)

every  runway  or  ramp  used  for  wheel  barrows,  hand  carts  or  hand

trucks is not less than one meter in width and is constructed of not less

than  fifty  millimeters  thick  planking  and  is  supported  and  braced

suitably for such use;

81

(b)

every runway or ramp located more than three meters above the floor

or  ground  is  provided  on  the  open  sides  with  suitable  guard  rails  of

adequate strength.

CHAPTER-VII

(Part-V)

Work On Or Adjacent To Water

152.  Transport by water Sc 40

(1)

The employer shall ensure at a construction site of a building or other

construction work that—

(a)     when  any  building  worker  has  to  proceed  to  or  from  any

working place by water for purposes of carrying on a building or

other construction work, proper measures are taken to provide

for  his  safe  transportation  and  vessels  used  for  such  purpose

are  used  in  charge  of  a  responsible  person,  and  are  properly

equipped  for  safe  navigation  and  are  maintained  in  good

condition;

(b)

maximum  number  of  persons  which  can  be  safely  carried  in  a

vessel  as  certified  under  the  relevant  law  in  force  is  marked

plainly and conspicuously on such vessel and such number is not

exceeded during use of such vessel for carrying persons.

(2)

The vessel referred to in clause (a) of sub-rule (1) shall conform to the

following, namely:

(i)

that  adequate  protection  is  provided  to  the  building  workers  in

such vessel from inclement weather;

(ii)

that such vessel is manned by adequate and experienced crew,

as per the relevant law for the time being in force;

(iii)

that  in  case  the  bulwarks  of  such  vessel  are  lower  than  sixty

centimeters from the level of the deck of such vessel, the open

edge  of  such  bulwarks  are  fitted  with  suitable  fencing  to  a

height of at least one meter above such deck and the post and

stanchions and similar parts used in such fencing are not spaced

more than two meters apart;

82

(iv)

that the number of life buoys on deck of such vessel is at least

equal to the number of crew members of such vessel and is not

less than two;

(v)

that all life buoys on deck of such vessel are kept in good state

of maintenance and are so placed that if such vessel sinks then

they  remain  to  float  and  one  of  such  buoys  is  within  the

immediate reach of the Steerman of such vessel and another is

situated after part of such vessel; and

(vi)

that the position of the Steerman of the vessel is such that he

has a reasonably free view of all sides.

153.  Prevention from drowning Sec. 40

The  employer  shall  ensure  at  a  construction  site  of  building  or  other

construction  work  that  where,  on  or  adjacent  to  the  work  place  of  any

construction  site  to  which  these  rules  apply,  there  is  water  into  which  a

building  worker  employed  for  work  on  such  site  is,  in  the  course  of  his

employment,  may  fall  and  has  the  risk  of  drowning,  suitable  rescue

equipment  is  provided  and  kept  in  an  efficient  state  for  ready  use  and

measures are taken to arrange for the prompt rescue of such building worker

from  the  danger  of  drowning  and  where  there  is  a  special  risk  of  such  fall

from the edge of adjacent land or from a structure adjacent to or above the

water or from floating stage on such water, secure fencing is provided near

the  edge  of  such  land,  structure  or  floating  stage,  as  the  case  may  be,  to

prevent  such  fall,  and  such  fencing  may  be  removed  or  allowed  to  remain

unerected for the time and to the extent necessary for the access of building

workers to such work or the movement of material for such work.

CHAPTER-VII

(Part-VI)

Transport And Earth Moving Equipment

154.  Earth moving equipment and vehicles Sec. 40

The  employer  shall  ensure  at  a  construction  site  of  building  or  other

construction work that--

(a)

all  vehicles  and  earth  moving  equipment  are  made  of  good  material,

proper  design  and  sound  construction  and  are  sufficiently  strong  for

83

the purpose for which such equipment are used and are maintained in

good state of repair and are properly used in accordance with standard

safe operating practices:

Provided  that  the  truck  or  trailer  employed  for  transporting

freight  containers  are  of  the  size  sufficient  to  carry  the  containers,

without  overhanging  and  are  provided  with  twist  locks  conforming  to

national  standards,  at  all  the  four  corners  of  each  of  such  truck  or

trailers  and  such  truck  or  trailers  are  certified  for  such  use  by  an

authority  under  the  relevant  law  for  the  time  being  in  force  and  is

inspected by a responsible person, at least once in a month and record

of such inspection is maintained;

(b)

all  transport  or  earth  moving  equipment  and  vehicles  are  inspected

alteast once a week by a responsible person and in case any defect is

noticed  in  such  equipment  or  vehicle,  it  is  immediately  taken  out  of

use;

(c)

power  trucks  and  tractors  are  equipped  with  effective  brakes,  head

lights  and  tail  lamps  and  are  maintained  in  good  repair  and  working

order;

(d)

side  stanchions  on  power  trucks  and  trailers  for  carrying  heavy  and

long objects are--

(i)

of sound construction and free from defects;

(ii)

provided with tie chains attached to the top across the loads for

preventing such stanchions from spreading out; and

(iii)

Kept in position while loading and unloading;

(e)

safe  gangways  are  provided  for  to  and  fro  movement  of  building

workers  engaged  in  loading  and  unloading  of  lorries,  trucks,  trailers

and wagons;

(f)

trucks and other equipment are not loaded beyond their safe carrying

capacity  which  shall  be  clearly  marked  on  such  trucks  and  other

equipment;

(g)

handles of hand trucks are so designed as to protect the hands of the

building workers working on such trucks, or such handles are provided

with knuckles guards;

(h)

no  unauthorized  person  rides  the  transport  equipment  employed  in

such work;

84

(i)

a  driver  of  a  transport  equipment  manoeuvres  such  equipment  under

the direction of a signaler;

(j)

adequate  precaution  such  as  isolating  the  electric  supply  or  erecting

overhead  barriers  of  a  safe  height  is  taken  when  earth  moving

equipment or vehicles are required to operate in dangerous proximity

to any live electric conductor;

(k)

vehicles and earth moving equipments are not left on a slope with the

engine of such vehicles or equipment running;

(l)

all  earth  moving,  equipments,  vehicles  or  other  transport  equipment

are  operated  only  by  such  person  who  are  adequately  trained  and

possess such skill as are required for safe operation of such equipment,

vehicle or other transport equipment.

155.  Power shovels and excavator Sec. 40

The  employer  shall  ensure  at  a  construction  site  of  building  or  other

construction work that--

(a)

a shovel or an excavator whether operated, by steam or electric or by

internal  combustion,  used  for  such  work  is  constructed,  installed,

operated, tested and examined as required under any law for the time

being in force and the relevant national standards;

(b)

excavator equipped for use as a mobile crane is--

(i)

examined  and  tested  in  accordance  with  the  requirements  for

such mobile crane under these rules; and

(ii)

fitted with an automatic safe working load indicator;

(c)     buckets  or  grabs  of  powers  shovels  are  propped  to  restrict  the

movement of such buckets or grabs while being repaired or while the

teeth of such buckets or grabs are being changed.

156.  Bulldozers Sec.

The employer shall ensure at a construction site of a building or other

construction work that—

(a)

an operator of a bulldozer before leaving such bulldozer--

(i)     applies

(ii)

lowers the blade and sipper; and

(iii)

puts the shift lever into neutral;

85

(b)

a  bulldozer  is  left  on  level  ground  at  the  close  of  the  work  for  which

such bulldozer is used;

(c)

the  blade  of  a  bulldozer  is  kept  low  when  such  bulldozer  is  moving

uphill;

(d)

the bulldozer blades are not used as brakes except in an emergency.

157.  Scrapers Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that--

(a)

a  tractor  and  scraper  is  joined  by  safety  line  at  the  time  of  its

operation;

(b)

the scraper bowls are propped while blades of such scraper are being

replaced;

(c)

a scraper moving downhill is left in gear.

158.  Mobile asphalt layer and finishers Sec. 40

The employers shall ensure on a construction site of a building or other

construction work that-

(a)

a mixture elevator is within a wooden or sheet metal enclosure with a

window for observation, lubrication and maintenance;

(b) 
(c)

bitumen scoops have adequate covers;

when  asphalt  plants  are  working  on  a  public  road,  adequate  traffic

control  is  established  on  such  road  and  the  building  workers  working

with such plant are provided with reflecting jackets;

(d)

a sufficient number of fire extinguishers are kept in readiness on such

work place where fire hazards may exist;

(e)

the  materials  are  loaded  on  the  elevator  after  the  drying  drain  has

warmed up of such elevator;

(f) 
(g)

no open light is used for ascertaining the level of asphalt;

inspection  opening  is  not  opened  till  there  is  a  pressure  in  the  boiler

which may cause injury to a building worker;

159.  Paves Sec. 40

The employer shall ensure at a construction site of a building or other

construction  work  that  pavers  are  equipped  with  guards  suitable  to  prevent

building workers from walking under the skip of such pavers.

86

160.  Road rollers Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that-

(a)

before  a  road  roller  is  used  on  the  ground,  such  ground  is  examined

for  its  bearing  capacity  and  general  safety,  especially  at  the  edges  of

slopes such as embankments on such grounds;

(b)

a roller is not moved downhill with the engine out of gear.

161.  General Safety Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that:-

(a)

every vehicle or earth  moving equipment is equipped with  -

(i)     silencers;

(ii)     tail

(iii)

power and hand brakes;

(iv)

reversing alarm; and

(v)

search  light  for  forward  and  backward  movement,  which  are

required  for  safe  operation  of  such  vehicle  or  earth  moving

equipment;

(b)

the  cab  of  vehicle  or  earth  moving  equipment  is  kept  at  least  one

meter from the adjacent face of a ground being excavated; and

(c)

When  a  crane  or  shovel  are  traveling,  the  boom  of  such  crane  or

shovel  is  in  the  direction  of  such  travel  and  the  bucket  of  scoop

attached  to  such  crane  or  shovel  is  raised  and  without  load,  except

when such traveling is downhill.

CHAPTER –VII

(Part-VII)

Concrete Work

162.  General provisions regarding use of concrete Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that:-

(a)

all  construction  with  the  use  of  concrete  or  reinforced  concrete  are

based on plans as –

87

(i)

include specifications of steel and concrete and other material to

be used in such construction;

(ii)

give  technical  details  regarding  methods  for  safe  placing  and

handling of such materials as specified in sub-clause(I);

(iii)

indicate  the  type,  quality  and  arrangement  of  each  part  of  a

structure of such construction; and

(iv)

explain the sequence of steps to be taken for completion of such

construction.

(b)

form work and shores used for concrete work are structurally safe and

are  properly  braced  or  tied  together  so  as  to  maintain  position  and

shape of such form work or shares;

(c)

form  work  structure  used  for  concrete  work  has  sufficient  cat  walks

and  other  secure  access  for  inspection  of  such  structure  if  such

structure is in two or more tiers.

163.  Preparation  and  pouring  of  concrete  and  erection  of  concrete

structure  Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that:-

(a)

a building worker handling cement or concrete –

(i)

wears  close  fitting  clothing,  gloves,  helmet  or  hard  hat,  safety

goggles, proper foot wear and respirator or mask to protect him

from danger in such handling;

(ii)

keep as much of his body covered as is required to protect him

from danger in such handling;

(iii)

takes all necessary precautions to keep cement and   concrete

away from his skin in such handling ;

(b)

lime pits are fenced or enclosed;

(c)

lime pits are filled and emptied by such devices which do not require

workers to go into the pit;

(d)    moving  parts  of  the  elevators,  hoists,  screens,  bunkers,  chutes,

grouting  equipment  used  for  concrete  work  and  of  other  equipment

used for storing, transport and other handling ingredients of concrete

are  securely  fenced  to  avoid  contact  of  building  workers  with  such

moving parts;

88

(e)

screw conveyors used for cement, lime and other dusty materials   are

completely enclosed.

164.  Buckets Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that –

(a)

concrete  buckets  used  with  cranes  or  aerial  cableways  are  free  from

projections from which accumulations of concrete could fall;

(b)

movements of concrete buckets are governed by signals necessary to

avoid any danger by such movements.

165.  Pipes and pumps Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that:-

(a)

a scaffolding carrying a pipe for pumped concrete is storing  enough to

support such pipe at time when such pipe is filled with

concrete

or water or any other liquid and to bear all the building

workers

who may be on such scaffold at such time, safely;

(b)

every pipe for carrying pumped concrete is –

(i)

securely anchored at its end point and at each curve on it;

(ii)

provided near the top of such pipe with an air release valve; and

(iii)

securely  attached  to  a  pump  nozzle  by  a  bolted  collar  or  other

adequate means;

(c)

the  operation  of  concrete  pumps  are  governed  by  standard  signals

relevant in accordance with the relevant national standards;

(d)     building  workers  employed  around  a  concrete  pump  wear  safety

goggles.

166.  Mixing and pouring of concrete Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that:-

(a)

the concrete mixture does not contain, any material which may unduly

affect  the  setting  of  such  concrete  weaken  such  concrete  or  corrode

steel used with such concrete;

(b)

when  dry  ingredients  of  concrete  are  being  mixed  in  confined  spaces

such as silos;

89

(i)

the dust shall be exhausted at the time of such mixing; and

(ii)

in case the dust cannot be exhausted, as specified in sub clause

(i)  the  building  workers  shall  wear  respirators  at  the  time  of

such mixing;

(c)

when concrete is being tipped from buckets, building workers are kept

out of the range of any kickbacks of such buckets;

(d)

loads are not dumped or placed on settling concrete.

167.  Concrete panels and slabs Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that:-

(a)

all parts of a concrete panel or concrete slab are hoisted uniformly;

(b)

concrete panels are adequately braced in their final positions and such

bracings shall remain in such position until such panels are adequately

supported by other parts of the construction for which such panels are

used;

(c)

temporary bracing of concrete panels are securely fastened to prevent

any  part  of  such  panels  from  falling  when  such  panels  are  being

moved.

168.  Stressed and tensioned elements Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that:-

(a)

building  workers  do  not  stand  directly  over  jacking  equipment  while

stressing of concrete girders and beams is being done;

(b)

a  prestressed  concrete  unit  is  not  handled  except  at  points  on  such

unit and by the devices specified for such work by the manufacturer of

such devices;

(c)

during transport prestressed concrete girders or concrete beams

are

kept upright by bracing or other effective means;

(d)

anchor fittings for pretensioned strands of prestressed concrete girders

or concrete beams are kept in a safe condition in accordance with the

instructions of manufacturer of such anchor fittings;

(e)

building  workers  do  not  stand  behind  jacks  or  in  line  with  tensioning

elements  and  jacking  equipment  during  tensioning  operations  of  pre

stressed concrete girders  or concrete or concrete beams;

90

(f)

Building workers do not cut wires of prestressed concrete beams under

tension  before  such  concrete  used  for  such  girders  or  beams  is

sufficiently hardened.

169.  Vibrators Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that:-

(a)

a building worker, who is in good physical condition, operates vibrators

used in concreting work;

(b)

all  practical  measures  are  taken  to  reduce  the  amount  of  vibration

transmitted to the operators working in concreting work;

(c)

when electric vibrators are used in concreting work:-

(i)

such vibrators shall be earthed;

(ii)

the leads of such vibrators shall be heavily insulated; and

(iii)

the current shall be switched off when such vibrators are

not

in use.

170.  Inspection and supervision Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that:-

(a)

a  person  responsible  for  a  concreting  work  supervises  the  erection  of

the  formwork,  shores,  graces  and  other  supports  used  for  such

concreting work;

(b)

a person responsible for concreting work makes a thorough inspection

of every formwork after erection of such formwork in such concreting

work to ensure that such formwork is safe;

(c)

a  person  responsible  for  a  concreting  work  regularly  inspects  the

formwork,  shores,  braces,  reshores  and  other  supports  during  the

placing of concrete;

(d)     any  unsafe  condition  which  is  discovered  during  the  inspections

mentioned under clause (b) and (c) is remedied immediately;

(e)      a  person  responsible  for  a  concreting  work  keeps  all  record  of

inspections referred to in clause (a) and clause (b) at the workers place

relating to such inspection and produces them for inspection upon the

demand of an inspector having jurisdiction.

91

171.  Beans, floors and roofs Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that:-

(a)

horizontal and diagonal bracings are provided to both longitudinal and

transverse directions as may be necessary to provide structural stability

to  formwork  used  in  concreting  work  and  shores  used  in  such

concreting work are properly seated top and bottom and are secured in

their places;

(b)

where  shores  used  in  concreting  work  rest  upon  the  ground,  base

plates are provided for keeping such shores firm and in level;

(c)

where  the  floor  to  ceiling  height  of  a  concreting  work  exceeds  nine

meters  or  where  the  formwork  deck  used  in  such  concreting  work  is

supported  by  shores  constructed  in  two  or  more  tiers,  or  where  the

dead  live  and  impact  loads  on  the  formwork  used  in  such  concreting

work exceed seven hundred kilogram per square meter., the structure

of such formwork is designed by a professional engineer in the relevant

field and the specifications and drawings of such form work are kept at

such  construction  site  and  produced  on  demand  before  the  inspector

having jurisdiction;

(d)

where  the  structure  of  the  formwork  used  in  concreting  work  is

designed   by   a   professional   engineer,   such   engineer   shall   be

responsible  for  the  supervision  of  construction  and  stability  of  such

structure.

172.  Stiffing Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that:-

(a)

striping  of  formwork  used  in  concreting  work  commences  until  the

concrete on such formwork is fully set, examined and certified to this

effect  by  the  responsible  person  and  record  of  such  examination  and

certification is maintained;

(b)

stripped forms in concreting work are removed or stock piled promptly

after stripping from all areas in which building workers are required to

work or pass;

(c)

protruding nails wire ties and other formwork accessories not required

for subsequent concreting work are pulled, cut or otherwise made safe.

92

173.  Reshoring Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that:-

(a)

reshoring used in concreting work is provided to a slab or beam for its

safe support after its stripping or where such slab or beam is subjected

to superimposed loads due to construction above such slab or beam;

(b)

the  provisions  applicable  to  shoring  in  a  concreting  work  under  this

chapter shall also be applicable to reshoring in such work.

CHAPTER VII

(Part-VIII)

Demolition

174.  Preparation

The employer shall ensure at a construction site of a building or other

construction  work  that  all  glass  or  similar  material  or  article  in  exterior

openings are removed before commencing any demolition work and all water,

steam  electric  gas  and  other  similar  supply  lines  are  put  off  and  suitably

gapped  and  the  concerned  department  of  the  appropriate  Government  or

local authority is informed and permission obtained wherever required before

commencing  such  demolition  work  and  wherever  it  is  necessary  to  maintain

water, gas or electric line or power during such demolition, such line shall be

so  located  or  protected  with  substantial  coverings  so  as  to  protect  it  from

damage and to afford safety to the building workers and the general public.

175.  Protection of adjacent structure Sec. 40

The employer responsible for a demolition work at a construction site

of  a  building  or  other  construction  work  shall  ,  during  demolition  process  of

such  demolition  work,  examine  the  walls  of  all  structures  adjacent  to  the

structure to be demolished to determine the thickness, method of support to

such  adjacent  structures  and  in  case,  such  employer  has  reason  to  believe

that  any  of  such  adjacent  structure  is  unsafe  or  may  become  unsafe  during

such  demolition  process,  he  shall  not  perform  demolition  activity  affecting

such unsafe adjacent structure unless and until remedial measures like sheet

93

piling,  shoring,  bracing,  or  similar  other  means  so  as  to  ensure  safety  and

stability to such unsafe adjacent structure from collapsing are taken.

176.  Demolition of walls, partitions, etc Sec 40

The employer shall ensure at a construction site of a building or other

construction work that:-

(a)

any  demolition  of  walls  or  partitions  is  proceeded  in  a  systematic

manner  as  per  the  standard  safe  operating  practices  and  all  work

above  each  tier  of  any  floor  beams  is  completed  before  the  safety  of

the supports of such beam is impaired;

(b)

masonry  is  neither  loosened  nor  permitted  to  fall  in  such  masses  or

volume or weight as to endanger the structural stability of any floor or

structural support;

(c)

no  wall,  chimney  or  other  structure  or  part  of  a  structure  is  left

unguarded in such a condition that it may fall, collapse or weaken due

to wind pressure or vibration;’

(d)

in  the  case  of  demolition  of  exterior  walls  by  hand,  safe  footing  is

provided for the building workers employed for such demolition, in the

form of sound flooring or scaffolds; and

(e)

walls  or  partitions  which  are  to  be  demolished  by  hand  are  not  left

standing  more  than  one  storey  high  above  the  uppermost  floor  on

which persons are working.

177.  Methods of operation Sec. 40

The employer shall ensure at a construction site of a building or other

construction  work  that  debris,  bricks  and  other  materials  or  articles  are

removed -

(a) 
(b) 
(c)

(d)

by means of chutes;

by means of buckets or hoists;

through openings in the floors; or

by any other safe  means;

178.  Access of floors Sec. 40

The employer shall ensure at a construction site of a building or other

construction  work  that  safe  access  to  and  egress  from  every  building  is

provided at all times in the course of demolition of such building by means of

94

entrances,  hallways,  stairways  or  ladder  runs  which  are  so  protected  as  to

safeguard  the  building  workers  using  such  means  from  falling  material  or

articles.

179.  Demolition of structural steel Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that:-

(a)

all  steel  structures  are  demolished  column  by  column  and  tier  by  tier

and  every  structural  member  which  is  being  demolished  is  not  under

any stress and such structural member is suitably lashed to prevent it

from any uncontrolled swinging or dropping or falling;

(b)

large structural members are not thrown or dropped from the buildings

but are carefully lowered by adopting suitable safe method;

(c)

where a lifting appliance like a derrick is used for demolition, the floor

on  which  such  lifting  appliance  rests  is  completely  planked  over  or

supported  and  such  floor  is  of  adequate  strength  to  sustain  bearing

load for such lifting appliance and its operation.

180.  Storage of material or article Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that:-

(a)

all  materials  or  articles  are  not  stored  or  kept  on  platform,  floor  or

stairways of a building being demolished;

Provided that this clause shall not apply to the floor of a building

when such floor is of such strength as to support safety the load to be

superimposed by storing such materials or articles;

(b)

an access to any stairway or passage way is not affected or blocked by

storing any material or article;

(c)

suitable  barricades  are  provided  so  as  to  prevent  materials  or  articles

from  sliding  or  rebounding  into  any  space  used  by  the  building

workers.

181.  Floor opening Sec. 40

The employer shall ensure at a construction site of a building or other

construction  work  that  every  opening  used  for  the  removal  of  debris  from

every  floor  which  is  not  closed  to  access,  except  the  top  or  working  floor  is

95

provided with an enclosure from such floor to its ceiling or such opening is so

barricaded that no building worker has access to within a horizontal distance

of six meters from such opening through which debris is being dropped.

182.  Inspection Sec. 40

The employer shall ensure at a construction site of a building or other

construction  work  that  a  person  responsible  for  demolition  work  makes

continuous inspections during demolition process of such demolition work so

as  to  detect  any  hazard  resulting  from  weakened  or  deteriorated  floors  or

walls or loosened materials or articles during such demolition process and that

no  building  worker  is  permitted  to  work  where  such  hazard  exist  unless

remedial measures like shoring or bracing are taken to prevent such hazards.

183.  Warning signs, barricades, etc Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that:-

(a)

barricades and warning signs are erected along every side throughout

the length and breadth of a building or other construction work to be

demolished  to  prevent  unauthorized  persons  from  entering  into  the

side  of  such  building  or  other  construction  work  during  demolition

operations;

(b)

during  the  demolition  of  an  exterior  masonry  wall  or  a  roof  from  a

point  more  than  twelve  meters  above  the  adjoining  ground  level  of

such wall or roof.  If persons below such wall or roof are exposed to

falling objects, suitable and safe catch platforms shall be provided and

maintained at a level not more than six meters below the working level

except  where  an  exterior  built  up  scaffold  is  provided    for  safe  and

adequate protection of such persons;

(c)

suitable  and  standard  warning  signs  in  accordance  with  national

standards are displayed or erected at conspicuous places or position at

the workplace.

184.  Mechanical methods of demolition Sec. 40

The employer shall ensure at a construction site of a building or other

construction  work  that  the  following  requirements  are  fulfilled  in  case  the

mechanical  method  of  demolition  like  use  of  swinging  weight,  clamshell

96

bucket, power shovel, bulldozer or other similar mechanical methods are used

for the purpose of demolition; namely:-

(a)

that the building or structure or remaining portion thereof shall be not

more than twenty- four meters in height;

(b)

that  where  a  swinging  weight  is  used  for  demolition,  a  zone  of  such

demolition having a radius of at least one and a half times the height

of  the  structure  or  portion  thereof  being  so  demolished  shall  be

maintained around the points of impact of such swinging weight;

(c)

where  a    clamshell    bucket  is  being  used  for  demolition  a  zone  of

demolition shall be maintained within eight meters of the line of travel

of such bucket;

(d)

that where other mechanical methods are being used to  affect total or

partial collapse of a building or other construction work, there shall be

maintained, in the area into which the affected portion of such building

or  other  construction  work  may  fall  a  zone  of  demolition  at  least  one

and a half times the height of such affected portion thereof; and

(e)

no person other than building workers or other persons essential to the

operation  of  demolition  work  shall  be  permitted  to  enter  a  zone  of

demolition  referred  to  in  clause  (a)  which  shall  be  provided  with

substantial barricades.

CHAPTER VII

(Part-IX)

Excavation And Tunnelling Works

185.  Notification of intention to carry out excavation and tunneling work

Sec. 40

(I)

Every  employer  carrying  out  any  excavation  or  tunnelling  work  at  a

construction site of a building or other construction work shall, within

thirty  days  prior  to  the  commencement  of  such  excavation  or

tunnelling work, inform in writing the detailed layout plans, method of

construction  and  Schedule     of  such  excavation  or  tunnelling  work  to

the Chief Inspector.

97

(2)  In  case  compressed  air  is  used  in  such  excavation  or  tunnelling

work  or  any  work  incidental  to  or  required  for  such  excavation  or

tunnelling work, the technical details and drawings of all man locks and

medical  locks  together  with  names  and  addresses  of  all  construction

medical  officers  having  qualification  as  laid  down  in  Schedule-V

annexed  to  these  rules  and  so  appointed  by  such  employer  for  the

purpose  of  such  excavation  or  tunnelling  work  shall  be  sent  to  the

Chief Inspector.

186.  Project Engineer Sec. 40

(I)

Every  employer  undertaking  any  excavation  or  tunnelling  work  shall

appoint a project engineer for safe operation of such projects of such

excavation or tunnelling work for which such engineer is appointed.

(2)

Such  project  engineer  shall  exercise  overall  control  of  operation  and

the  activities  at  such  project  and  be  responsible  for  carrying  out  the

activities safely.

187.  Responsible person Sec. 40

(I)      Every  employer  undertaking  excavation  or  tunnelling  work  at  a

construction site of a building or other construction work shall appoint

a  responsible  person  for  safe  operation  for  such  excavation  or

tunnelling work

(2)

Duties and responsibilities of the responsible person referred to in sub

rule (I), person shall include-

(a) 
(b)

to carry out smoothly such excavation or tunnelling work;

to  inspect  and  rectify  any  hazardous  situation  relating  to  such

excavation or tunnelling work;

(c)

to  take  remedial  measures  to  avoid  any  unsafe  practice  or

conditions relating to such excavation or tunnelling work;

(3)

the name and address of  the   responsible person referred to in sub-

rule (1) shall be forwarded to the Chief Inspector.

188.

and notices Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that -

(a)

suitable  warning  signs  or  notices,  required  for  the  safety  of  building

workers carrying out the work of an excavation or tunnelling, shall be

98

displayed or erected at conspicuous places in Hindi and in a language

understood by the majority of such building workers at such excavation

of tunnelling work;-

(b)

such warning signs and notices with regard to compressed air working

shall include –

(i)

the danger involved in such compressed air work;

(ii)

fire and explosion hazards;

(iii)

the emergency procedures for rescue from such danger or

hazards.

189.  Register of employment etc Sec. 40

(I)

Every employer shall ensure that at a construction site of a building or

other  construction  work  where  an  excavation  or  tunnelling  work  is

being carried on, a register of employment of building workers carrying

out such excavation or tunnelling work, is maintained and produced on

demand to the Inspector having jurisdiction.

(2)

periods of work of such excavation or tunnelling work; in which such

building worker are employed shall be maintained in a register on day-

to-day  basis  and  such  register  shall  be  produced  on  demand  to  the

Inspector having jurisdiction.

190.  Illumination

(I)

Every employer shall ensure that at a construction site of a building or

other  construction  work  that  all  work  places  where  excavation  or

tunnelling  works  are  carried  out  shall  be  adequately  illuminated  in

accordance with a relevant national standards.

(2)     Every  employer  carrying  out  excavation  or  tunnelling  works  at  a

construction site of building or other construction work shall provide for

emergency  generators  on  such  construction  site  to  ensure  adequate

illumination  at  all  work  places  where  such  excavation  or  tunnelling

work is being carried out, in case of power failure.

191.  Stability of structure Sec. 40

Every employer shall ensure that at a construction site of a building or

other construction work that--

99

(a)

where there is any doubt as to the stability of any structure adjoining

the  work  place  or  other  areas  to  be  excavated  or  where  tunnelling

work is to be carried out, the project engineer referred to in rule 186

arranges for measures like underpinning, sheet piling, shoring, bracing

or other similar means to support such structure and to prevent injury

to any building worker working adjacent to such structure or damage

to property or equipment adjacent to such structure;

(b)

where any building worker engaged in excavation is exposed to hazard

of  falling  or  sliding  material  or  article  from  any  bank  or  side  of  such

excavation which is more than one and a half metre above his footing,

such workers is protected by adequate piling and bracing against such

bank or side;

(c)

the  excavation  and  its  vicinity  are  checked  by  a  responsible  person

referred  to  in  rule  187  after  every  rain,  storm  or  other  occurrences

carrying  hazards  and  in  case  a  hazard  is  noticed  at  such  checking

adequate protection against slides and cave into prevent such hazard is

provided;

(d)

temporary sheet piling installed for the construction of a retaining wall

after excavation is not removed except on the advice of the responsible

person referred to in  rule 187 after an inspection carried out by such

responsible person;

(e)     where  banks  of  an  excavation  are  undercut,  adequate  shoring  is

provided to support the material or article over hanging such bank;

(f)

excavated material is not stored at least zero point six five meters from

the  edge  of  an  open  excavation  or  trench  and  the  banks  of  such

excavation  or  trench  are  stripped  of  loose  rocks  and  other  materials

which may slide, roll or fall upon a building worker working below such

bank;

(g)

adequate and suitable warning signs are put up at conspicuous places

at the excavation work to avoid any person falling in to the excavations

or trenches;

(h)    the  responsible  person  referred  to  in  rule  187  ensures  at  the

excavation  work  that  no  building  worker  is  permitted  to  work  where

such building worker may be struck or endangered by the excavation

machinery or material or article used in such excavation;

100

192.  Pilling, shoring and bracing Sec. 40

Every employer shall ensure that at a construction site of a building or

other construction work that --

(a)

plank used for sheet piling in excavation or tunnelling work is of sound

material with adequate strength;

(b)     shores  and  braces  used  in  excavation  or  tunnelling  work  are  of

adequate  dimensions  and  are  so  placed  as  to  be  effective  for  their

intended purposes;

(c)

earth  supported  shores  or  braces  used  in,  excavation  or  tunnelling

work  bear  against  a  footing  of  sufficient  area  and  stability  to  prevent

the shifting of such shores or braces.

193.  Safe access Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that ladders, stair cases or ramps are provided, as the case

may be, for safe access to and egress from excavation  where the depth of

such excavation exceeds one point five metres and such ladders, stair cases

or ramps comply with the relevant national standards.

194.  Trenches

The employer shall ensure at a construction site of a building or other

construction work that a trench  or excavation  is protected against falling of

a  person  by  suitable  measures  if  the  depth  of  such  trench    or  excavation

exceeds one and a half metre and such protection is an  improved protection

in accordance with the design and drawing of a professional engineer, where

such depth exceeds four metres.

195.  Depth of trenches Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that-

(a)

where the depth of a trench requires two lengths of sheet piling, one

above  the  other,  the  lower    piling  is  set  inside  the  bottom  strings  or

wales  of  the  upper  piling  and  such  sheet  piling  is    driven  down  and

braced as the excavation continues;

101

(b)

all metal sheet piles used in excavation or a trench are welded end to

end and secured by other similar means.

196.  Positioning and use of machinery Sec. 40

The employer shall ensure at a construction site of a building  or other

construction work that any machinery used in excavation  and tunnelling work

is  positioned  and  operated  in  such  a  way  that  such  machinery  does  not

endanger the operator of such machinery or any other person in the vicinity.

197.  Breathing apparatus Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that-

(a)

suitable  breathing  apparatus  is  provided  to  a  building  worker  while

working  in  compressed  air  environment  for  his  use  at  excavation  or

tunnelling work; and

(b)

such breathing apparatus is maintained in good working condition at all

times.

198.  Safety measures dor tunneling operation Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that.-

(a)

where there is a danger of falling or sliding of material from the roof

face  or  wall  of  a  tunnel,  adequate  measures  such  as  shoring,

supporting by means of rock bolts, segments or steel sets are taken for

the safety of building workers;

(b)

the  excavated  areas  are  made  safe  by  use  of  suitably  designed  and

installed steel sets, rock bolts or similar other safe means;

(c)

the responsible  person referred to in rule 187 examines and inspects

the workplaces in a tunnel before the commencement of work in such

tunnel,  and  at  regular  intervals  thereafter,  to  ensure  safety  of  the

building workers in such tunnel; and

(d)

the portal areas of a tunnel with loose soil or rock, likely to cause injury

to a person are adequately  protected with supports.

102

199.  Pneumatic tools Sec. 40

The employer shall ensure at a construction site of a building or other

construction  work  that  supply  lines  to  pneumatic  tools  used  within  a  tunnel

are fitted with water trap or safety chain or safety wire, as the case may be.

200.  Shafts Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that-

(a)   surroundings  of  a  shaft  used  in  excavation  or  tunnel  work    are

protected from being washed away by construction of sufficient height;

(b)

where a building worker is required to enter a  shaft at an excavation or

tunnelling work, safe means of access is provided for such entry;

(c)    every  shaft  at  excavation  or  tunnelling  work  is  provided  with  a  steel

casing,  concrete  piping,  timber  shoring  or  other  materials  of  adequate

strength for the safety of building workers working in such shaft;

(d)    such    casing  and  bracing  are  provided  to  a  shaft  at  an  excavation  or

tunnelling  work  up  to  the  depth  of  such  shaft  at  an  excavation    or

tunnelling    work  according  to  the  appropriate  design  for  such  casing

and bracing; and

(e)

a reinforced concrete raft and beam is provided around the opening of

a  shaft  at  an  excavation  or  tunnelling  work  if  the  ground  surrounding

such opening is unstable or unsafe.

201.  Lift for shafts Sec. 40

The employer shall ensure at a construction site of a building or other

construction  work  that  lift  is  provided  for  transport  of  building  workers  and

materials or articles at an excavation  or tunnelling work required to descend

more than fifty metres in a shaft.

202.  Means of communication Sec. 40

The  employer  shall  ensure  at  a  construction  site  of  building  or  other

construction work that-

(a)

reliable  and  effective  means  of  communication  such  as  telephone  or

walkie-talkie    are  provided  and  are  maintained  in  working  order  for

103

arranging  better  and  effective  communication  at  an  excavation  or

tunnelling work at the following locations, namely,-

(i)

working chamber at the face of an excavation;

(ii)

intervals of hundred  metres along the tunnel;

(iii0

working chamber side of a man lock near the door of such man

lock;

(iv)

interior of each chamber of a man lock;

(v)

location conspicuous a lock attendant’s station;

(vi)

a compressor plant;

(vii)

a first-aid station; and

(viii)  outside the portal or the top of a shaft;

(b)

such number of bells and whistles are made available at all time at the

locations referred to in sub-clause ( i ) to sub-clause (viii) of clause (a)

as are necessary for the safety of  person at such locations.

203.  Signals Sec. 40

The employer shall ensure at a construction site of a building or other

construction  work  that  the  standard  audio  or  video  signals  are  used  in

excavation    or  tunnelling  work  and  are  conspicuously  located  or  displayed

near  entrance  to  the  workplace    and    in  such  other  locations  as  may  be

necessary to bring such signals to notice of all building workers employed in

such excavation or tunnelling work.

204.  Clearance Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that-

(a)

the minimum  lateral clearance of half a metre is maintained between

any  part  of  a  vehicle  and  any  fixture    or  any  equipment  used  in  an

excavation    or  tunnelling  work  after  allowing  the  throw  or  swing  of

such fixture or equipment.

(b)     the  overhead  clearance  for  a  locomotive    drive  at  excavation  or

tunnelling work is not less than one point one zero  meters above the

seat  of  such  driver  and  not  less  than  two  meters  above  the  platform

where  such  driver  stands  or  of  any    other  dimension  in  accordance

with the relevant  national standard.

104

205.  Shelter Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that the adequate number of shelters for the safeguards of

the building workers are provided where, in the course of working, they are

liable to be struck by a moving vehicle or other material handling equipment

in a tunnel.

206.  Use of internal combustion engine Sec. 40

The  employer shall ensure at a construction site of a building or other

construction work that no internal combustion engine is used under ground in

excavation or tunnelling work unless such engine is so constructed that-

(a)

the air entering the engine gets cleared before entry; and

(b)

no fumes or sparks are emitted by the engine.

207.  Inflammable oils Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that inflammable oils with the flash point below the working

temperature  that  is  likely  to  be  encountered  in  a  tunnel  are  not  used  in

excavation or tunnelling work.

208.  Coupling and hoses Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that only high pressure hydraulic hoses and couplings are

used  on  hydraulic  plants  underground  and    such  hoses  and    couplings  are

adequately  protected  against  any  possible    damage  in  excavation  or

tunnelling work.

209.  Hose installation Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that all hydraulic lines and plants working at a temperature

exceeding  seventy  degree  celsius  are  protected  by  adequate  insulation  or

otherwise against accidental human contact in excavation or tunnelling work.

210.  Fire resistant hoses Sec. 40

105

The employer shall ensure at a construction site of a building or other

construction  work  that  no  fire  hydraulic  hoses  other  than  fire  resistant

hydraulic  hoses  are  used  when  hydraulically  activated  machinery  and

equipment is employed in tunnels.

211.  Flame proof equipment Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that only flame proof equipment of appropriate type as per

relevant  national standards is used where there is a danger of flammable  or

explosive  atmosphere  being prevalent inside  the tunnel.

212.  Storing of oil and fuel underground Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that-

(a)     all  oils,  greases  or  fuels  stored  ground  under  in  excavation    or

tunnelling  work  are  kept  in  tightly  sealed  containers  and  in  fire

resistant  areas  at  safe  distances  away  from  explosive  and  other

flammable chemicals;

(b)

appropriate  flame  proof  installation  is  used  in  such  storage  areasas

specified in clause(a).

213.  Use of gases under ground Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that-

(a)

petrol  or  liquified  petroleum  gas  or  any  other  flammable  substances

are not used, stored inside the tunnel except with the prior approval of

the project engineer under rule 118;

(b)

after  the  use  of  the  petroleum  or  liquified  petroleum  gas,  or  highly

inflammable      substances   referred   to   in   clause(a)   all   remaining

petroleum   or   liquified      petroleum   gas   or   highly   inflammable

substances are removed immediately from such tunnel; and

(c)     no  oxy  acetelene  is  used  in  a  compressed  air  environment  in

excavation or tunnelling work.

214.  Water for fire fighting Sec. 40

106

The employer shall ensure at a construction site of a building or other

construction work that-

(a)     adequate  number  of  water  outlets  are  provided  on  excavation  or

tunnelling work and are readily made accessible throughout the tunnel

for fire fighting  purposes  and such water outlets are maintained  for

effective fire fighting;

(b)

all air locks are equipped with fire fighting facilities  at excavation  or

tunnelling work;

(c)     an  audible  fire  alarm  is  provided  to  warn  the  building  workers

whenever a fire breaks out on an excavation  or tunnelling work;

(d)

adequate  number  and  types  of  fire  extinguishers;  in  accordance  with

relevant    national  standards,  are  provided  and  made  readily  available

to fight any outbreak of fire at an excavation of tunnelling work;

(e)

fire  extinguishers  with  vapourising    liquids  and  high  pressure  carbon

dioxide  are not used in tunnels or other confined spaces;

(f)

the  instructions  regarding  steps  to  be  followed  to  fight    out  break  of

fire,  at  an  excavation  or  tunnelling  work,  written  in  Hindi  or  local

language understood by the majority of the building workers employed

on  such  excavation  or  tunnelling  work  are  displayed  at  conspicuous

and vulnerable places of such excavation or tunnelling work.

215.  Flooding

The employer shall ensure at a construction site of a building or other

construction work that –

(a)

water tight bulkhead doors are installed at the entrance of a tunnel to

prevent flooding during a tunnelling work where more than one tunnel

is driven from a shaft.

(b)

all necessary measures are taken to ensure that no building worker is

trapped in any isolated section of a tunnel when any bulkhead door of

such tunnel is closed.

(c)

where  there  is  likelihood  of  flooding  or  water  rushing  into  a  tunnel

during  a  tunnelling  work,  arrangements  are  made  for  immediate

starting  of  water  pumps  to  take  out  water  of  such  flooding  or  water

rushing  and  for  giving  alert  signals  to  the  building  workers  and  other

persons to keep them away from danger.

107

216.  Steel curtains Sec. 40

The employer shall ensure at a construction site of a building or other

construction  work  that  air  tight  steel  curtains  are  provided  in  areas  liable  to

flooding  at  tunnelling  work  and  in  case  of  descending  tunnel  to  ensure  the

retention of pockets of air for rescue purpose.

217.  Rest shelters Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that-

(a)

where building workers employed in a compressed air environment in

a tunnelling work are required to remain at the work site for one hour

or  more  after  de-compression  from  pressure  exceeding  one  bar,

adequate and suitable facilities are provided for such building workers

to rest;

(b)

every  man-lock  medical  -lock  and  any  other  facility  inside  these  locks

at an excavation a tunnelling work is maintained in a clean state and in

good repairs;

(c)

a  first-aid  room  is  provided  and  is  readily  available  at  a  construction

site of a tunnelling work;

(d)

each  man-lock  attendant  station  is  provided  with  a  first-aid  box  at  a

construction site of a tunnelling work.

218.  Permissible limits of exposure of chemicals Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that-

(a)     the  working  environment  in  a  tunnel  or  a  shaft  in  which  building

workers  are  employed  does  not  contain  any  of  the  hazardous

substances  in  concentrations    beyond  the  permissible  limits  as  laid

down in the Schedule-XI annexed to these rules;

(b)

the responsible  person referred to in rule 187 conducts necessary test

before  the  commencement  of  a  tunnelling  work  for  the  day  and  at

suitable  intervals  as  fixed  by  the  Chief  Inspector  to  ensure    that  the

permissible limits of exposure are not exceeded and a record of such

test  is  maintained    and  is  made  available  for  inspection  to  the

Inspector having jurisdiction, on demand.

108

219.  Ventilation

The employer shall ensure at a construction site of a building or other

construction work that all working areas in a free air tunnel are provided with

ventilation  system  as  approved  by  the  Chief  Inspector  and  the  fresh  air

supplied in such tunnel is not less than six cubic, metre per minute for each

building  worker  employed  underground  in  such  tunnel  and  the  free  air-flow

movement inside such tunnel is not less than nine metres per minute.

220.  Air supply intake point Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that the air intake points for all air compressors are located

at places where such intake air does not get contaminated with dust, fumes,

vapor and exhaust gases or other contaminants.

221.  Emergency generator Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that-

(a)

every  compressed air system in a tunnel is provided with emergency

power supply system for maintaining continued supply of  compressed

air  in  such  compressed  air  system  and  is  capable  of  operating    air

compressor and ancillary systems of such compressed air system ; and

(b)     the  emergency    power  supply  system  is  maintained  and  is  readily

available at all times at an excavation or tunnelling work.

222.  Air mains Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that every air main supplying air to the working chamber,

man  lock  or  medical-lock  used  at  an  excavation  or  tunnelling  work  is

protected  against  accidental  damage  and  where  it  is  not  practicable  to

provide such protection, a stand-by  air main is provided.

223.  Bulk head and air locks Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that-

109

(a)

a  bulk  head  or  air  tight  diaphragms  retaining  compressed  air,  when

used  within  a  tunnel  or  a  shaft,  is  constructed  to  withstand  the

maximum  pressure  at  one  point  to  five  times  the  maximum  working

pressure of such bulk head or diaphragm is tested before its each use

by  a  responsible    person  referred  to  in  rule  187  to  ensure  that  such

bulk head or diaphragm is in proper working order;

(b)

such  responsible  person  keeps  the  record  of  each  test  referred  to  in

clause (a) and such record is produced for inspection to the Inspector

having jurisdiction on demand;

the  bulk  head  or  diaphragm  referred  to  in  clause(a)  are  made  of

sound  material  of  adequate  strength  and  are  able  to  withstand  the

maximum pressure on which they are subjected to at any time  of their

use;

(d)      bulk-head  anchorage  and  air  lock  is  tested  at  its  work  place  at  an

excavation  or  tunnelling  work  immediately  after  their  installation  at

such place.

224.  Diaphragms Sec. 40

The employer shall ensure at a construction site of a building or other

construction  work  that  all  diaphragms  which  are  in  the  form  of  horizontal

decks  across  a  shaft  used  at  excavation  or  tunnelling  work  are  securely

anchored.

225.  Potable electrical hand tools Sec. 40

The  employer  shall  ensure  at  a  construction  site  of  building  or  other

construction work that all portable electrical hand tools and inspection lamps

used underground or in a confined space at an excavation or tunnelling work

are operated at a voltage not exceeding twenty tour volts.

226.  Circuit breakers Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that-

(a)     adequate  numbers  of  differential  ground  fault  circuit    breakers  are

installed    for  every  electrical  distribution  system  and  its  sub-systems

used at an excavation  or tunnelling work and the sensitivity  of each

110

of circuit breakers is adjusted in accordance with the requirement set

out in accordance with the relevant  national standards;

(b)    no  semi-enclosed  fuse  unit  is  used  in  underground  place  at  an

excavation  or tunnelling work.

227.  Transformer Sec.

The employers shall ensure at a construction site of a building or other

construction work that no transformer is used in any section of a tunnel under

compressed  air  unless  such  transformer  is  of  the  dry  type  and  conforms  to

the relevant national standard.

228.  Live wires Sec. 40

The employers shall ensure at a construction site at a building or other

construction  work  that  there  is  no  exposed  live  wire  in  working  areas  at  an

excavation  or tunnelling work which are accessible to building workers other

than those authorised to work on such live lines.

229.  Welding sets Sec. 40

The employer shall ensure at a construction site at of building or other

construction  work  that  all  welding  sets  used  in  a  tunnel  are  of  adequate

capacity and of suitable type approved by Chief Inspector.

230.  Quality and quantity of air Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that,-

(a)     every  working  chamber  at  an  excavation  or  tunnelling  work  where

compressed air is used, the supply  of such air is maintained not less

than  zero    point  three  cubic  meters  per  minute  per  person  working

therein;

(b)

a  reserve  supply  of  compressed  air  is  made  available  at  all  time  for

man-locks and medical locks used at a tunnelling work;

(c)

the air supplied in a compressed air environment at a tunnelling work

is  as  far  as  practicable  free  from  odour  and  other  contaminants,

namely, dust, fumes and other toxic substances.

111

231.  Working temperature Sec. 40

The employer shall ensure at a construction  site of a building or other

construction  work  that  the  temperature  in  any    working  chamber  at  an

excavation or tunnelling work where building  workers are employed does not

exceed twenty nine degree celsius and that the arrangement is maintained for

keeping records in  which the temperatures, measured  by dry bulb and wet

bulb  inside  such  working  chamber  once  in  every  hour  and  to  produce  such

records for inspection on demand to the inspector having jurisdiction.

232.  Man-lock and working in compressed air environment Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that-

(a)

man-locks used at a tunnelling work are of adequate strength, made of

sound material and designed to withstand any air  pressure, internal or

external,  to  which  it  may  be  subject  to  in  the  normal  use  or  in  an

emergency;

(b)

(i)

doors of man locks at an excavation or tunnelling work are

made of steel;

(ii)

men-locks used at a tunnelling work are airtight and devices are

provided for sealing   the doors when such locks are under

pressure;

(iii)    the  anchorage  of  man-lock  used  at  tunnelling  work  have

adequate strength to withstand the pressure exerted  by air on

the man-locks;

(iv)    there  is  adequate  room  available  for  the  building  worker  for

working in the man-lock  used at tunnelling work;

(v)

where work is carried out  in any compressed air tunnel, a man-

lock in accordance with the relevant national standards is used

for such tunnel.

(c)

(i)

where a man-lock is used at tunnelling  work, safety instructions

in  Hindi  and  in  local    language  understood  by  majority  of

112

building workers employed therein are displaced at conspicuous

place at such tunnelling work;

(ii)    except  in  an  emergency,  compression  and  decompression

operations  are  carried  out  in  a  man-lock  used  at  tunnelling

work;

(iii)

in  an  emergency  any  material  lock  may  be  used  at  tunnelling

work for compression and decompression of building worker and

a  record  is  kept  in  writing  and  produced  for  inspection  on

demand to the Inspector having jurisdiction;

(iv)

material lock is used with the permission of Chief Inspector  for

compression and decompression of building workers, where it is

impracticable to install both the man-lock and the material lock

at a tunnelling work;

(v)

de-compression of all building worker to atmospheric  condition

at   tunnelling   work   is   carried   out   in   accordance   with   a

decompression in procedure approved by the Chief Inspector:

(vi)     the  man-lock  at  tunnelling  work  is  not  used  for  any  purpose

other than compression or de-compression of building workers;

(vii)    no  de-canting  of  building  workers  at  tunnelling  work  is  carried

out  without  prior  approval  of  Chief  Inspector  except  in  an

emergency;

(viii)   in case a building worker collapses or is taken ill during his de-

compression  in  a  man-lock  used  at  tunnelling  work,  the  lock

attendant of such man-lock raises the pressure in such man lock

until such pressure is equal to the maximum pressure which that

building worker was exposed to  in the working chamber prior to

such  de-compression  and  such  lock  attendant  immediately

reports the matter relating to such collapse to the medical lock

attendant and medical officer on duty at such tunnelling work;

(ix)

a  building  worker  who  had  previously  received  training  with  a

trained building worker to work in a compressed air environment

at tunnelling work is employed to work independently in such a

compressed air environment;

113

(x)

a  building  worker  who  had  undergone  three  de-compressions

from a pressure exceeding one bar in a period of eight hours at

tunnelling  work  is  not  allowed  to  enter  a  compressed  air

environment  except  for  the  purpose  of  carrying  out  rescue

work;

(xi)

a building worker employed in a compressed air environment for

a  period  of  eight  hours  in  a  day  at  tunnelling  work  is  not

employed  again  in  such  environment  unless  he  has  spent  not

less  than  twelve  consecutive  hours  of  rest  at  atmospheric

pressure.;

(xii)   no   building   worker   is   engaged   in   a   compressed      air

environment   at   a   pressure   which   exceeds   three   bars   at

tunnelling  work  unless  prior  permission,  in  writing,  has  been

obtained from the Chief Inspector  for such engagement;

(xiii)    no   building   worker   is   employed   in   a   compressed   air

environment  for  more  than  fourteen  consecutive  days  in  a

month at tunnelling work;

(xiv)   a  register  of  employment  of  all  building  workers  employed  in

compressed air environment at tunnelling work, is maintained;

(xv)    an   identification   badge   is   supplied   to   a   building   worker

employed in compressed air environment at tunnelling work;

(xvi)   the  badge  of  a  building  worker  referred  to  in  sub-clause    (xv)

contains  particulars  of  his  name,  location  of  the  Medical  lock

allotted   to   him   for   work   the   telephone   number   of   the

Construction  Medical  Officer  concerned  for  his  treatment  and

the  instructions  in  case  of  his  illness  of  unknown  and  doubtful

causes;

(xvii)     record  of  all  identification  badges  supplied  to  building  workers

under sub-clause(xvi) is kept in a register;

(xviii)  every    building  worker  whose  name  appears  in  the  register

referred to in sub-clause(xvii) wears the badge supplied to him

under  sub-clause(xv),  at  all  time  during  his  duty  hours  at

tunnelling work;

(xix)   suitable  warning  signs  are  displayed,  in  the  compressed  air

environment at tunnelling work, for the prohibition of following,

namely:-

114

(a)

use of alcoholic drinks;

(b)

use  and  carrying  of  lighters,  matches  or  other  sources

of ignition;

(c)      smoking;

(d)

an entry  to person who has consumed alcoholic drinks.

233.  Safety instruction Sec. 40

The employer shall ensure at a construction site of a building or other

construction  work  that  all  building  workers  employed  in  compressed  air

environment at tunnelling work follow the instructions issued for their safety

in the course of such employment.

234.  Medical lock Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that,-

(a)

a  suitably  constructed  medical  lock  is  maintained  at  tunnelling  work

where  building  workers  are  employed  in    a  working  chamber  at  a

pressure exceeding one bar;

(b)

where  more  than  one  hundred  building  workers  are  employed  in  a

compressed  air  working  environment  exceeding  one  bar  at  tunnelling

work,  one  medical  lock  is  provided  for  every  one  hundred  building

workers  or  part  thereof  and  such  medical  lock  is  situated  as  near  as

possible to the  main lock used at such tunnelling work.

CHAPTER-VII

(Part-X)

Construction, Repair And Maintenance Of Steep Roof

235.  Work on steep roof Sec. 40

The employer shall ensure at a construction site of a building or other

construction  work  that  all  practicable  measures  are  provided  to  protect  the

building workers against sliding when carrying out work on steep roofs.

236.  Construction and installation of roofing brackets Sec. 40

115

The employer shall ensure at a construction site of a building or other

construction work that-

(a)

roofing brackets are constructed to fit the pitch of steep roof and such

brackets are used to provide level working platform;

(b)

a  roofing  bracket  referred  to  in  clause  (a)  is  secured  in  its  place  by

nailing  pointed  metal  projections  attached  to  the  underside    of  such

bracket  and  securely  driven  in  to  a  steep  roof  on  which  it  is  used  or

secured by a rope passed over the ridge pole and tie of such roof.

237.  Crawling boards Sec. 40

The employer shall ensure at a construction site of a building or other

construction work –

(a)

all  crawling    boards  used  for  work  on  steep  roofs  are  of  adequate

strength,  made  of  sound  material  and  of  the  type  approval  for  the

purpose of their use as per relevant national standards;

(b)

crawling  board  referred  to  in  clause  (a)  are  kept  in  good  repairs  and

inspected by a responsible person before being taken into use;

(c)

crawling  board  referred  to  in  clause(a)  is  secured  to  a  steel  proof  on

which it is used by ridge hooks or other effective means;

(d)

a  firmly  fastened  life  line  of  adequate  strength  is  strung  beside  each

crawling    board  referred  to  in  clause(a)  throughout  its  length  while

using such crawling boards.

CHAPTER –VII

(Part-XI)

Ladders And Step-Ladders

238.  Construction and safe use Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that-

a)

every ladder or step-ladder used in building or other construction work

is  of  good  construction,  made  of  sound  material  and  of  adequate

strength for the purpose for which such ladder or step-ladder is used;

116

(b)

when  a  ladder  is  used  as  a  means  of  communication,  such  ladder  is

lashed to a fixed structure so that while working on such ladder it does

not slip;

(c)

a  ladder  or  step-ladder  does  not  stand  on  loose  bricks  or  other  loose

packing and has a level and firm footing;

(d)

where  it  is  required,  in  case  of  used  of  fixed  ladders,  sufficient  foot-

hold and hand-hold are provided for use by the building worker;

(e)     every

(i)

secured so as to prevent undue swaying;

(ii)

equally and properly supported on each of its upright ;

(iii)

so used as not to cause undue sagging; and

(iv)

placed as nearly as possible at an inclination of four in one;

(v)

the use of all ladders and step-ladders conform  to the relevant

national standards for their use.

239.  Rungs Sec. 40

The employer shall ensure at a construction site of a building or other

construction  work  that  no  ladder  is  used  which  has  a  missing  or  defective

rung or a rung which depends for its support solely on nails, spikes or other

similar fixing.

240.  Material for ladders Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that all wooden ladders used in building work-

(a)

are constructed upright of adequate strength and are made of straight-

grained  wood,  free  from  defects  and  having  the  grain  of  such  wood

running lengthwise;

(b)     have  rungs  made  of  straight-grained  wood  free  from  defects  and

mortised or securely  notched into the upright; and

(c)     have  reinforcing  metal  ties,  if  the  tenons  of  such  ladders  are  not

secured by wedge.

CHAPTER-VII

(Part-XII)

Catch Platform And Hoardings, Chutes, Safety Belts And Nets

117

241.  Catch platform Sec. 40

The  employer  shall  ensure  at  a  construction  site  of  building  or  other

construction work that-

(a)

catch  platform    is  not    used  for  storage  of  material  or  as  a  working

platform;

(b)

catch platform is at least two meters wide and is inclined so that the

position of outer edge of such platform is fifteen hundred  millimeters

higher than the inner edge;

(c)

the open end of catch platform is properly fenced to the height not less

than one meter.

242.  Hoarding Sec. 40

The  employer  shall  ensure  at  a  construction  site  of  building  or  other

construction    work  that  hoardings  are  constructed  when  the  Chief  Inspector

consider  it  necessary  for  protection    of  building    workers  and  directs  such

employer to construct such hoardings.

243.  Chutes, its construction and use Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that;

(a)

wooden or metal chutes which are at an angle of more than forty five

degrees  to  the  horizontal  and  used  for  the  removal  of  materials  are

closed  on  all  sides  except  at  their  openings  used  for  receiving  or

discharging of materials or articles;

(b)

all  openings of chutes except their top openings are closed when not

in use;

(c)     every

(i)      is  constructed  of  sound  material,  adequate  strength  and  is

suitable for the purpose it is intended for use;

(ii)

exceeding twelve meters in height is constructed in accordance

with the design and drawings of a professional engineer for such

construction  and approval of the Chief Inspector;

(d)

a suitable warning notice is displayed at conspicuous location, written

in Hindi and in a local language, at the discharge  end of every chute;

118

(e)

every chute is cleared when debris has accumulated to a height which

can  pose  danger  to  building  worker  but  such  clearance  is  done  in  no

case less frequently than once a day.

244.  Safety belt and its use Sec. 40

The  employer  shall  ensure  at  a  construction  site  of  building  or  other

construction work that-

(a)

safety belt, life lines and devices for the attachment of such life lines

conform to the relevant national standards;

(b)

every  building  worker  is  supplied  with  safety  belt  and  safety  life  lines

for  his  protection  and  such  building  worker  uses  such  belts  and  life

lines during the performance of his work;

(c)

all  building  workers  using  safety  belts  and  safety  life  lines  have  the

knowledge of safe use and maintenance of such belts and life lines and

are supplied with necessary instructions;

(d)     the  responsible  person  for  supervising  the  use  of  safety  belts  and

safety life lines referred to in clause (b) inspects and ensure that such

safety belts and life lines are fit for use before taken into use at every

time.

245.  Safety net and its use Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that-

(a)

every safety net is of adequate strength, made of sound material and

is  suitable for use and conforms to the relevant national standards;

(b)

the  responsible  person  for  maintenance  of  safety  nets  and  their  use

ensures safe fixing of such safety nets  and provides such safety nets

with suitable and sufficient anchorage so that the purposes for which

such safety net is intended for use, is served.

246.  Storage of safety belts nets etc Sec. 40

The  employer shall ensure at a construction site of a building or other

construction  work  that  proper  arrangement  is  made  for  the  safe  storage  of

safety belts, safety life lines and safety nets when they are not in use and are

protected against mechanical damage, damages from chemicals and damages

from biological agents.

119

CHAPTER-VII

(Part-XIII)

Structural Frame And Form Work

247.  General provision Sec 40

The employer shall ensure at a construction site of a building or other

construction work that-

(a)

the  trained  building  worker  under  the  direct  supervision  of  a  person,

responsible  for  structural  frame  and    form-work,  are  employed  for

erection of such structural frame or form work, dismantling of building

and    structure  and  performance  of  an  engineering  work  form  work,

false work and shoring work;

(b)

adequate  measures  are  taken  to  guard  against  hazards  arising  from

any temporary state of weakness or unsuitability of a structure.

248.  Form work, false work and shoring Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that-

(a)

form-work and false work are so designed, constructed and maintained

that  such  form  work  and  false  work  support  the  load  that  may  be

imposed on them;

(b)

Such form -work is so erected that working platform, means of access

bracing,  means  of  handling  and  stabilizing  could  easily  be  fixed  with

such form work.

249.  Erection or dismantling steel and prefabricated structure Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that-

(a)

the safety of building workers employed for the erection or dismantling

of steel structures and prefabricated structures is ensured from danger

by using appropriate means such as the following, namely-

(i)

ladders, gangways or fixed platforms;

(ii)

platforms buckets, boatswain’s chair or other appropriate means

suspended from lifting appliances;

120

(iii)

safety harness, life lines, catch net or catch platform;

(iv)

power-operated  mobile working platforms;

(b)

the work of erection or dismantling of buildings of structures or form-

work  or  false  work  or  shoring  or  any  other  civil  engineering  work  is

carried  out  by  trained  building  workers  under  the  supervision  of  a

person responsible  for such work;

(c)

Steel or prefabricated structures are so designed and made that such

structures  can  be  safely    transported  or  erected;  and  weight  of  each

unit of such structures is clearly marked on such unit;

(d)

the  design  of  each  such  part  maintains  stability  of  each  part  of  the

structures  referred  to  in  clause  (a),  clause(b)  and  clause(c),  when

erected,  and  to  prevent  danger,  the  design  shall  explicitly  take  into

account-

(i)      the  relevant  conditions  and  methods  of  attachment  in  the

operations   of   stripping,   transport,   storing   and   temporary

support during erection of such parts; and

(ii)

safeguards, such as provision of railings with working platforms

and  for  mounting  such  railings  and  platforms  easily  on  the

structural steel or prefabricated parts;

(e)

the hooks and other devices built in or provided on the structural steel

or  prefabricated  parts  that  are  required    for  lifting  and  transporting

such parts are so shaped, dimensioned and positioned to withstand the

stresses to which such hooks or other devices are subjected;

(f)     prefabricated  parts  made  of  concrete  are  not  stripped  or  erected

before  such  concrete  has  set  and  hardened  sufficiently  to  the  extent

provided  for  in  the  plans,  and  such  parts  are  examined  by  the

responsible person for any sign of damage before their use;

(g)

Storeplaces are so constructed that-

(i)

there is no risk of structural steel or prefabricated parts  falling

or overturning;

(ii)

storage conditions generally ensure stability  and avoid damage

having  regard  to  the  method  of  storage  and  atmospheric

conditions; and

(iii)

racks are set on firm ground and designed so that units cannot

move accidentally in such store places;

121

(h)

structural  steel  or  prefabricated  parts  are  not  subjected  to  stresses

prejudicial  to  their  stability  while  they  are  stored  or  transported  or

raised or set down ;

(i)

tongs,  clamps  and  other  appliances  for  lifting  structural  steel  and

prefabricated parts are-

(a)     in  such  shape  and  dimensions  as  to  ensure  a  secure  grip

without damaging such parts; and

(b)    marked  with  the  maximum  permissible  load  in  the  most

unfavorable lifting conditions;

(j)

Structural  steel  or  prefabricated  parts  are  lifted  by  such  methods  and

appliances that prevent them from spinning accidentally;

(k)

structural  steel  or  prefabricated  parts  are  provided  with  railing  and

working  platforms  before  raising  such  parts  to  prevent  any  danger  of

falling of building workers, materials or articles at the time of any work

with such parts;

(l)

all reasonably practical measures are taken to avoid injury to building

workers,  building  structure  or  equipment  while  structural  steel  or

prefabricated  parts  are  handled  or  stores  at  transported  or  raised  or

lowered;

(m)

structures  are  not  worked  on  during  violent  storms  or  high  winds  or

any other such hazardous situation;

(n)      the  risk    of  falling  to  which  building  workers,  moving  on  high  or

slopping girders, may be exposed is limited by all means of adequate

collective  protection  or  by  the  use  of  a  safety  harness  which  is  well

secured to a sufficiently strong support;

(o)

structural steel parts which are to be erected at a great height are, as

far as practicable, assembled on the ground;

(p)       when  structural  steel  or  pre-fabricated  parts  are  being  erected,  a

sufficiently extended area underneath the workplace shall be barricade

or guarded;

(q)

steel trusses which are being erected are adequately shored, braced or

guyed until they are permanently secured in position;

(r)

Structural members are not forced in to place by the hoisting machine

while  any  building  worker  is  in  such  a  position  that  he  is  likely  to  be

injured  by such operation.

122

250.  Form Work Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that-

(a)     all  form  work  are  properly  designed  keeping  in  view  the  safety  of

building workers, building or structures;

(b)

a responsible person for structural frame and form work-

(i)

inspects and examines the material, timber structural steel and

scaffolding for its strength and suitability before being taken into

use;

cover all stages of such structural

frame and form work;

(iii)

supervises such structural frame and form work;

(iv)     take  all  necessary  steps  or  measures  to  correct  any  situation

with a view to prevent accident or dangerous occurrence during

performances of such structural frame and form work.

251.  Deshoring Sec. 40

The employer shall ensure at a construction site of a building or other

construction work -

(a)

when  shoring  is  removed,  sufficient  props  are  left  in  place  of  such

shoring to prevent any possible hazard; and

(b)     deshoring  is  adequately  braced  or  tied  together  with  support  to

prevent any hazard.

CHAPTER-VII

(Part-XIV)

STACKING AND UNSTACKING

252.  Stacking and unshackling of material and articles Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that,

(a)     where  stacking  un-stacking,  stowing  or  un-stowing  of  construction

material or article, or handling in connection therewith cannot be safely

carried   out unaided, reasonable measures to guard against accident

or dangerous occurrences are taken by shoring or otherwise to prevent

any danger likely to be caused by such handling;

123

(b)

stacking of material or article is made on firm foundation not liable to

settle and deviate such material or article  and does not overload the

floor on which such stacking is made;

(c)

the material or articles, are not stacked against partition or walls of a

warehouse or store place unless it is known that such partition or the

wall is of sufficient strength to withstand the pressure of such materials

or articles;

(d)

the materials or articles are not stacked to such a height and in such a

manner  as  would  render  the  pile  of  such  stack  unstable  and  cause

hazards to the building  workers or the public in general;

(e)

where the building workers are  working on stack exceeding one point

five meters in height, safe means of access to the stack is provided.;

(f)     all  stacking  or  un-stacking  operations  are  performed  under  the

supervision of a responsible person for such stacking or un-stacking;

(g)

the stacking of construction materials or articles is not made near the

site of excavation shaft, pit or any other such opening; and

(h)

stacks    which  may  lean  heavily  or  become  unstable  or  collapse  are

barricaded.

253.  Stacking of cement and other material bags Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that,-

(a)

a stack pile is not  more than ten bags in height unless such stack pile

is stacked in a suitable enclosure or otherwise  adequately supported ;

(b)

while removing bags from the stack pile, the stability of such stack pile

is ensured;

(c)

bags containing cement or time are stored in dry places;

(d)

the materials like bricks, tiles or blocks are stored on a firm ground;

(e)

reinforcing steel is stored according to its shape, size and length;

(f)

stack of reinforcing steel is kept as low as possible;

(g)

no pipe is stored on rack or in stack where such pipe is likely to fall by

rolling;

(h)

the angle of repose is maintained where loose materials are stacked;

(i)

when  dust  laden  material  is  to  be  stored  or  handled,  measures  are

taken to suppress the dust produced by  such   storing or handling and

124

suitable personal protective equipment are supplied to and used by the

building workers working for such storing or handling.

CHAPTER-VII

(Part-XV)

Scaffold

254.  Scaffold

The  employer  shall  ensure  at  a  construction  site  of  a  building  other

construction work that-

(a)      every   scaffold   and   every   component   thereof   is   of   adequate

construction,  made  of  sound    material  and  free  from  defects  and  is

safe for the purposes for which it is intended for use;

(b)

in  case  bamboo  used  for  scaffolding,  such  bamboo  is  of  suitable

quality, good condition, free from protruding knots and stripped off to

avoid any injury to building workers during handling such bamboo;

(c)

all metal scaffolds used in building or other construction work conform

to the relevant national standards.

255.  Supervision by a responsible person Sec. 40

The employer shall ensure at a construction site of a building or other

construction  work  that  no  scaffold  is  erected,  added,  altered  or  dismantled

except  under  the  supervision  of  a  responsible  person  for  such  erection,

addition, alteration of dismantling.

256.  Maintenance Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that-

(a)

the scaffold used in building or other construction work is maintained

in  good  repairs  and  the  measures  are  taken  against  its  accidental

displacement or any other hazard;

(b)

no scaffold or part thereof is partly dismantled and allowed to remain

in such a condition unless:-

(i)     the  stability  or  safety    of  the  remaining  portion  of  such

scaffolding  has  been  ensured  by  a  responsible  person  for  the

safety of such scaffolds;

125

(ii)

in case the remaining part of such  scaffold can not be used by

the building  workers, necessary warning notice written in Hindi

and  in  a  language  understood  by  the  majority  of  the  building

workers  that  such  scaffold  is  unfit  for  use,  is  displayed    at  the

place where such scaffold is erected.

257.  Standards, ledger, putlogs Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that.-

(a)

standard of a scaffold are-

(i)

plumb, where practicable;

(ii)

fixed  sufficiently  close  together  to  secure  the  stability  of  such

scaffold having regard to all the possible working situations and

conditions for the intended use of such-scaffold;

(iii)

spaced, as close as practicable, to ensure safety and stability of

such scaffold;

(b)

adequate measures are taken to prevent displacement of a standard of

a scaffold either by providing sole plate or a base plate, as necessary;

c)

ledgers  of  metal  scaffold  are  placed  at  vertical  intervals  with  due

regard to safety and stability of such scaffold;

d)

bamboo  ledgers  are  kept  as  nearly  as  possible  and  are  placed  and

fastened to the standards of a scaffold with due regard to the stability

of such scaffold.

258.  Working platform Sec.

The employer shall ensure at a construction site of a building or other

construction work that-

(a)

working  platform  is  provided  around  the  face  or  edge  of  a  building

adjoining at every upper most permanent floor of such building under

construction  and at any level where construction work of such building

is carried out;

(b)

a  platform  is  designed  to  suit  the  number  of  building  workers  to  be

employed  on  each  bay  of  a  scaffold  work  on  such  platform  and  the

materials or articles and tools to be carried with them in such bay;

(c)     the  safe  working  load  and  the  number  of  building  workers  to  be

employed in each bay of a scaffold  are displayed for the information of

all the building workers employed at such construction site.

126

259.  Board plank and decking Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that-

(a)     board,  plank  and  decking  used  in  the  construction  of  a  working

platform  is  of  uniform  size  and  strength  and  is  capable  of  supporting

the  load  and  number  of  building  workers  in  accordance  with  the

relevant national standards keeping in view  the safety of such building

workers;

(b)

metal decking, which forms part of a working platform, is provided with

non-skid surface;

(c)     no  board  or  plank  which  forms  the  working  platform  is  projected

beyond its end support unless it is effectively prevented from tripping

or lifting;

(d)

board, plank or decking is fastened and secured;

(e)

at  any  one  time,  not  more  than  two  working  platforms  per  bay,  are

used to support building workers or materials or articles at such bay;

(f)

adequate measures are taken to prevent injury which may be caused

by  falling  material  and  objects  by  using  safety  nets  or  other  suitable

means;

(g)

concrete,  other  debris  or  materials  are  not  allowed  to  accumulate  at

any platform on a scaffold;

(h)

Where a work is to be done at the end of a wall, working platform at

such  workplace  is  faced  or,  wherever  practicable,  at  least  zero  point

sixty meters beyond the end of such wall.

260.  Repair of damaged scaffold Sec. 40

The employer shall ensure at a construction site or a building or other

construction work that-

(a)

no building  worker is permitted to work  on a scaffold which has been

damaged  or  weakened  unless  adequate  safety  measures  have  been

taken to ensure the safety  of such building worker;

(b)

necessary warning signs are displayed at such places where repairs of

scaffold are undertaken.

261.  Opening

127

The employer shall ensure at a construction site of a building or other

construction work that-

(a)

there is no opening in any working platform except for allowing access

to such working platform;

(b)

wherever  opening  on  a  platform  is  unavoidable,  necessary  measures

for protection against falling of objects or building workers from such

platform are taken by providing suitable safety nets, belts or any other

similar means;

(c)

access  from one working platform to another platform on a scaffold, if

required,  is  provided  with  suitable  and  safe  ladder  for  the  use  of  a

building workers working on such platform.

262.  Guardrails Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that every side of a working platform from which a person

liable  to  fall  is  provided  with  suitable    and  safe  guardrails  and  toe-board  of

adequate  strength  to  prevent  fall  of  any  building  worker,  material  or  tools

from such platform.

263.  Scaffold used by building workers of different employers Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that,-

(a)

where a scaffold or a part of a scaffold is used, which has previously

been used by another employer for his building workers, such scaffold

or  part  thereof  is  used  only  after  its  inspection  and  examination  by  a

responsible person for its use that such scaffold or part is safe and fit

for such use;

(b)     if  any  rectification,  alteration  or  modification  in  a  scaffold  or  part

thereof  is  needed  to  suit  its  use,  such  rectification,  alteration  or

modification  is  made  in  consultation  with  the  responsible  person

referred to in clause (a) before using such scaffold or part.

264.  Protection against electric power line Sec. 40

The employer shall ensure at a construction site of a building or other

construction  work  that  all  necessary    and  practical  measures  for  protection

128

are taken to prevent any building worker, working on a scaffold, from coming

into contact with the electric wires or dangerous equipment.

265.  Screening net and wire net Sec. 40

The employer shall ensure at a construction site of  a building or other

construction  work  that  where  a  scaffold  is  erected  in  an  area  where  the

construction  activities  may  pose  hazards  to  pedestrians  or  vehicular  traffic

nearby  from  the  falling  of  objects,  wire  nets  or  screening  nets  are  used  to

envelope such scaffold.

266.  Tower scaffold Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that-

(a)     the   height   of   every   tower   scaffold   used   in   building   or   other

construction  work  is  not  more  than  eight  times,  the  lesser  of  a  base

dimension of such scaffold;

(b)

a tower scaffold is lashed to a building or a fixed structure before being

used by the building workers;

(c)

any tower scaffold which can be moved or castered  is –

(i)

constructed with due regard to the stability and, if necessary ;

(ii)

used only on plain and even surface; and

(iii)  has casters provided with positive  locking devices to hold   such

scaffold in position;

(d)

no building worker remains on board scaffold, tools material when it is

being shifted from one position to another position.

267.  Gear for suspension of scaffold Sec. 40

The employer shall ensure at a construction site of a building  or other

construction work that-

(a)

chains,  ropes  or  lifting  gears  used  for  suspension  of  a  scaffold  are  of

adequate  strength,  made  of  sound  material  and  suitable  for  the

purpose  of their use and are maintained in good repairs;

(b)     chains,  wires,  ropes  or  metal  tubes  used  for  the  suspension  of  a

scaffold are-

129

(i)

properly and securely fastened to every anchorage point and to

the scaffold ledgers of  other main supporting members used for

the support of such scaffold; and

(ii)

so positioned as to ensure stability of the scaffold.

268.  Trestle scaffold and cantilever Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that-

(a)

no  trestle  scaffold  is  constructed  with  more  than  three  tiers  or  if  its

working  platform  is  more  than  four  point  five  meters  above    the

ground  or  floor  or  other  surface  upon  which  such  scaffold  is  erected,

such trestle scaffold is designed by  professional engineer and has the

approval of Chief Inspector before being taken into use;

(b)

no trestle scaffold is erected on a suspended scaffold;

(c)

no cantilever of jib scaffold is used unless it is adequately  supported,

fixed  and  anchored  on  opposite  side  of  its  support  has  out-riggers  of

adequate  length  and  where  necessary  sufficiently  supported  and

braced to ensure safety and stability of such scaffold;

(d)

no working  platform  resting on bearers let into a wall at one end and

without  other  support  is  used  unless  such  bearers  are  of  adequate

strength, braced through the wall and securely  fastened on the other

side.

269.  Scaffold supported by building Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that-

(a)

no  part  of  a  building  is  used  as  support  or  part  of  a  scaffold  unless

such  part  of  the  building  is  made  of  sufficient  strength  and  made  of

sound material to afford safe support;

(b)

overhanging  eaves  gutters   are  not  used  for  supporting  scaffold ;

(c)

suspended scaffold is made of in accordance with the relevant national

standards before being used by the building workers.

270.  Use of winches and climbers for suspended scaffold Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that-

130

(a)

no  suspended  scaffold  is  raised  or  lowered  by  winches  or  climbers

unless  such scaffold is made of sound material, adequate strength and

has been tested and certified safe for use of winches or climber of such

raising  of  lowering  by  a  competent    person  before  being  taken  into

use;

(b)

all  suspended  scaffolds  counter-balanced  by  counter  weights  are  of

types, approved by the Chief Inspector before being taken into use for

building or other constructed work;

(c)

the  working  platform  of  a  suspended  scaffold  is  securely  fastened  to

the  building  or  structure  as  to  be  safe  and  to  prevent  such  platform

from swing;

(d)     the  safe  working  load  which  a  suspended  scaffold  can  carry,  is

displayed where such scaffold is being used.

271.  Safety devise for suspended scaffold Sec. 40

The employer shall ensure at a construction site of a building or other

construction  work  that  every  suspended  scaffold,  raised  or  lowered  by  the

winches or climbers, is provided at each of its suspension point with a safety

rope  with  automatic  safety  devices  mounted  on  each  of  such  rope  so  that

such safety rope with such automatic safety  device supports the platform of

such  scaffold  in  the  event  of  failure  of  the  primary  suspension  wire  ropes,

winches, climbers  or any part of the mechanism used for raising or lowering

such suspended  scaffold;

Provided that this rule shall not apply-

(a)    where  the  platform  of  such  scaffold  is  supported  at  two

independent suspension wire rope at or near each end of such

platform so that in the vent of failure of one of such suspension

wire  rope,  the  other  wire  rope  is  capable  of  sustaining  the

weights of such platform and its load and prevent it from tilting

; or

(b)

where a system is incorporated which operates automatically to

support the platform of such scaffold and its load in the event of

failure of the primary suspension wire rope of such scaffold.

131

CHAPTER-VII

(Part-XVI)

Cofferdams And Caissons

272.  general provision Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that-

(a)

every cofferdam and caisson is-

(i)

of good construction, sound material and of adequate strength;

(ii)

provided  with  adequate  means  for  building  workers  to  reach

safely at the top of such cofferdam or caisson, as the case may

be, in the event of an in rush of water;

(iii)    provided  with  safe  means  of  access  to  every  place  where

building  workers  are  employed  in  such  cofferdam  and  caisson,

as the case may be;

(b)     the   work   relating   to   construction,   positioning,   modification   or

dismantling  of  cofferdam  or  caissons  is  carried  out  under  the

supervision of a responsible person;

(c)

all cofferdams and caissons are inspected by a responsible person at

intervals as specified by the Chief Inspector ;

(d)

a  building  worker  is  allowed  to  work  in  a  cofferdam  or  caisson  after

such cofferdam or caisson is inspected and found safe by responsible

person  within  such  preceding  period  as  approved  by  the  Chief

Inspector and a record of such inspection is maintained in a register;

(e)

the work is compressed air in a cofferdam or caisson is-

(i)

carried  out  in  accordance  with  the  procedure  laid  down  in  the

relevant   national standards;

(ii)     carried  out  by  such  building  workers  who  have  completed

eighteen  years  of  age  and  are  medically  examined  as  required

under rule 221; and

(iii)

carried out  under the supervision of a responsible person;

(f)

if the work in cofferdam or caisson is carried out in shifts, a record of

the time spent by each building worker in each such shift for carrying

out the work is maintained in a register with particulars of time taken

for the compression of such building worker, if any ;

132

(g)

at every work site or project in  a cofferdam or caisson, where building

workers  are  employed  to  work  in  compressed  air  environment,  a

construction  medical  officer  assisted  by  a  nurse    or  trained  first-aid

attendant, is available at all times at such site or project  during such

work;

(h)

there is one stand by reserve compression to meet the emergency at

each work place or project in a cofferdam or caisson.

273.  Pressure Plant and equipment Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that-

(a)

The pressure plant and equipment-

(i)

are examined and tested by the competent person before being

put into use for such work;

(ii)     is  of  proper  design  and  construction,  sound  material  and

adequate strength to platform the work for which it is used;

(iii)

is properly  maintained in good repairs and working condition;

(b)

the pressure plant and equipment referred to in clause(a) is fitted with-

(i)

a  suitable  safety    valve  or  other  effective  device  to  provide

maximum  safe  discharge  pressure  from  being  exceeded  at  any

time;

(ii)

a  suitable  pressure  gauge  with  a  dial  range  not  less  than  one

point five times and not exceeding twice the maximum working

pressure,  easily  visible  and  designed  to  show  at  all  times,  the

internal pressure in kilogram per square  centimeter and marked

with  the  maximum  safe  working  pressure  at  such  plant  and

equipment;

(iii)

a  suitable  stop  valve  or  valves  by  which  the  pressure  plant  or

the  system  of  the  pressure  plant  may  be  isolated  from  the

source of supply of pressure or otherwise;

(c)

every pressure plant or equipment shall be thoroughly examined by the

competent person-

(i)

externally, once in every period of six months;

ii)

internally, once in every period of twelve months; and

(iii)

by hydraulic test once in a period of four years.

133

CHAPTER-VII

(Part-XVII)

Safety Organisation

274.  safety Committee Sec. 38

(1)

Every  establishment  wherein  five  hundred  or  more  building  workers

are ordinarily employed, there shall be a safety committee constituted

by  the  employer  which  shall  be  represented  by  equal  number  of

representatives  of  employer  and  building  workers  employed  in  such

establishment.  In  no  case  the  number  of  representative  of  the

employer  shall  exceed  the  representatives  of  building  workers.  The

Committee  shall  be  represented  by  representatives  of  the  recognised

unions wherever such unions exits.

(2)

The main functions of the Safety Committee shall be-

(a)

to  identify  probable  causes  of  accident  and  unsafe  practices  in

building  or  other  construction  work  and  to  suggest  remedial

measures;

(b)

to stimulate interest to employer and building workers in safety

by  organizing  safety  weeks,  safety  competition,  talks  and  film

shows  on  safety,  preparing  posters  or  taking  similar  other

measures as and when required or as necessary;

(c)

to  go  round  the  construction  site  with  a  view  to  check  unsafe

practices  and  detect    unsafe  conditions  and  to  recommend

remedial  measures  for  their  rectification  including  First  Aid

Medical and Welfare Facilities;

(d)     to  look  into  the  health  hazards  associated  with  handling

different  types  of  explosives,  chemicals  and  other  construction

material  and  to  suggest  remedial  measures  including  use  of

proper personal protective equipment;

(e)     to  suggest  measures  for  improving  welfare  amenities  in  the

construction  site  and  other  miscellaneous  aspects  of  safety,

health and welfare in building or other construction work;

(f)

to  bring  to  the  notice  of  the  employer  the  hazards  associated

with  use,  handling  and  maintenance  of  the  equipment  used

during the course of building and other construction work.

134

(3)

The safety  committee shall meet at regular intervals at least once in a

month  and  it  shall  be  chaired  by  the  senior  person  having  over  all

control over the affairs of the construction site.

(4)     The  agenda  and  minutes  of  the  meeting  shall  be  circulated  to  all

concerned  and  it  shall  be  in  the  language  understood  by  majority  of

the  building  workers  and  shall  be  produced  to  the  Inspector  on

demand for inspection .

(5)

The    decisions  and  recommendation  of  the  safety  committee  shall  be

complied with by the employer within reasonable time limits.

275.  Safety Officer Sec.

(1) in every establishment wherein five hundred or more building workers are

ordinarily employed, the employer shall also appoint safety officers as

per the scale laid down in Schedule-XII annexed to these rules. Such

safety officers may be assisted by suitable  and adequate staff.

(2)     Duties,  qualifications  and  the  condition  of  service  of  safety  officers

appointed  under  sub-rule  (1)  shall  be  as  provided  in  Schedule-XII

annexed to these rules.

(3)

Wherever number of workers employed by single employer is less than

five  hundred,  such  employers  may  form  a  group  and  appoint  a

common,  such  safety  officer  for  such  group  of  employers  with  prior

permission of Chief Inspector.

276.  Reporting of accidents Sec. 39

(1)

Notice of any accident on the construction site which either-

(a)

causes loss of life; or

(b)

disables  a  building    worker  from  working  for  a  period  of  forty

eight  hours  or  more  immediately  following  the  accident,  shall

forthwith  be  sent  by  telegram,  telephone,  fax  or  similar  other

means including special messenger within four hours in case of

fatal accidents and seventy two hours, in case of other accidents

involving building workers, to-

(i)     the  Assistant  Director,  Industrial  Safety  and  Health

having jurisdiction in the area in which the establishment

in  which    such  accident  or  dangerous  occurrence  took

place is located. The Assistant Director, Industrial Safety

135

and    Health  shall  be  the  authority  appointed  under

section 39 of the Act;

(ii)

Board with which the building worker involved in accident

was registered as a beneficiary;

(iii)

Chief Inspector ; and

(iv)    the  next  in  kin  or  other  relative  of  building  worker

involved in accident.

(2)

Notice  of  any  accident  at  a  construction  site  of  a  building  or  other

construction work which-

(a)

causes loss of life; or

(b)

disables such building worker from work for more than ten days

following the accident, shall also be sent to-

(i)

the officer in charge of the nearest police station;

(ii)     the  District  Magistrate  or  if  the  District  Magistrate  by

order so desires to the Sub-Divisional Magistrate.

(3)

In case of an accident falling under clause (b)of sub-rule (1) or clause

(b) of sub-rule (2), the injured building worker shall be given first-aid

and immediately thereafter be transferred to a hospital or other place

for medical treatment.

(4)

Where any accident causing disablement subsequently results in death

of  a  building  worker,  notice  in  writing  of  such  death  shall  be

communicated to the authorities as mentioned in sub-rule (1) and sub-

rule(2) within seventy two hours of such death.

(5)

The  following  classes  of  dangerous  occurrences  shall  be  reported  to

the  Inspector  having  jurisdiction,  whether  or  not  any  death  or

disablement is caused to a building worker, in the manner prescribed in

sub-rule(1), namely:-

(a)

collapse  or  failure  of  lifting  appliances  or  hoist  or  conveyors  or

other  similar  equipment  for  handling  building  or  construction

material  or  breakage  or    failure  of  rope,  chain  or  loose  gears;

over  turning  of  cranes  used  in  building  or  other  construction

work; falling of objects from height;

(b)

collapse    or  subsidence  of  soil,  any  wall,  floor,  gallery,  roof  or

any other part of any structure, platform staging, scaffolding or

any means of access including form-work;

(c)

contact work, excavation, collapse of transmission;

136

(d)

explosion  of  receiver  or  vessel  used  for  storage,  at  a  pressure

greater than atmospheric pressure, of any gas or gases or any

liquid or solid use as building material;

(e)

fire and explosion causing damage to any place on construction

site where building workers are employed;

(f)

spillage  or  leakage  of  hazardous  substances  and  damage  to

their container;

(g)

collapse, capsizing, toppling  or collision of transport equipment;

(h)

leakage or release of harmful toxic gases at the construction in

site.

(6)

In  case  of  failure  of  a  lifting  appliance,  loose  gear,  hoist  or  building

and  other  construction  work  machinery  and  transport  equipment  at  a

construction  site  of  a  building  or  other  construction  work,  such

appliances, gear, hoist, machinery or equipment  and the site of such

occurrence  shall,  as  far  as  practicable,  be  kept  undisturbed  until

inspected by the inspector having jurisdiction.

(7)

Every notice given under sub-rule (1), sub-rule (2) or sub-rule (4) shall

be  followed  by  a  written  report  to  the  Inspector,  authority,  under

Section  39  of  the  Act,  the  Board  and  the  Chief  Inspector  in  Form-

XLVI under proper acknowledgement.

277.   Procedure   for   enquiry   into   causes   of   accident   or   dangerous

occurrence Sec. 39

(1)

The enquiry under sub-Section (2) or sub-section (3) of section 39 of

the  Act,  as  the  case  may  be,  shall  be  conducted  by  the  authority

referred to in  sub-clause (I) of clause (b) of sub-rule (1) of rule 276,

in the following manner, namely:-

(a)

the  enquiry  shall  be  commenced  as  early  as  it  may  be,  and  in

any case, within fifteen days of the receipt of notice of accident

or dangerous occurrence under rule 276;

(b)

the  enquiry  may  be  conducted  by  the  authority  referred  to  in

sub-clause(i) of clause(b) of sub-rule(1) of rule 276 himself  or

by an enquiry officer appointed by such authority;

(c)

the authority or enquiry officer, as the case may be shall serve

or  cause  to  be  served,  notices  in    writing  informing  the  date,

time and place of such enquiry to all persons entitled to appear

137

in such enquiry and whose names and addresses are  known to

such authority or enquiry officer;

(d)

notwithstanding  the  provision  of  clause  (b),  for  the  purpose  of

notifying other persons who may in any way be concerned or be

interested  in  such  enquiry,  the  authority  or  enquiry  officer,  as

the case may be, may publish notice of such enquiry in one or

more  local  newspapers,  informing  the  date,  time  and  place  of

such enquiry.

(2)

The person entitled to appear at the enquiry may include-

(a)

an  inspector  or  any  officer  of  the  Central  Government  or  the

State Government  or an undertaking or public body, concerned

with the enforcement or compliance of safety provisions of the

Act and these rules in the concerned establishment:

(b)    a  trade  union  or  a  workers  association  or  an  employers’

association;

(c)     the  worker  involved  in  the  accident  or  his  legal  heir  or

authorized representative;

(d)

the owner of the premises in which the accident took place;

(e)     any  other  persons,  at  the  discretion  of  the  authority  or  the

enquiring officer, as the case may be, who may be interested in

or  be  concerned  with  the  cause  of  an  accident  or  may    have

knowledge  about  such  cause  or  is  likely  to  give  material

evidence  or  produce  a  relevant  document  in  connection  with

such accident or dangerous occurrence.

(3)     In  case  the  entitled  person  referred  to  in  sub-rule(2)  is  a  body

corporate, a company, or any other organization, association ,group of

person  such  group  may  be  represented  through  an  authorized

representative including a counsel or a solicitor.

(4)

Subject  to  the  provisions  of  sub-rule  (5)  the  enquiry  shall  be  held  in

public

(5)

In case where-

(a)

the State  Government is of the opinion that the matter of the

enquiry  or  any  part  of  it  are  of  such  nature  that  it  would  be

against  the  interests  of  national  security  to  hold  the  enquiry  in

public and  directs  the said authority or the enquiry officer, as

the case may be, to hold the enquiry in camera; or

138

(b)     on  an  application  made  by  any  party    to  the  enquiry,  the

authority or the enquiry officer, as the case may be , referred to

in  sub-rule(1),  if  it  or  he  is  of  the    opinion  that  the  holding  of

public enquiry will lead to disclosure of information relating to a

trade  secret,  decides    to  hold  the  enquiry  of  such  part  of  it  in

camera, such enquiry shall not be held in public.

(6)

Information  disclosed  by  any  person  during  the  course  of  hearing  or

evidence in the cases covered under sub-rule(5) shall not be disclosed

to any person except for the purpose of the enquiry.

(7)

The person entitled to appear under sub-rule (2) called for evidence or

representing  in  an  enquiry  shall  be  entitled  to  make  an  opening

statement,  give  evidence,  request  the  enquiry    officer  to  call  for

specified document or evidence, cross examine other person or to the

extent  and  at  the  stage  permitted  by  the  authority  or  enquiry  officer

holding the enquiry.

(8)

Any evidence in  an enquiry  may be admitted at the discretion of the

authority  or enquiry  officer during the enquiry, who may, also direct

that  documents  to  be  tendered  in  evidence  may  be  inspected  by  any

person  entitled  or  permitted  to  appear  at  such  enquiry  and  that

facilities be afforded to such person to take or obtain copies thereof.

(9)

The authority or the enquiry officer holding an enquiry may authorize

any  person,  being  an  officer  of  the  State  Government,  to  assist  such

authority  or  enquiry  officer  where  necessary,  for  the    purpose  of

conducting  the  enquiry,  and  the  officer  so  authorized  may  enter  the

premises of the concerned establishment during working hours, inspect

the  records  relevant  to  such  enquiry,  investigate  and  take  such

evidence as may be required to conduct such enquiry.

(10)   The  findings  of  the  enquiry  along-with  all  evidence,  in  original,

including  statements  of  witnesses  shall  be  forwarded  to  the  authority

specified under section 39 of the Act within five days of the completion

of the enquiry in cases where sub-enquiry was not conducted by such

authority itself.

(11)

A copy of the findings along with a brief statement of facts relating to

an  enquiry  conducted  under  this  rule  shall  be  forwarded  to  the  Chief

Inspector  and  the  State  Government  by  the  authority  referred  to  in

sub-rule (1) of rule 276.

139

CHAPTER-VII

(Part-XVIII)

Explosives

278.  Handling of explosives Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that-

(a)     all  explosives  are  handled,  used  or  stored  in  accordance  with  the

instructions and the material data sheet supplied by the manufacturer

of such explosives;

(b)

the  use  of  explosives  is  carried  out  in  safe  manner  to  avoid  injury  to

any person and under the direct supervision of a responsible person;

(c)

before using any explosive, necessary warning and danger signals are

erected,  at  conspicuous  places  of  such  use  to  warn  the  building

workers and the general public of the danger involved in such use.

279.  Precautions Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that-

(a)

notwithstanding  the  provisions  of  rule  278,  the  following  precautions

are observed at the places of transporting, handling, storage and use

of such explosives, namely-

(i)     prohibition  of  smoking,  naked  lights  and  other  sources  of

ignition in the vicinity where explosives are handled, stored and

used;

(ii)     to  keep  safe  distance  and  to  use  non-sparking  tools  while

opening packages containing explosives.

(iii)     to  stop  the  use  of  explosives  and  handling  thereof  while  the

weather conditions are not suitable for such use or handling.

(b)    in  addition  to  the  provisions  of  this  chapter,  all  measures  and

precautions    required  to  be  observed  for  use,  handling,  storing  or

transportation   of   explosives   under   the   rule   framed   under   the

Explosives Act, 1884(4 of 1884), are observed.

140

CHAPTER-VII

(Part-XIX)

Piling

280.  General Provisions Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that-

(a)

all pile driving equipment are of good design and sound construction,

taking  into  account    the  ergonomic  principles  and  are  properly

maintained.

(b)

a pile driver is firmly  supported on a heavy timber sill, concrete    bed

or other secured foundation;

(c)

in case a pile driver is required to be erected in dangerous proximity to

an  electrical  conductor  all  necessary  precautions  are  taken  to  ensure

safety.

(d)

the  houses  of  steam  and  air  hammer  are  securely    lashed  to  such

hammer so as to prevent them from whipping in case of connection or

break;

(e)

adequate  precautions  is  taken  to  prevent  the  pile  driver  from  over

turning;

(f)

all necessary precaution is taken to prevent hammer from missing the

pile;

(g)

a  responsible  person  for  inspecting  pile  driving  equipment,  inspects

such  equipment  before  taking  it  into    use  and  takes  all  appropriate

measures  as  required  for  the  safety    of  building  workers  before

commencing  piling work by such equipment.

281.  Stability of adjacent structure Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that where there is any question of stability of a structure

for  its  adjoining  areas  to  be  piled,  such  structure  is  supported,  where

necessary, by underpinning, sheet pilling, shoring, bracing or by other means

to ensure safety and stability of such structure and to prevent injury to any

person.

141

282.  Protection of operator Sec. 40

The employer shall ensure at a construction site of a building or other

construction  work  that  an  operator  of  every  pile  driving  equipment  is

protected  from  falling  objects,  steam,  cinders  or  water  by  sub-stantially

covering or otherwise or by other means.

283.  Instruction to and supervision of building workers working on a pile

driving equipment Sec. 40

The employer  shall ensure at a construction site of a building or other

construction  work  that  every  building  worker  working  on  a  pile  driving

equipment is given instructions regarding safe work procedure to be followed

in piling operation and is supervised by a responsible person throughout such

work.

284.  Entry of unauthorized person Sec. 40

The employer  shall ensure at a construction site of a building or other

construction  work  that  all  piling  areas  where  pile  driving  equipment  is  used

are effectively cordoned off to prevent entry of unauthorized persons.

285.  Inspection and Maintenance of pile driving equipment Sec. 40

The employer shall  ensure at a construction site of a building or other

construction work that-

(a)

pile -driving equipment is not taken into use until it has been inspected

by a responsible person and found to be safe for such use;

(b)

pile-driving equipment in use is inspected by a responsible person for

such    inspection  at  suitable  intervals  to  ensure  safety  to  the  building

worker working on such equipment;

(c)

all pile lines and pulley blocks  are inspected by a responsible person

before the beginning of each shift of piling operations.

286.  Operator pile driving equipment Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that-

(a)

only  experienced  and  trained  building  worker  operates  pile-driving  so

as to avoid any probable  danger from such operation.

142

(b)    pile-driving  operations  are  governed  by  generally  prevalent  and

accepted  signals  so  as  to  prevent  any  probable  danger  from  such

operations;

(c)

every  building  worker  employed  in  pile  driving    operation  or  in  the

vicinity of such pile driving operation wears  car protection  and safety

helmet  or hard hat and safety shoes.

(d)

piles are prepared at a distance , at-least equal to twice the length of

the longest pile, from the place of pile driving operations.

(e)

when  a  pile  driver  is  not  in  use,  the  hammer  of  such  pile  driver  is

blocked at the bottom of the heads of such pile driver.

287.  Working platform on piling frames Sec. 40

The employer shall ensure at a construction site of a building or other

construction  work  that  where  a  structural  tower  supports  the  lead  of  a  pile

driver,  suitable  working  platforms  of  adequate  strength    are  provided  on

levels  of such leads at which it is necessary for the building workers to work

and such platforms are provided with a safety railing and toe boards on each

side or such platforms except on the hammer of such pile driver or lead sides

of  such  platform  and  where  such  platforms  cannot  be  provided  with  such

railing and toe boards, a safety belt is provided to each such building worker.

288.  Pile testing Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that-

(a)

the testing of pile is conducted under the supervision of a responsible

person for such testing;

(b)     all   practicable      measures   like   displaying   of   warning   notices,

barricading, the area and other similar measures are taken to protect

the area where the pile testing  is carried out;

(c)

entry  to  a  pile  testing  area  is  prohibited  to  general  public  to  ensure

safety.

143

CHAPTER VIII

MISCELLANEOUS PROVISIONS

POWERS OF CHIEF INSPECTOR AND INSPECTORS

289.  Power to engage experts, agencies Sec. 42

(1)     The  Chief  Inspector  may  engage  experts  or  agencies,  as  deemed

necessary,  from  the  fields  of  civil  engineering,  structural  engineering,

architecture,  and  other  disciplines  of  occupational  safety,  health  and

environment as and when required for the purpose of conducting any

inspection, investigation or enquiry into the cause of an accident or a

dangerous occurrence or otherwise-

(2)

The experts  referred to in sub-Rule(1) shall-

(a)     possess  a  degree  in  the  relevant  field  from  a  recognised

university;

(b)

possess  not  less  than  ten  years  experience  of  working  in  the

relevant field out of which at-least five years shall be in the field

of occupational safety, health and environment.

(3)

Agencies  referred    to  in  sub-Rule  (1)  shall  be  of  national  standing  in

the relevant field and registered under the relevant law.

(4)

The  State Government may, from time to time prepare a panel of

experts and agencies referred to in sub-Rule(1).

(5)

An engineer or expert or agency  employed under sub-Rule(1) shall be

paid such traveling allowances and daily allowances as are allowed to

him  by  his  organization  where  he  is  employed  or  such  traveling

allowance and daily allowance as is admissible to officer of the rank of

a Class –I Officer of State Government.

(6)

In  addition  to  traveling    allowance  and  daily  allowance  referred  to  in

sub-Rule (5) to an engineer or architect or agency, they shall also be

paid    honorarium  at  the  rates  as  may  be  specified    by  the  State

Government by notification in the Official Gazette from time to time.

290.  Power of Inspectors Sec. 43

(1)     An  Inspector  may,  at  a  construction  site  of  a  building  or  other

construction work within local limits for which  he is appointed-

144

(i)

examine such construction site or place or premises used or to

be used for such building or construction work.

(ii)

take  on  the  spot  or  otherwise    such  evidence  of  any  person

which   he   may   deem   necessary   for   the   purpose   of   any

examination or enquiry connected with such building and other

construction work directly or indirectly;

Provided  that  such  person  shall  not  be  compelled  to

answer any question or give any evidence tending to incriminate

him ;

(iii)    take  photograph,  video  clips,  sample  weight  or  measure  or

record or make such sketches as he may consider necessary for

the purpose of any examination or inquiry under these Rules;

(iv)

hold  an  inquiry  into  the  cause  of  any  accident  or  dangerous

occurrence  which  he  has  reasons  to  believe  was  the  result  of

any  operation  connected  with  or  incidental  to  such  building  or

other  construction  work,  or  of  non-compliance  with  any  of  the

provisions of the Act or these Rules.

(2)

An  Inspector  may,  within  the  local  limits  for  which  he  is  appointed

issue show cause notice or warning to employers regarding  the safety

,  health  or  welfare  of  building  workers  provided  under  the  Act  or  the

Rules.

(3)

An Inspector may, within the local limits for which he is appointed, file

in a court having jurisdiction a compliant or other proceeding relating

to  an offence under the Act.

(4)

An  Inspector  may,  within  the  local  limits  for  which  he  is  appointed,

direct any contractor or any employer for getting the building workers

medically examined in accordance with the provisions of these Rules.

(5)

An  Inspector  may,  within  the  local  limits  for  which  he  is  appointed

require  a  person  having  power  of  supervision  and  control  of  a

construction  site  of  a  building  or  other  construction  work  or  the

employer, project incharge or site in charge of such contraction sites,

as  the  case  may  ,  to  provide  such  means  or  assistance  as  may  be

required  by  such    inspector  for  entry,  inspection,  examination  or

inquiry for the exercise of his powers under sub-section (1) of section

145

43  of  the  Act  or  this  Rule  in  relation  to  such  construction  site,  or

project.

291.  Prohibition

(I)

If  it  appears  to  the  Inspector  that  any  site  or  place    at  which  any

building  or  other  construction  work  is  being  carried  on,  is  in  such

condition  that  it  is  dangerous  to  life,  safety  or  health  of  building

workers  or  the  general    public,  he  may,  in  writing,  serve  on  the

employer of building workers or on the owner  of the establishment or

on the  person in charge of such site or place an order prohibiting any

building or other construction work at such site or place until measures

have been taken to remove  the cause of the danger to his satisfaction.

(2)

An Inspector serving an order under sub-Rule (1) shall endorse a copy

to the Chief Inspector.

(3)      Such  prohibition    order  shall  be  complied  with  by  the  employer

forthwith.

(4)

The Inspector may enforce the prohibition order with the assistance of

any  public  servant  including  police,  if  such  an  order  is  not  complied

with by the establishment or the person in charge at the relevant time

within 24 hours.

(5)

Disobedience of any prohibition order shall be punishable under section

188 of the Indian Penal Code, 1860 (45 of 1860).

(6)

Any  person  aggrieved  by  an  order  under  sub-Rule  (I)  may  ,  within

fifteen days from the date on which the order is communicated to him,

prefer an appeal to the Chief Inspector or where such order is by the

Chief Inspector to the Secretary  Labour of the State Government and

the  Chief  Inspector  or  the  Secretary,  as  the  case  may  be  shall,  after

giving  the  appellant  an  opportunity  of  being  heard,  dispose  of  the

appeal as expeditiously  as possible.

Provided that the Chief Inspector or the Secretary Labour as the

case  may  be,  may  entertain  the  appeal  after  the  expiry  of  the  said

period  of  fifteen  days    if  he  is  satisfied  that  the  appellant  was

prevented by sufficient cause from filing the appeal in time;

146

Provided  further  that  the  prohibition  order  shall  be  complied

with, pending the decision of the Chief Inspector and Secretary Labour

of the State Government.

CHAPTER-IX

(Part-I)

RESPONSIBILITIES AND DUTIES OF EMPLOYERS, ARCHITECTS, PROJECT

ENGINEERS AND DESIGNERS, BUILDING WORKERS ETC.

292.   Duties  and  responsibilities  of  employers,  employee  and  others  Sec.

44

(1)

It  shall  be  the  duty  of  every  employer  who  is  undertaking  any  of  the

operations  or  works  related  to  or  incidental  to  building  or  other

construction work to which these rules apply--

(a)

to comply with such of the requirements of these rules as

are

related to him;

Provided  that  the  requirements  of  this  clause  shall  not

affect any building worker if and so long as his presence in any

place  of  work  is  not  in  the  course  of  performing  any  work  on

behalf  of  his  employer  and  he  is  not  expressly  or  impliedly

authorized or permitted by his employer to do the work; and

(b)

to  comply  with  such  of  the  requirements  of  these  rules  as  are

related to him in relation to any work, act or

operation

performed or about to be performed by  him.

(2)

It shall be the duty of every employer who erects or alters any scaffold

to  comply  with  such  of  the  requirements  of  the  provisions  of  these

rules as relate to the erection or alteration of scaffolds having  regard

to  the  purpose  or  purposes  for  which  the  scaffold  is  designed  at  the

time of erection or alteration; and such employer, who erects, installs,

works or uses any plant or equipment to which any of the provisions of

147

these  rules  apply,  shall  erect,  install,  work  or  use  such  plant  or

equipment in the manner which complies with those provisions.

(3)

Where a contractor, who is undertaking any of the operations or works

to  which  these  rules  apply,  appoints  any  artisan,  tradesman  or  other

person to perform any work or services under a contract for services, it

shall  be  the  duty  of  the  contractor  to  comply  with  such  of  the

requirements of these rules as affect that artisan, tradesman or other

person  and  for  this  purpose  any  reference  in  these  rules  to  an

employee shall include a reference to such artisan tradesman or other

person and the contractor shall be deemed to be his employer.

(4)

It shall be the duty of every employee to comply with the requirements

of  such  of  these  rules  as  are  related  to  the  performance  of  or  the

refraining from an act by him and to co-operate in carrying out these

rules.

(5)

It shall be the duty of every employer not to permit an employee to do

anything  not  in  accordance  with  the  generally  accepted  principles  of

standard  safe  operating  practices  connected  with  building  and  other

construction work as specified by the State Government.

(6)

No  employee  shall  do  anything  which  is  not  in  accordance  with  the

generally  accepted  principles  of  standard  safe  operating  practices

connected  with  building  and  other  construction  work  as  specified  by

the Central Government.

(7)

No person related with any building and other construction work shall

willfully do any act which may cause injury  to himself or to others.

(8)

It shall be the duty of every

employer

not

to

allow

lifting

appliance,  lifting  gear,  lifting  device,  transport  equipment,  vehicles  or

any  other  device  or  equipment  to  be  used  by  the  building  workers

which does not comply with the provisions given in these rules.

(9)     It  shall  be  duty  of  the  employer  to  maintain  the  latrines,  urinals,

washing facilities and canteen in an clean and hygienic condition, The

canteen shall be located in a place away from the latrines and urinals

and polluted atmosphere and at the same time be easily accessible to

the building workers.

(10)    It  shall  be  the  duty  of  the  employer  to  abide  by  the  dates  fixed  and

notified by him for payment of wages for a period in accordance with

these  rules  and  no  change  in  such  dates  and  such  period  shall  be

148

effected without notice to the building workers and the inspector. The

employer  shall  ensure  timely  payment  of  wages  as  specified  under

these  rules  and  at  the  place  and  time  notified  by  him.  Where  the

employer  is  a  contractor,  he  shall  ensure  that  the  wages  of  building

workers are paid in the presence of a representative of the employer of

establishment or owner of premises from whom he has taken the work

on  contract  and  obtain  signatures  of  such  representative  in  token  of

having witnessed the payment of wages.

(11)

It shall be the duty of the employer to ensure that the lifting appliance,

lifting gear, earth moving equipment, transport equipment or vehicles

used  in  the  building  or  other  construction  work  undertaken  by  him

conforms  to  the  requirements  relating  to  testing,  examination  and

inspection of such equipment as provided under these rules. It shall be

the duty of every person in the service of the government or any local

or  other  public  authority  to  comply  with  the  requirements  relating  to

him as given in these rules.

293.   Responsibilities  of  architects,  project  engineers  and  designers  Sec.

40

(1)     It  shall  be  the  duty  of  the  architect,  project  engineer  or  designer

responsible for the design of any project or part thereof or any building

or other construction work to ensure that, at the planning stage, due

consideration is given to the safety and health aspects of the building

workers who are employed in the erection, operation and execution of

such projects and structures, as the case may be.

(2)

Adequate  care  shall  be  taken  by  the  architect,  project  engineer  and

other  professionals  involved  in  the  project,  not  to  include  anything  in

the  design  which  would  involve  the  use  of  dangerous  structures  or

other processes or materials, hazardous to health or safety of building

workers during the course of erection, operation and execution as the

case may be.

(3)

It shall also be the duty of the professionals, involved in designing the

buildings,  structures  or  other  construction  projects,  to  take  into

account  the  safety  aspects  associated  with  the  maintenance  and

upkeep of the structures and buildings where maintenance and upkeep

may involve special hazards,

149

294.  Duties and responsibilities of workers Sec. 40

(1)

It  shall  be  the  duty  of  every  building  worker  to  comply  with  the

requirements of such of these rules as relate to him, and act and co-

operate  in  carrying  out  the  requirements  of  these  rules  and  if  he

discovers any defects in the lifting appliance, lifting gear, lifting device,

concerning  any  transport  equipment  or  other  equipment,  to  report

such defects without unreasonable delay to his employer or foreman or

other person in authority.

(2)

No  building  worker,  shall  unless  duly  authorized  or  except  in  case  of

necessity, remove or interfere with any fencing, gangway, gear, ladder,

hatch   covering,   life   saving   appliances,   lighting   or   other   things

whatsoever required by the Act and these rules to be provided. If any

of aforesaid things is removed, such thing shall be restored at the end

of the period during which its removal was necessary, by the persons

engaged in that work.

(3)     Every  building  worker,  shall  use  only  means  of  access  provided  in

accordance  with  these  rules  and  no  person  shall  authorize  or  order

another to use means of access other than such means of access.

(4)

It  shall  be  the  duty  of  a  building  worker  to  keep  the  latrines,  urinals,

washing points, canteen and other facilities provided by the employer

for securing his welfare in a clean and hygienic condition

295.  Exemption Sec. 40

The  State  Government  may,  by  order  in  writing  and  subject  to  such

conditions and for such period, as may be specified therein, exempt from all

or any of the requirements of these rules to--

(a)

any building or other construction work, if such Government is satisfied

that  such  building  work  is  confined  to  such  workers,  where  it  is  not

convenient to take measures as provided in these rules, or

(b)     any  appliance,  gear,  equipment,  vehicle  or  other  device  if  such

Government is satisfied that the requirement of such appliance, gear,

equipment, vehicle or other device is not necessary for use or equally

effective measures are taken in lieu thereof.

150

Provided that such Government shall not grant exemption under

this rule unless it is satisfied that such exemption would not adversely

affect the safety, health and welfare of building workers.

CHAPTER IX

(Part-II)

Wages

296.  Payment of wages Sec. 45

An employer shall ensure at a construction site of a building or other

construction work that-

(a)

the wages of every building worker employed at such construction site

where less than one thousand such building workers are employed are

paid  before  the  expiry    of  seventh  day  and  in  other  cases  before  the

expiry of tenth day after the last day of the period in respect of which

such wages are payable;

(b)

In case the employment of such building worker is terminated by or on

behalf  of  such  employer,  the  wages  earned  by  such  building  worker

are paid before the expiry of the second working day from the day on

which employment of such building worker is terminated;

(c)

All payment of wages are made on a working day at such construction

site and during the working time and on a date notified in advance and

in  case  the  work  is  completed  the  final  payment  of  wages  is  made

within forty eight hours of such completion of work.

297.  Display of notices of wages regarding date of payment of wages

An employer shall ensure at a construction site of a building or other

construction work that a notice showing the period for which wages are to be

paid,  place  and  time  of  disbursement  of  such  wages  is  displayed  at  a

conspicuous place of such construction site in English, in Hindi and in a local

language  understood  by  the  majority  of  building  workers  employed  at  such

construction site.

CHAPTER-IX

(Part-III)

151

Information To Bureau Of Indian Standards.

298.  Furnishing of information to Bureau of Indian Standards Sec. 40

The employer shall ensure at a construction site of a building or other

construction work that-

(a)     every  architect  and  other  professional  like  structural  engineer  or

project  engineer  involved  in  the  execution  of  a  building    or  other

construction  project,  furnishes to the Bureau of Indian Standards, the

details regarding the performance of and deviations or short-coming, if

any  ,  of  the  building  materials,  articles  or  processes  used  in  such

building and other construction project for which the Indian Standards

are already available;

(b)

the architect and other professional referred to in clause (a) informs to

the  Bureau  of  Indian  Standards,  the  details  of  building  materials,

articles  or  processes  used  in  the  building  and  other  construction

activities for which the Indian Standards do not exit with the Bureau of

Indian  Standards  and  the  performance  of  such  materials,  articles  or

processes  alongwith  the  suggestions  for  their  improvement  to  enable

the  Bureau  of  Indian  Standards  to  consider  and  form  necessary

standards.

152

SCHEDULE-I

(see rule 90)

PERMISSIBLE EXPOSURE IN CASES OF CONTINUOUS NOISE

Total   time   of   exposure   Sound level (in dBA) 
(continuous  or  a  number

of   short-term   exposure) 
per day (in hours)

Pressure

(1)                            (2)

8                             90

6                             92

4                             95

3                             97

2                            100

11/2                           102

1                            105

3/4                           107

1/2                           110

1/4                           115

Notes:

(1)

No. exposure in excess of 115 dBA is to be permitted.

2)

For any period of exposure failing in between any figure and the

next  higher  or  lower  figure  as  indicated  in  column  (1),  the

permissible  sound  pressure  level  is  to  be  determined  by

extrapolation on a proportionate basis.

SCHEDULE-II

(see rule 147 (iv) and 111(a)(iii)]

Periodicity Of Medical Examination of Building Workers.

1.

The employer shall arrange a medical examination of all the building workers

employed as drivers, operators of lifting appliances and transport equipment

before employing, after illness or injury, if it appears that the illness or injury

might have affected his fitness and, thereafter, once in every two years up to

the age of forty and once in a years, thereafter.

153

2.

Complete and confidential records of medical examination shall be maintained

by the employer or the physician authorized by the employer.

3.

The medical examination shall include-

(a)

full medical and occupational history.

(b)

clinical examination with particular reference to-

(i)     General

(ii)

Vision-Total  visual  performance  using  standard  orthorator  like

Titmus  Vision  Tester  should  be  estimated  and  suitability  for

placement  ascertained  in  accordance  with  the  prescribed  job

standards;

(iii)     Hearing-Persons  with  normal  hearing  must  be  able  to  hear  a

forced  whisper    at  twenty  four  feet.  Person  using  hearing  aids

must  be  able  to  hear  a  warning  shout  under  noisy  working

conditions;

(iv)

Breathing-Peak  flow  rate  using  standard  peak  flow  meter  and

the average peak flow rate determined out of these readings of

the    test  performed.  The  results  recorded  at  pre-placement

medical examination could be used as a standard for the same

individual at the same altitude for reference during subsequent

examination;

(v)

Upper limbs-Adequate arm function and grip (both arms);

(vi)

Lower limbs Adequate leg and foot function;

(vii)

Spine-Adequately flexible for the job concerned;

(viii)

General-Mental alertness and stability with good eye, hand and

foot co-ordination

(c)

Any other tests which the examining doctor considers necessary.

SCHEDULE-III

(see rule 113)

Hazardous process:-

1.      Roof

2.      Steel

3.

Work under and over water

4.      Demolition

5.

Work in confined spaces.

154

SCHEDULE-IV

[(see rule 113)(b)]

Services and facilities to be provided in occupational health centers.-

(1)

One  full  time  construction  medical  officer  for  building  or  other  construction

work, employing workers upto one thousand and one additional construction

medical officer for every additional one thousand workers or part thereof.

(2)

The  staff,  including  one  nurse,  one  dresser-cum-compounder,  one  sweeper

cum ward  boy with each  construction medical officer for full work  hours.

(3)

The occupational health center with a floor area of minimum fifteen square

meter   constituting two rooms with smooth walls and inpern service,

adequately illuminated and ventilated.

(4)

Adequate equipment for day to day treatment.

(5)

Necessary equipment to manage any medical emergency.

SCHEDULE-V

(see rule 185 (2) and 113(c)

Qualification of construction medical officer.

(1)

MBBS degree from a medical institute recognized by the Medical  Council of

India;

(2)

Diploma in Industrial health or equivalent post graduate certificate of training

in industrial health or health.

(3)     A  medical  officer  having  working  experience  in  organization/establishment

involved  in  policy,  execution  and  advice  and  safety  and  health  of  workers

employed  in  mines,  ports  and  docks,  factories  and  building  and  other

construction  work, for a period of not less than three years may, subject to

the  satisfaction  of  the    Chief  Inspector,  not  be  required  to  possessing  the

training referred to in item (2) above.

(4)    The  syllabi  of  the  courses  leading  to  the  above  certificates  and  the

organization  conducting  such  courses  shall  be  approved  by  the  State

Government  who    may  also  from  time  to  time  prepare  a  panel  of  such

organizations.

155

(5)     Complete  particulars  including  name,  qualification  and  experience  of  the

construction  medical  officer  will  be  intimated  to  the  inspector  having

jurisdiction.

SCHEDULE VI

(see rule 114)

Article for ambulance room

(i) 
(ii)

(iii) 
(iv)

(v) 
(vi) 
(vii)

(viii) 
(ix)

(x) 
(xi)

(xii) 
(xiii)

(xiv) 
(xv)

(xvi) 
(xvii)

(xviii) 
(xix)

(xx) 
(xxi)

(xxii)

A glazed sink with hot and cold water always available

A table with a smooth top at least 180 cm x105 cm

Means for sterilizing instruments.

A couch.

Two stretchers.

Two buckets  or containers with close fitting lids.

Two rubber hot water bags.

A kettle and spirit stove or other suitable means of boiling water.

Twelve plain wooden splints 900 cm x 100 cm  x 6 cm

Twelve plain wooden splints 350 cm x 75cmx6 cm

Six plain wooden splints 250 cmx50 cmx12 cm

Six woolen blankets.

Three pairs artery forceps.

One bottle of spiritus annemiae aremations (120 ml).

Smelling salt(60 gm)

Two medium size sponges

Six hand towels.

Four kedney trays.

Four cakes of toilet, preferably antiseptic soap.

Two glass tumblers and two wine glasses.

Two clinical thermometers.

Two tea spoons.

(xxiii)

Two graduated (120 ml) measuring glass

(xxiv)

Two minimum measuring glasses.

(xxv)

One wash bottle (1000 cc) for washing eyes.

(xxvi)

One bottle (one liter) carbolic lotion 1 in 20.

(xxvii)        Three

156

(xxviii)

One Screen

(xxix)

One electric hand torch.

(xxx)

Four first-aid boxes or cupboards stocked to the standards prescribed

in the Schedule-VII.

(xxxi)

An adequate supply of tetanus toxide.

(xxxii)

Injections-morphia,  pethidine,  atrophine,  adrenaline,  coramine,  no

vocaine(6 each).

(xxxiii)

Carmine liquid (60 ml)

(xxxiv)        Tablets-antihistaminic

(xxxv)

Syringes with needles -2 cc, 5cc, 10 cc and 500 cc)

(xxxvi)        Three

(xxxvii)

Two needle holders, big and small.

(xxxviii)

Suturing needles and materials.

(xxxix)

Three dissecting forceps.

(xxxx)        Three

(xxxxi)        Three

(xxxxii)

One stethoscope and a B.P.aparatus.

(xxxxiii)

Rubber bandage-pressure bandage.

(xxxxiv)

oxygen cylinder with necessary attachments.

(xxxxv)

Atropine eye ointments.

(xxxxvi)

I.V. Fluids and sets 10 Nos.

(xxxxvii)

Suitable, foot operated, covered, refuse containers.

(xxxxviii)

Adequate number of sterilized, paired, latex hand gloves.

SCHEDULE-VII

(see rule 115)

Contents Of Ambulance Van Or Carriage

THE AMBULANCE VAN SHALL HAVE EQUIPMENTS PRESCRIBED AS UNDER;

(a)    General:  A  portable  stretcher  with  folding  and  adjusting  devices  with  the

Head  of  the  stretcher  capable  of  being  titled  upward.  Fixed  suction  in  unit

with  equipment.  Fixed  oxygen  supply  with  equipment.  Pillow  with  case,

sheets, blankets, towels, emergency bag, bed pan, urinal glass.

(b)    Safety Equipment-Flaros  with  life  of  three  thousand  minutes,  floor  lights,

flash lights, fire extinguishers (dry power type), insulated gantlets.

(c)    Emergency

157

(i)

Resuscitation:  Portable  suctin  unit,  portable  oxygen  unit,  bag  valve

mask,  hand  operated  artificial  ventilation  unit,  airways,  mouth  gag

tracheotomy   adapters,   short   spine   board,   I.V.   FLUIDS   with

administration unit, B.P. manometer cuff stethoscope.

(ii)     Immobilization-Long  and  short  padded  boards,  wire  ladder  splints,

triangular bandage-long and short spine boards.

(iii)

Dressing-Gauaze pads-100mx100mm universal dressing 250x1000 mm

roll  of  aluminum  foils-soft  roller  bandages  150  mm  X  5mm  yards

adhesive tape in 75 mm roll safety pins, bandage sheets, burn sheets.

(iv)

Poisoning-Syrup of Ipecac, activated charcoal prepacketed does, snake

bite kit, drinking water.

(v)     Emergency   Medicines-as   per   requirement(under   the   advice   of

construction Medical Officer.)

SCHEDULE-VIII

(see rule118(a)

Notifiable Occupational Diseases In Building and Other Construction Work

1.      Occupational

2.      Occupational

3.      Asbestosis

4.      Silicosis

5.

Lead poisoning  including poisoning by any preparation or compound of lead

or their squealed.

6.

Benzene Poisoning, including poisoning by any of its homologues, their nitro

or amino derivatives or its squeal.

7.      Occupational

8.      Pesticide

9.      Carbon

10.    Toxic

11.    Toxic

12.

Compressed air illness (Caissons disease)

13.

Noise induced hearing loss

14.

is ocyanates poisoning

158

15.    Toxic

SCHEDULE-IX

[(see Rule 119(b)]

Contents Of A First-Aid Box

(i)

A  sufficient  number  of  eye  wash  bottles  filled  with  distilled  water  or

suitable  liquid  clearly  indicated  by  a  distinctive  sign  which  shall  be

visible at all times.

(ii)

4  percent  xylocaine  eye  drops,  and  basic  acid  eye  drops  and  soda

bycarbonate eye drops.

(iii)

Twenty four small sterilized dressings

(iv)

Twelve medium size sterilized dressings.

(v)

Twelve  large size sterilized dressing.

(vi)

Twelve large size sterilized burn dressings.

(vii)

Twelve (fifteen cm) packets of sterilized cotton wool.

(viii)

(Two  hundred  ml)  bottle  of  certimied  solution  (1  percent)  or  suitable

antiseptic solution.

(ix)

One (two hundred ml) bottle of mercurochrome(2 per cent) solution in

water.

(x)

One (one hundred twenty ml) bottle of salvolatile having the doses and

mode of administration indicated on the label.

(xi)

One pair of scissors.

(xii)

One roll of adhesive plaster (six cm x one meter)

(xiii)

Two rolls of adhesive plaster( two cms x one meter)

(ixv)

Twelve pieces of sterilized eye pads in separate sealed packets.

(xv)

A bottle containing hundred tablets (each of three hundred twenty five

mg) of aspirin or any other analgesic.

(xvi)

Twelve roller bandages ten cms wide.

(xvii)

Twelve roller bandage five cms wide.

(xviii)         One

(xix)

A supply of suitable splints.

(xx)

Three packets of safety pins.

159

(xxi)          Kidney

(xxii)

A snake bite lancet.

(xxiii)

One (thirty ml ) bottle containing potassium permanganate crystals.

(xxiv)

One copy of first-aid leaflet issued by the Chief Inspector.

(xxv)

Six triangular bandages.

(xxvi)

Two pairs of suitable, sterilized, latex hand gloves.

SCHEDULE –X

[(See Rule 122(a),129(a), 136(a), 137(a)(ii))]

Manner of test and examination before taking lifting appliance, lifting gear

and wire rope into use for the first time.

Test Loads:

(1)    Lifting

Every lifting appliance with its accessory gear, shall be subjected to a

test load which shall exceed the safe working load (SWL) as specified in the

following table-

TABLE

Safe Working load

Test load

Upto 20 tonnes

25 percent in excess of safe working load

20 to 50 tonnes

5 tonnes in excess of safe working load

Over 50 tonnes

10 percent in excess of safe working load

(2)    Lifting

(a)    Every  ring,  hook,  chain,  shackle,  swivel,  eye-bolt,  plate  clamp,

triangular  plate  or  pulley  block  (except  single  sheave  block)  shall  be

subjected  to  a  test  load  which  shall  not  be  less  than  the  load  as

specified in the following table:-

TABLE

Safe Working load (in tonnes)

Test load (In tonnes)

Upto 25

2 X safe working load

Above 25

(1.22 x safe working load) + 20

(b)

In  the  case  of  a  single  sheave  block,  the  safe  working  load  shall    be

the  maximum  load  which  can  safely  be  lifted  by  the  block  when

suspended by its head fitting and the load is attached to a rope which

160

passes  around  the  sheave  of  the  block  and  a  test  load  not  less  than

four times the proposed safe working load shall be applied to the head

of the block.

(c)

In the case of a multi sheave block, the test load shall not be less than

the load as specified in the following table:-

TABLE

Safe Working load

Test load ( in tonnes)

Upto 25

2xsafe working load

25 to 160

0.9933xsafe working load) + 27

Over 160

1.1 x safe working load

(d)

In  the  case  of  hand-operated  pulley  blocks  used  with  pitched  chains

and rings, hooks shackles or swivels, permanently attached thereto, a

test load not less than 50 percent in excess of the safe working  load

shall be applied.

(e)

In  the  case  of  a  pulley  block  fitted  with  a  bucket  the  bucket  shall  be

tested and the load applied to the bucket when testing that block will

be accepted as test load of the bucket.

(f)

In the case of a sling having two legs, the safe working load shall be

calculated  when  the  angle  between  the  legs  is  90  degree.  In  case  of

multi-legged  slings  the  safe  working  load  shall  be  calculated  as  per

national standards.

(g)

Every lifting beam, lifting frame, container spreader, bucket, tub or

other similar devices shall be subjected to a test load which shall not

be less than the load as specified in the following  table:-

TABLE

Proposed safe working load (in tonnes)

Test load (In tonnes)

Upto 10

2x safe working load

10 to 160

(1.04 x safe working load)+9.6

Above 160

1.1xsafe working load

(h)    wire

In  the  case  of  wire  ropes  a  sample  shall  be  tested  to

destruction. The test procedure shall be in accordance with recognised

161

national  standards.  The  safe    working  load  of  the  rope  is  to  be

determined  by  dividing  the  load  at  which  the  sample  broke  by  a  co-

efficient of utilization, determined as specified in the following table:-

TABLE

of

(a)  Wire  Rope  Forming  part  of  Sling.

5

Safe  working  load  of  the  Sling:  Safe

working load above  10 tonnes and up

to and equal to 160 tonnes

10 
(8.85xSWL) + 1910

load up to and equal to 10 tonnes

(b)  Wire  Rope  as  Integral  part  of  a

Lifting  Appliance;  SWL  of  the  lifting

10 
(8.85xSWL) + 1910

appliance  ;  Safe  Working    load  upto

load up to and equal to 10 tonnes

and equal to 160 tonnes

(i)      Before  any  test  is  carried  out,  a  visual  inspection  of  the  lifting

appliance,  or  lifting  gear  involved  shall  be  conducted  and  any    visible

defective gear shall be replaced or renewed.

(j)

After  being  tested,  all  the  lifting  gears  shall  be  examined  to  see

whether any parts have been injured or permanently deformed by the

test,

PROCEDURE FOR TESTING

(3)    Derricks;

(a)

A  derrick  shall  be  tested  with  its  boom  at  the  minimum  angle  to  the

horizontal  for  which  the  derrick  is  designed  (generally  15  degrees)  or

at such greater angle as may be agreed. The angle at which the test

has  been  carried  out  shall  be  mentioned    in  the  test  certificate.  The

test  load  shall  be  applied  by  hoisting  moveable  weights.  During  the

test, the boom shall be swung with the test load, as far as practicable,

in both directions.

162

(b)     A  derrick  boom,  designed  to  be  raised  with  power,  with  the  load

suspended, shall, in addition to the tests at (a), be raised with the load

suspended)  to  its  maximum  working  angle  to  the  horizontal  and  the

two outermost positions.

(c)     While  test  loading    of  a  heavy  lift  derrick,  the  competent  person

responsible for tests using moveable weights shall ascertain  from the

owner of the vessel or floating platform that the stability of the vessel

or platform is adequate for the test.

(4)

The  derricks  tested  under  clause(3)  shall  not  be  used  in  union  purchase  rig

unless:

(a)

the  derricks  rigged  in  union  purchase  are  tested  with  the  test  load

appropriate to the SWL, in Union purchase (at the designed  headroom

and with the derrick booms in their approved working positions);

(b)

the  safe  working  load  of  that  derrick  in  union  purchase  rig  has  also

been specified by a competent person in a report in Form-V.

(c)

any  limitations  or  conditions  specified  in  the  said  report  are  complied

with; and

(d)    The  two  hoist  ropes  are  coupled  together  by  a  suitable  swivel

assembly.

Note:

The  safe  working  loads  of  derrick  (for  each  method  of  rig

including  union  purchase  )  shall  be  shown  on  the  Certificate

of test and marked on the derrick booms.

(5)    Lifting

(a)

The  test  load  shall  be  lifted  and  swing,  as  far  as  possible,  in  both

directions. If  the jib or boom of the crane has a variable  radiums, it

shall be tested with test loads at the maximum and minimum radii. In

case of hydraulic cranes when owing to the limitation of pressure, it is

impossible to lift a test load in accordance with table under item(1), it

will be sufficient to lift the greatest possible load which shall be more

than safe working load,

(b)

The  test  shall  be  performed  at  maximum,  minimum  and  intermediate

radius  points  as  well  as  such  points  in  the  are  of  rotation,  as  the

competent  person  may  decide.  The  test  shall  consist  of  hoisting,

lowering, breaking and swinging  through all positions and operations

normally performed. An additional test shall be made by operating the

163

machinery  at  maximum  working  speed  with  the  safe  working  load

suspended.

(6)

Use of spring or hydraulic balances, etc. for test to loading;

All tests shall normally be carried on with the help of dead weights. In

case of periodical test, replacements or renewals, test load may be applied by

means of suitable springs or hydraulic balances. In such case, test load shall

be applied with the boom , as far out as practicable, in both directions, The

test  shall  not  be  taken  as  satisfactory  unless  the  balance  has  been  certified

for accuracy by the competent authority within 2.0 percent  and the pointer of

the  machine  has  remained  constant  at  the  test  load  for  a  period  of  at  least

five minutes.

(7)

Testing machines and dead weights:

(a)

A  suitable  testing  machine  shall  be  used  for  testing  of  chains,  wire

ropes and other lifting gears;

(b)

Testing  machines  and  balance  to  be  used  in  test  loading,  testing  and

checking shall not be used unless they have been certified for accuracy

at  least  once  in  the  preceding  twelve  months  by  the  competent

authority;

(c)

Moveable  weights  used  for  the  test  loading  of  the  lifting  appliances

having  a  safe  working  load  not  exceeding  twenty  tonnes  shall  be

checked  for  accuracy  by  means  of  suitable  weighing  machine  of

certified accuracy.

(8)

Thorough  examination after testing or test loading;

After being tested or test loaded, every lifting appliance and associated

gear shall be thoroughly examined to see that no part has been damaged or

permanently deformed during the test. For this purpose, the lifting appliance

or  gear  shall  be  dismantled  to  the  extent    considered  necessary  by  the

competent person.

SCHEDULE-XI

[(see rule 218 (a)]

Permissible Levels of Certain Chemical Substances in The Work

Environment.

S.No  Substance

Permissible limit of exposure

Average

Short term exposure

concentration (TWA) (8

limit (STEL) ( 15 min)

164

hrs)

ppm

mg   m

ppm

mg   m

5

1.      Acetabldehyde

100            180            150            270

acid

10              25              15              37

3.      Acetone

750           1780          1000          2375

4.      Artolein

0.1            0.25            0.3             0.8

(S.C.)             2              4.5              -                -

6.      Alddrin-Skin

0.25             -                -

cholride

1               3               2               6

8.      Ammonia

35             27

9.      Aniline-Skin

2              10               -                -

10.    Anisidine(O-p-isomers)            0.1           0.5            -              -

Skin

11.

Arsenic and soluble                   -            0.2            -             -

compounds (as As)

12.    Benzene (S.C.)

10             30

-

-

13.

Beryllium and Compound           -           0.002           -              -

(As be) (S.C.)

trifluoride-C

1

3

-

-

15.    Bromine

0.1            0.7            0.3

2

16.    Butane

800           1900             -

-

ethyl         200          590          300          885

Ketone-MBK)

acetate

150            710            200            950

alocohol-skin-C            50            150

-

-

acetate            200            950

-

-

mercaptan

0.5            1.5

-

-

22.

Cadmium Dust and salts            -           0.05           -             -

(as Cd)

oxide

-

2

-

-

(Sevin)

-

5

-

-

(Furadan)

-

0.1

-

-

disulphide-skin            10             30

-

-

monoxide

50             55            400            440

tetrachloride-skin          5

30

-

-

165

(SC)

29.    Chlordance-skin

-

0.5

-

-

30.    Chlorine

1

3

3

9

31.    Chlorobenzene                       75           350            -              -

(monochlorobenzene)

(S.C.)

10             50

-

-

33.

bis (Chloromethy) either         0.001        0.005           -              -

(H.C.)

34.

Chormie acid and                      -           0.05           -             -

chromates (as Cr.) (water

soluble)

35.

Chormous salts (as Cr.)

-

0.5

-

-

fume

-

0.2

-

-

dust,

-

0.2

-

-

isomers-skin             5

22

-

-

CN)skin

-

1

-

-

40.    Cyanogen

10             20

-

-

(Dishlorodipheny              -             1             -             -

trichloroethance)

42.    Demeton-skin

0.01           0.1

-

-

43.    Diazinon-skin

-

0.1

-

-

phthalate

-

5

-

-

45.

Dicholorvos (DDVP)- skin

0.1

1

-

-

skin

-             0.25             -

-

(all               0.15           1             -             -

isomers) skin

48.    Dinitirotoluene-skin

-

1.5

-

-

(Biphenyl)

0.2            1.5

-

-

(Thiodan)-              -            0.1            -             -

Skin

51.    Endrin-skin

-

0.1

-

-

acetate

400           1400             -

-

alcohol

1000          1900             -

-

54.    Ethylamine

10             18

-

-

55.

Fluorides (as F)

-

2.5

-

-

56.    Fluorine

1

2

2

4

166

(S.C.)

1.0            1.5

2

3

acid

5

9

-

-

59.    Gasoline

300            900            500           1500

60

Hydrogen Skin (S.C)

0.1

0.1

-

-

61.

Hydrogen chloride-C

5

9

-

-

cyanide-skin-c          10             10

-

-

63.

Hydrogenfluorine (as F) –          3            2.5            -             -

C

64.    Hydrogen peroxide

1

1.5

-

-

sulphide

10             14             15             21

66.    lodine-C

0.1

1

-

-

67.

Iron Oxide Fume (FeD)              -             5             -             -

(as Fe)

acetate

100            525

-

-

alcohol

100            360            125            450

alcohol

50            150

-

-

dusts                     -           0.15           -             -

fumes (as Pb)

72.    Lindane-skin

-

0.5

-

-

73.    Malathion-skin

-

10

-

-

74.

Manganese dust and                 -             5             -             -

compounds(as Mn)-C

75.

Manganese fume (as Mn)

-

1

-

-

Hg)-skin

(i)Alkyl compounds

-

0.01

-0.03

(ii)All forms except alkyl

-

0.05

--

vapour

(iii) Atyl and inorganice

-

0.1

-

-

compounds

alcohol                       200          260          250          310

(Methanol)-skin

collosolve                     5

16            -             -

(2Methoxy-ethanol)-skin

79.

Methyl isobutyl ketone

50

205

75

300

skin          0.02           0.05             -

-

81.    Naphthalene

10             50             15             75

167

82.    Nickel

-

83.    Nitric

2

5

4

10

84.    Nitric

25             30

-

-

85.    Nitrobenzene- skin

1

5

-

-

86.    Nitrogn dioxide

3

6

5

10

87.    Oil

-

5

-

10

88.    Ozone

0.1            0.2            0.3            0.6

89.    Parathion-

-

0.1

-

-

90.    Phenol-skin

5

19

-

-

91.

Phorate (Thimet)- skin

-

0.05

-

0.2

92.    Phosgene

chloride)

93.    Phosphine

0.3            0.4

1

1

94.    Phosphoric

-

1

-

3

95.    Phosphorus

-

0.1

-

-

96.    Phosphosrus                          0.1            1             -             -

pentachloride

97.    Phosphorus

3

98.    Pierie

-

0.1

-

0.3

99.    Pyridine

5

15

-

-

100.   Silane

tetrahydride)

101    Sodium hydroxide-C

-

2

-

-

102.   Styrene, monomer                   50           215                            425

(Phenylethylene)

103.   Sulphur

2

5

5

10

104.   Sulphur

1000

6000

-

-

105.   Sulphuric

-

1

-

-

106.

Tetraethyl lead (as Pb)              -            0.1            -             -

skin

107.   Tolune (Toluol)

100

375

150

560

108.   O-Toluidine-skin

2

9

-

-

109.   Tributvi

0.2

2.5

-

-

110.   Trichlowthylene

50

270

200

1080

111.   Uranium,

-

0.2

-

0.6

112.   Vinyl

5

10

-

-

168

fumes

-

5

-

-

114.   Xylene (o-,m-,p-isomers)         100

435

150

655

oxide

-

-

-

-

(i)Fume

-             5.0              -              10

(ii)Dust (Total dust)

-

10.0

-

-

compounds (as          -             5             -             10

Zr)

Ppm

Parts of vapor or gas per million parts of contaminated air by volume

at 25 C and 760 mm of Hg.

mg   m

milligram of substance per cubic meter of air

*

Not  more  than  4  times  a  day  with  at  least  60  min.  interval  between

successive  exposures.

Molecularweight× ppm 
	24.45

G

denotes Ceiling Limit

Skin

denotes potential contribution to the overall exposure by the cutaneous

route including mucous membranes and eye.

S.C.

denotes Suspected Human Carcinogen.

H.C.

denotes Confirmed Human Carcinogen

Substance concentration

Permissible time weighted average

(TWA) (8 Hrs.)

Silica, SiO

(a)     Crystalline

(i)     Quartz

(1)

In terms of dust count

10600

% Quartz+ 10

(2)

In terms of respirable dust

10

% respirable Quartz+ 2

mg   ml

(3)

in terms of total dust

30

% Quartz+ 3

mg   ml

(ii)

Cristobalite

Half the limits given against quartz.

(iii)

Tridvmite

Half the limits given against quartz.

169

(iv)

Silica, fused

Same limits as for quartz.

(v)

Tripoli

Same limits as in formula in item (2) given against

quartz.

(b)

Amorphous Silicates 10 mg/m Total dust

Asbestos  (h.c)

*  2  fibres/  ml.  greater  than  5  um  in  length  and

less  than  3  um  in  breadth  with  length  to  breadth

ratio equal to or greater than 3:I

Portland

10  mg/m,  Total  dust  containing  ess  than  1%

quartz. Coal Dust 2mg/m3, respirable dust fraction

containing less than 5% Quartz.

mmpcm

Million  particles  per  cubic  meter  of  air,  based  on  impinger  samples

counted by light field techniques.

*

As   determined   by   the   membrane   filter   method   at   400-450x

magnification (4 mm objective) phase contrast illumination.

Respirable Dust:

Fraction passing a size selector with the following characteristics:

Aerodynamic Diameter (um)

% passing selector

(unit density sphere)

<2                                  90

2.5                                  75

3.5                                  50

5.0                                  25

10                                  00

SCHEDULE-XII

{(see rule 275 (1) and (2)}

No. Of Safety Officers, Qualification, Duties, Etc.

Appointment of Safety Officers.

Number  of  Safety  Officers-Within  six  months  of  coming  into  operation  of

these  Rules,  every  establishment  employing  more  than  five  hundred  building

workers and every other employer of building worker shall appoint safety officers, as

laid down in the scale given below:-

170

1.

Upto  1000 building workers-one safety officer.

2.

Upto 2000 building workers-two safety officers.

3.

Upto 5000 building workers-three safety officers.

4.

Upto 10000 building workers-four safety officers.

For every additional 5000 building workers or part thereof-one safety officer.

Any  appointment,  when  made  shall  be  notified  to  the  inspector  having,

jurisdiction in the area, giving full details of the qualification, terms and conditions of

service of such safety officer.

Qualification

(a)

A person shall not be legible for appointment as a safety officer unless he;

(i)     possesses  a  recognised  degree  in  any  branch  of  engineering  or

technology  or architecture and had a practical experience of working

in a building or other construction work in a supervisory capacity for a

period  of not less than two years or possesses a recognized diploma in

any  branch  of  engineering  or  technology  and  has  had  practical

experience  of  building  or  other  construction  work  in  a  supervisory

capacity for a period of not less than five years;

(ii)

possesses  a  recognised  degree  or  diploma  in  industrial  safety  with  at

least one paper in construction safety(as an elective subject);

(iii)    has  adequate  knowledge  of  the  language  spoken  by  majority  of

building  workers  from  the  construction  site  in  which  he  is    to  be

appointed;

(b)

Notwithstanding the provision contained in clause(a) any person who-

(i)

possesses a recognised degree or diploma in engineering or technology

or  architecture  and  has  had  experience  of  not  less  than  five  years  in

the field, dealing with the administration of Factories Act, 1948  or the

Dock  Workers  (Safety,  Health  and  Welfare)  Act,  1986  or  the  Building

and   Other   Construction   workers(Regulation   of   Employment   and

Conditions of Service)Act, 1996;

(ii)

possess  a  recognised  degree  or  diploma  in  engineering  or  technology

and has had experience of not less than five years or has under gone

171

training  in  education,  consultancy  or  research  in  the  field  or  accident

prevention  in  industry,  port,  or  in  any  institution  or  an  establishment

dealing with building or other construction work,

shall also be eligible for appointment as a Safety officer:

Provided that, in case of person who has been working as safety officer

in  industry  or  port,  institution  or  an  establishment  dealing  with  building  or

other construction work for a period of -not less than three years on the date

of commencement of these Rules, the Chief Inspector  may, subject to such

conditions that he may specify, relax all or any of the above said qualification.

Condition of Service

(a)

Where number of safety officers appointed exceeds one, one of them

shall  be  designated  as  Chief  Safety  Officer,  and  shall  have  the  status

higher  than  the  others.  The  Chief  Safety  officer  shall  be  in  over  all

charge of the safety functions as envisaged in sub-clause(iv) and also

other safety officers working under his control;

(b)

the Chief Safety Officer or Safety Officer, where only one safety officer

is  appointed,  shall  be  given  the  status  of  a  Senior  Executive    and  he

shall  work  directly  under  the  control  of  his  Chief  Executive.  All  other

Safety  Officers  shall  be  given  appropriate  status  to  enable  them  to

dispatch their functions effectively;

(c)

the  scale  of  pay  and  allowances  to  be  granted  to  the  safety  officers

including  the  Chief  Safety  Officer  and  the  other  conditions  of  their

service  shall  be  the  same  as  those  of  the  officers  of  corresponding

status of the establishment in which they are employed.

Duties  of  Safety  Officer-(a)  The  duties  of  a  Safety  Officer  shall  be  to

advise and assist the employer in the fulfillment of his obligations, statutory

or  otherwise  concerning  prevention  of  personal  injuries  and  maintaining  a

safe working environment. These duties shall include the followings namely:-

(i)

to  advise  the  building  workers,  in  planning  and  organizing  measures

necessary for effective control of personal injuries;

(ii)

to advise on safety aspects in a building or other construction work and

to carry out detailed safety studies of selected activities;

172

(iii)

to check and evaluate the effectiveness of action taken or proposed to

be taken to prevent personal injuries.

(iv)    to  advise  purchasing    and  ensuring  quality  of  personal  protective

equipment confirming to national standards;

(v)

to carry out safety inspections of building or other construction work in

order to observe the physical conditions of work and the work practices

and procedures followed by building workers and to render advice on

measures  to  be  adopted  for  removing  unsafe  physical  conditions  and

preventing unsafe actions by building workers;

(vi )

to investigate all fatal and other selected accidents;

to

cases  of  occupational  diseases  contracted  and

reportable dangerous occurrences;

(Viii)   to  advise  on  the  maintenance  of  such  records  as  are  necessary  with

regard to  accidents, dangerous occurrences and occupational disease;

(ix)

to promote the working of safety committees and to act as an advisor

to such committees;

(x)

to  organize,  in  association  with  concerned  departments,  campaigns,

competitions,  contents    and  other  activities  which  will  develop  and

maintain  the  interest  of  building  workers  in  establishing  and  maintain

safe conditions of work and procedures;

(xi)     to  design  and  conduct  either  independently  or  in  collaboration  with

other  agencies,  suitable  training  and  educational  programmes  for

prevention of accidents to building workers;

(xii)    to  frame  safe  Rules  and  safe  working  practices  in  consultation  with

senior officials of the establishment;

(xiii)   supervise  and  guide  safety  precautions  to  be  taken  in  building  and

other construction work of the  establishment.

Facilities  to  be  provided  to  safety  of  officers.-  The  employer  shall

provide  each  safety  officer  with  such  facilities,  equipment  and  information

that are necessary to enable him to dispatch his duties effectively.

Prohibition  of  performance  of  other  duties.-  No  safety  officer  shall  be

required  or  permitted  to  do  any  work  which  is  unconnected  to,  inconsistent

with  or  detrimental  to  the  performance  of  the  duties  prescribed  in  this

Schedule.

173

Exemption. Chief Inspector  may, in writing, exempt any employer or  group  from

any or all of the provisions of these Rules subject to compliance  with

such alternative arrangements as may be approved and notified by him

in the order or such exemption.

FORM-I

(see rule 17) (1)

APPLICATION FOR REGISTRATION OF ESTABLISHMENT EMPLOYING

BUILDING WORKERS.

1.

Name and location of the Establishment where building or other construction

work is to be carried on.

2.

Postal address of the Establishment.

3.

Full name and permanent address of the Establishment, if any.

4.      Full  name  and  address  of  the  Manager  or  person  responsible  for  the

supervision and control of the Establishment.

5.

Nature of building or other construction work carried/is to be carried on in the

Establishment

6.

Maximum number of building workers to be employed on any day.

7.

Estimated date of commencement of building or the other construction work.

8.

Estimated date of completion of the building or other construction work.

9.

Particulars  of  demand  draft,    enclosed  (name  of  the  bank  amount,  demand

draft No. and date)

Deceleration by the employer

174

i)

I hereby declare that the particulars given above are true to the best of

my knowledge and belief.

I  under-take  to  abide  by  the  provisions  of  the  Building  and  other

Construction  workers  (Regulation    of  Employment  and  Conditions  of

Service) Act, 1996 and  the Rules made there-under.

Employer

Seal and Stamp

Office of the Registering Officer appointed under the Building and other Construction

Workers (Regulation of Employment and Conditions of Service) Act, 1996 and Rules

made there-under.

Date of receipt of application;

FORM-II

(see rule 18 (1)

GOVERNMENT OF HARYANA.

OFFICE OF THE REGISTERING OFFICER.

No.:

Date:

A  certificate  of  Registration  is  hereby  granted  under  Sub-section  (3)  of

Section 7 of the Building and Other Construction Work (Regulation  of Employment

and Conditions of service ) Act, 1996 and the Rules made there-under to M/s --------

--------------------------having the  following particulars subject to conditions laid down

in the Annexure:

1.

Postal  Address/location  where  building  or  other  construction  work  is  to  be

carried on by the employer.

2.

Name  and  address  of  employer  including  location  of  the  building  and  other

construction work.

3.

Name and permanent address of the establishment.

175

4.

Nature  of  work  in  which  building  workers  are  employed  or  are  to  be

employed.

5.

Maximum  number  of  building  workers  to  be  employed  on  any  day  by  the

employer.

6.

Probable date of commencement and completion of work.

7.

Other particulars relevant to the  employment of building workers.

Signature of Registering Officer with Seal

Annexure

The registration granted herein above is subject to the following conditions, namely-

(a)

the certificate of registration shall be non-transferable;

(b)

the  number  of  workmen  employed  or  building  workers  in  the  establishment

shall    not  on  any  day  exceed  the  maximum    number  specified  in  the

certificate of registration;

(c)

save  or    provided  in  these  Rules,  the  fees  paid  for  the  grant  of  registration

certificate shall be non-refundable;

(d)

the rates of wages payable to building workers by the employer shall not be

less  than  the  rates  prescribed  under  the  Minimum  Wages  Act,  1948  (II  of

1948)  for  such  employment  where  applicable,  and    where  the  rates  have

been  fixed  by  agreement  settlement  or  award,  not  less  than  the  rates  so

fixed; and

(e)

the employer

shall

comply with

the

provisions of

the Act and the

Rules

made there-under.

FORM-III

[(see rule19 (2)]

REGISTER OF ESTABLISHMENTS

S.N   Registration   Name  and  Address  location  Name  of  the  Nature  of

176

Employer                         other

construction work

or other construction work is  address

to be carried on

1.         2.                          3.                          4.                  5.

Maximum   No.   Probable  duration  of   Remarks

permanent

of

building    or    other

commencement  workers  to  be  construction     work

Establishment

of work

and probable date of

any day.

completion.

10.

FORM IV

[(see rule 20 (3) and 86 (I)]

NOTICE OF COMMENCEMENT/COMPLETION OF BUILDINGOR OTHER

CONSTRUCTION WORK.

1.

(i)

Name and address (Permanent) of the establishment.

(ii)

Name of the employer and address.

2.

Name  and  situation  of  place  where  the  building  and  other  construction  is

proposed  to  be carried on.

3.

No. and date of Certificate of registration------------------------------

4.

Name and address of the person in-charge of the construction work.

5.      Address   to   which   the   communications   relating   to   building   or   other

construction work may be sent.

6.

Nature  of  work  involved  and  the  facilities  including  plant  or  machinery

provided.

7.

The arrangement storage of explosives, if any, to be used in building or other

construction work.

177

8.

In case the notice is for  commencement of work, the approximate   duration

of work

I/We  hereby  intimate  that  the  building  or  other  construction  work

(Name of work) having registration No.----------------------- dated----------------

------is likely to commence is likely to be completed with effect from------------

-----(date/ on (date).

Signature        Employer

To,

The Inspector,

FORM V

[{See Rule 28 (4)]

APPLICATION FOR REGISTRATION

1.    Name

2.    Address 
3.    Whether

:

:

:

4.  
5.

Name of Father

Martial Status

:

:

(Married, Unmarried or widow)

6.    Date

:

7.

Name, address and Register No.      :

of the Establishment where The

applicant is Working

8.  
9.

E.S.I. / P.F. No.

Name and address of employer

:

:

10.   Total

:

178

11.

Rate of Subscription

:

12.

Name of Bank and Branch, where    :

Subscription is to be paid

13.

If the applicant is already a

:

member of any other Welfare

Board, the name of such Boards

and registration No. of the

applicant.

The above facts are true to the best of my knowledge and information.

Place:

Date:

Signature        applicant.

Name and Signature of Employer.

FORM VI

[(see rule 28 (7)]

NOMINATION FORM

I  Nominate  the  following  persons  as  rightful  dependants,  to  receive  all  the

dues from the Fund on my behalf and in the event of my death, as rightful heirs to

receive all benefits due to me.

Name and address Of

Relationship with

Age of Nominee

Amount to be given

Nominee/ Nominees

member

to each Nominee

Place:

Date:

179

Name, Address Registration No. and

Address of the worker

FORM VII

[see rule 28 (8) ]

FORM OF IDENTITY CARD

PAGE-I

PHOTO

Signature, date and official

Designation of the registering

Authority (with office seal)

PAGE-II

Name of Member

:

Address

Male / Female

:

Name of job

:

Registration No.

:

District

Date of Registration

:

Name of Bank and Branch in

:

Which subscription is to Be paid

Subscription rate

: Rs. 20

180

PAGE-III

Date of birth

:

Completed age

:

Date of retirement

:

Marital status Married / Unmarried

:

Name of Wife / Husband

:

Address                                       :

Whether wife / husband, a Member of this

:Yes / No

Board

If so, name and registrationNumber

:

Name of nominees

:

Relationship with the member

:

Signature / Thumb impression of the

:

member

:

Official designation and

signature of Registering Authority

FORM VIII

[see rule 28 (8) ]

REGISTER OF IDENTITY CARD

Name of district  _________________

S. No.

No. of Identity

Date of issue

Name and address Of

Signature of District

Remarks

Card

the Worker

Executive Officer

(1)          (2)              (3)                 (4)                    (5)             (6)

181

FORM IX

[(see rule 30 (2)]

HARYANA BUILDING AND OTHER CONSTRUCTION WORKERS WELFARE 
	BOARD

Return for the month of______________regarding the details of workers

Name and Address of the Establishment

:

S. No.   No. of workers as  No.  and  Name  (s)   No.   and   Name   No.  of  workers

on  the  Close    of   of  workers (s) who  (s) of worker (s)  as  on  the  close

previous Month

left  service  during  to be registered

of current Month

the month

Place:

Date:

Name and Signature of the Employer

(Office Seal)

FORM X

[see rule 30 (3)]

HARYANA BUILDING AND OTHER CONSTRUCTION WORKERS WELFARE

BOARD

Particulars Of Establishment

1.

Name of the Establishment

:

2.     Nature  of  Establishment  Whether  :

Company  /  Partnership  Firm  /  Sole

Proprietorship

3.

Names of the partners / Directors /  :

proprietor

182

4.    Name   of   Managing   partner   /

Managing  Director  /  person  who  is

in    ultimate    control    of    The

establishment

5.     Details

:

6.

Details of occupiers

:

Name, Signature and Designation

Office Seal)

Place:

Date:

FORM XI

[see rule 50]

APPLICATION FOR MATERNITY BENEFIT

1.

Name and address of applicant

:

2.        Registration

:

3.  
4.

Age and date of birth

Name of husband

:

:

5.        Date

:

6.  
7.

Have you applied for this benefit earlier

If so how many times and give details

:

:

8.        Date

:

9.

Date of payment of 1st subscription and amount

:

10.

Date of payment or last subscription

:

11.

Name of bank and place

:

12.      List

(a)

Copy of Challans or copy of Pass book

:

(b)

Medical certificate in original.

:

The facts furnished above are true to my knowledge and information.

Place:

Name

183

Date:

FORM OF MEDICAL  CERTIFICATE

(To be obtained form a Medical Officer not below the rank of an Assistant Surgeon)

I  have  examined  Smt.  _________________________  age  __________and

wife  of  Shri  _______________________________________    she  is  pregnant

running  ___________________________________  month.    She  had  delivered  a

child on______________.

Place:

Name

Date:

FORM XII

[see rule 52 (1) ]

APPLICATION FOR PENSION

1.   Name and Address of applicant

:

2.   Registration

:

3.   Date of Completion of 60 years

:

4.   Date of payment of 1st subscription amount and name  :

of Bank

5.   Default if any and reasons thereof

:

6.   Date of payment of last subscription amount, date and  :

name of Bank.

7.   List of documents

:

(a) Identity Card

(b)

(c)

8.

Address at which pension is to be sent

:

9.   Any  other  information  (Details  of  benefit  if  any,  from   :

other welfare Boards)

The facts mentioned above are true to my knowledge and information.

184

Place:

Name

Dated:

FORM XIII

[see rule 52(6)]

REGISTER OF PAYMENT OF PENSION

PPO       Name          and

No.   and   date

No.        address   of   the

retirement

Service

Of   Order   of

pensioner     with              of

sanctioning

Membership   No.

Authority

in              The

D.B.O.C.W.W.

scheme

Board

(1)             (2)               (3)

Date       of

Remarks

Commence

pension Rs.

Secretary D.E.O.     Order   on   cancellation   of

ment      of

pension  etc.  may  be  noted

pension

here  with  reason  and  date

Effect    under    initials    of

Secretary/DEO

(7)                (8)                   (9)                         (10)

185

Details of Pension Paid

Month  /   Amount       of                    of

Remarks (Details of undelivered

Year

Pension               sending      of

H.O. etc. may be noted here

Money Order

(11)          (12)             (13)                        (14)                                  (15)

FORM XIV

[[see rule 53](1)]

HARYANA BUILDING AND OTHER CONSTRUCTION WORKERS WELFARE

BOARD

Application

Fee

APPLICATION FOR HBA

1.

(a)

Name of the appliant

:

(b)

:

(c)

:

2.

Date of Birth

:

3.

Date of retirement

:

4.    (a)    Register

:

(b)

Date of Registration

:

186

(c)

rate of remittance

:

(d)

Date of first remittance

:

(e)

Date of last remittance

:

(f)

Total amount remitted

:

(g)

Whether the membership has Ever been revived,

:

if so Details

Details

:

5.

Purpose of advance (new construction/

:

Maintenance/purchase of land with Building)

6.

Whether the applicant has a house of his own

:

(give details)

7.

Amount of advance required

:

8.

Details of landproperty

:

(a)

:

(b)

:

(c)

:

(d)

:

(e)

:

(f)               No.

:

(g)

Valuation of the property

:

9.

Whether the applicant has received Any other loan

:

for House Building Advance, given Details

10.

Estimate for construction/ Maintenance of building

:

as per plan

11.

Details of the amount raised apart from the loan

:

12.

Whether the applicant has received loan

:

previously from this Board

DECLARATION

I hereby declare that the above statements are true and correct to the best of

my knowledge and belief.

Signature

Date:

Name:

187

Details of documents to be produced:

1.  
2.

Plan and estimate (approved)

Encumbrance Certificate of 14 years

3.      Location

4.      Land

5.      Original

6.  
7.

8.  
9.

Attested copy of ration card (page 2,4) for maintenance application.

Ownership of the building (for maintenance only)

Terminal benefit declaration

Attested copies of identity card and passbook

10.

Title clearance certificate

11.

Age certificate of the building (for maintenance only)

12.

Valuation certificate of the building (for maintenance

13.

No  objection certificate from the authorities for construction.

14.

Declaration from the applicant that neither he/she/nor his/her spouse or

children own a house (for new construction).

MORTGAGE DEED

This  Deed  of  Mortgage  is  executed  on  this  the  .......................  day  of

..................  two  thousand  and  ......………….by  Shri/Smt  .........................………...

son  /  daughter  /  wife  of  .......................................  aged  .................  residing  at.

village  ....................  Taluk  ......................  District  .......................  and  Shri/Smt

................... son / daughter / wife  of Shri ..................... aged .............. residing at

village .............…….. Taluk ............…..... District ............................... (Thereinafter

called   the   Mortgagor/Mortgagors   which   expression   shall   include   his/her/their

executers, administers, legal representatives and assigns) in favour of the Haryana

Building  and  Other  Construction  Workers  Welfare  Board  established  under  the

Building and Other Construction Workers Welfare Act and having its Head Office at

Chandigarh  (thereinafter    called  'the  Mortgage'  which  expression  shall  include  its

successors  or  assigns  wherever  the  context  or  meaning  thereof  shall  so  require  or

permit).

188

Whereas  the  Mortgagor/Mortgagors  has/have  applied  to  the  Mortgage  for  a

loan of Rs. 50,000 (Rupees fifty thousand only for the construction of a house on the

land, more particularly mentioned and described in the schedule hereunder written:-

AND  WHEREAS  on  the  request  of  the  Mortgager/Mortgagors  the  Mortgagee

has  agreed  to  lent  an  advance  in  two  installments  to  be  mortgager  a  loan  of  Rs.

50,000 (Rupees fifty thousand only) subject to the covenants, terms and conditions

herein  contained  and  having  the  repayment  thereof,  secured  in  the  manner

hereinafter expressed.

NOW THIS DEED WITNESSETH AS FOLLOWS

1.

In pursuance of the said agreement and in consideration of  the sum of Rs.

50,000 (Rupees fifty thousand only) now lent  and advance/and paid by the

Mortgagee to the Mortgagor/Mortgagors, (the receipt whereof the Mortgagor

hereby   admits   and   acknowledges)   the   Mortgagor/Mortgagors   hereby

transfers/transfer by way of simple Mortgage the immovable property, more

particularly  mentioned  and  described  in  the  schedule  hereunder  written

together   with   the   building   to   be   constructed   thereon   and   other

improvements  thereon  from  time  to  time  to  the  intent  that  of  the  said

property  and  the  building  and  other    improvements    shall    remain    and    be

charged  as  security  for payment to the Mortgagee of the said loan amount

interest  and  cost  and  the  Mortgagee  shall  have  the  first  charge  over  the

same.

2.

The loan amount shall be paid to the Mortgagor/Mortgagors by the Mortgagee

in  two  instalments  that  the  first  instalment  of  a  sum  of  Rs.  20,000  (Rupees

Twenty thousand only) equal to 40% of the loan sanctioned shall be paid to

the  Mortgagor/Mortgagors  for  starting  construction,  that  the  2nd  and  final

instalment of Rs. 30,000 (Rupees thirty thousand only) equal to 60 % of the

loan  shall  be  paid  after  completing  the  construction  of  roof  and  on  starting

finishing  works.    The  construction  of  the  building  shall  be  completed  in  all

respects  utilizing  the  2nd  instalment  and  certificate  of  completion  shall  be

produced within two months from the receipt of last instalment.

3.

The instalments shall be paid only subject to the availability of funds and the

non-payment  of  amounts  due  to  paucity  of  funds  shall  not  entitle  the

Mortgagor/Mortgagors  to  realize  any  loss  that  he/she/they  may  sustain  on

that account from the Mortgagee.

189

4.

The  Mortgagor/Mortgagors  hereby  assures/assure  upto  the  Mortgagee  that

he/she/they  is/are  the  absolute  owners  of  the    property  mentioned  in  the

schedule  hereto  and  that  they  are  free  from  any  encumbrance  or  charge  of

any description, whatsoever or any attachment or restraints on alienation.

5.

The  Mortgagor/Mortgagors  shall  not  at  any  time  during  the  continuance  of

this  security  create  any  Mortgage  lien  or  charge  by  way  of  hypothecation,

pledge or otherwise create encumbrance of any kind whatsoever in respect of

the properties described in the schedule hereto or any part thereof, or let or

lease them except with the prior permission in writing of the Chief Executive

Officer,  Delhi  Building  and  Other  Construction  Workers  Welfare  Board  until

the whole amount with interest are fully repaid.

6.

The loan shall bear interest at the rate of 5% per annum or such other higher

rate of interest as may be fixed by the Mortgagee from time to time.

7.

The loan shall be repaid by the Mortgagor/Mortgagors in monthly instalments

at  the  rate  as  would  be  fixed  and  intimated  by  the  Mortgagee.    The  first

instalment  becoming  due  on  the  expiry  of  six  months  from  the  date  of

disbursement of the first instalment, subsequent instalments shall be paid on

or  before  the  10th  day  of  succeeding  month  for  167  months.    Any  interest

due on the loan amount outstanding on the date of payment of an instalment

shall be paid along with the instalment.

8.

At the time of  disbursement  of  the  2nd  instalment  the  Mortgagee shall

deduct the interest and other expenses due on the 1st instalment till the date

of payment of the 2nd instalment.  If the Mortgagee pays only a part of the

loan amount to the Mortgagor due to the non-availability of funds  such part

of  the  loan  shall  be  repaid  by  the  Mortgager  in  instalments  at  the  rate  as

would be fixed and intimated by the  Mortgagee.

9.

If  the  Mortgagor  /  Mortgagors  dies  /  die  before  the  disbursement  of  the

remaining  instalments  of  the  loan  after  having  received  one  or  more

instalments  of  the  loan  and  if  his  /  her  /  their  heir  or  heirs  executor  /

executors  refuses  /  refuse  to  avail  of  the  remaining  instalment  and  also

refuses  or  refuse  to  complete  the  construction  of  the  house  according  the

approved plan and estimate within one year after the date of disbursement of

the first instalment of the loan the whole loan advance with interest shall be

liable  to  be  summarily  recovered  by  proceedings  against  the  property

movable  or  immovable  of  the  deceased  Mortgagor  /  Mortgagors  under  the

provisions  of  the  revenue  recovery  at  for  the  time  being  enforced  and  the

190

relevant  provisions  of  the  Haryana  Building  and  Other  Construction  Workers

Welfare Rules, as if some were arrears of public revenue due on  land or in

such other manner as the Mortgagee may deem fit.

10.

If the heir / heirs executors of the deceased Mortgagor / Mortgagors does /

do not require the balance instalments of the  loan and are, however willing

to complete the construction at her / his/ their cost, the amount already paid

to the Mortgagor / Mortgagors out of the sanctioned loans will be treated as

the actual amount of the loan sanctioned and the recovery shall be effective

at the rate of instalment prescribed for that amount of loan.

11.    The  Mortgagor  /  Mortgagors  shall  remit  the  intalments  in  the  Banks

prescribed  by  the  Mortgagee  in  the  manner  specified  for  this  purpose  or  by

the  challans  prescribed  by  the  Haryana  Building  and  Other  Construction

Workers Welfare Board.

12.

If  any  instalment  of  principal  or  interest  is  not  remitted  on  the  due  dates  a

penal  interest  at  the  rate  of  5%  in  addition  to  the  usual  rates  shall  be  paid

and such amount as are not paid on due dates.

13.     The  Loan  amount  shall  be  utilized  only  for  the  purpose  for  which  it  is

sanctioned.  Each instalment of the loan referred to  in clause II above shall

be  utilized  within  the  time  limit  prescribed.    Incase  the  Mortgagor  /

Mortgagors fails / fail to claim the subsequent instalment within three months

from the drawal of the previous instalments such  previous  instalment  shall

be    treated    as    the    last    instalment    unless    the  time  is  extended  by  the

mortgagee  and  recovery  shall  commence  as  provided  in  the  terms  and

conditions prescribed for the grant of the loan.

14.

If  the  Mortgagor  /  Mortgagors  fails  /  fail  to  utilize  any  instalment  of  loan

within  the  maximum  period  admissible  and  does  not  apply  for  subsequent

instalment  of  loan  as  provided  in  the  conditions  the  entire  amount  already

disbursed  shall  be  recoverable  from  him  /  her  /  them  with  interest  in

lumpsum.

(a)

If the Mortgagor / Mortgagors is / are found to have failed in utilizing

the amount for the construction of house as specified in the mortgage

deed within the prescribed period, the mortgagee is entitled to realize

the entire loan amount plus other charges with interest in a lump after

the issuance of a registered notice directing to pay the amount within a

period of 30 days.

191

(i)     If  the  Mortgagor  /  Mortgagors  repay  the  amount  due  in

lumpsum within  the stipulated period the mortgage deed shall

be  released.

(ii)

If  the  Mortgagor  /  Mortgagors  fails  /  fail  to  repay  the  amount

due  within  the  period  of  60  days  as  stipulated  above  the

mortgagee will have the right to take step to realize the entire

dues  to  the  Board  in  lump.    In  addition  to  that  a  penalty  not

exceeding  5%  of  the  loan  amount  actually  received  by  the

loanee  or  Rs.  1000  (Rupees  one  thousand  only)  whichever  is

higher shall  also be realized from the Mortgagor / Mortgagors.

15.

In the event of any information furnished in the  application being found false

or  materially  incorrect,  the    Mortgagee  shall  be  cancel  the  loan  and  recover

the  entire  amount  outstanding  in  lump  with  interest  accrued  thereon  by

selling  the  mortgaged  property  besides  taking  such  legal  action  against  the

borrower as may be considered desirable.

16.

The Mortgagor / Mortgagors shall not alter or modify the building constructed

in accordance with the plan approved by the  Mortgagee so as to diminish the

value of the property or construct any other building in the property, offered

as security till the entire amount with interest are repaid.

17.     In  case  of  the  Mortgagor  /  Mortgagors  at  any  time  make  default  in  the

payment of two consecutive instalments or commits  breach of all or any of

the terms and conditions contained herein the balance of the principal of sum

which  shall  for  the  time  being  remain  unpaid  together  with  interest  accrued

thereon  and all sums found due to the Mortgagee under or by virtue of these

presents shall forthwith become payable in a lump at  once  and  in  case  of

default  of  payment  of  the  whole  sum immediately the Mortgagee shall

have  power  without  the  intervention  of  any  court  to  take  possession  of  the

Mortgaged property and to sell the same.  The balance of the sale proceeds

after  adjusting  all  amounts  due  to  the  Mortgagee  will  be  disbursed  to  the

Mortgagor    The  Mortgagee  shall  also  have  all  the  powers  vested  in  the

Mortgagee under the provision of the Transfer of property Act, 1882.

18.     Without  prejudice  to  any  or  all  of  the  other  rights  and  remedies  of  the

Mortgagee all sums found due to the Mortgagee under or by virtue of these

presents shall be recoverable from the Mortgagor / Mortgagors and his / her /

their properties, movable and immovable under the provisions of the Revenue

Recovery Act for the time being in force as though they are arrears of Public

192

Revenue  due  on  land  and  in  accordance  with  the  relevant  provisions  of  the

Haryana Building and Other Construction Workers Act in any other manner as

the Mortgagee may deem fit.

19.

The Mortgagor / Mortgagors shall be bound by the terms of the  application

form and the conditions attached thereto which shall form part of this deed as

if they are incorporated on this deed.

20.

This  Mortgage  has  been  fully  explained  to  the  Mortgagor  /  Mortgagors  and

the  Mortgagor  /  Mortgagors  has  /  have  executed  these  presents  fully

understanding  the  implications  thereof  and  all    his  /  her  /  their  obligations

thereunder and after receiving such advice.

THE SCHEDULE ABOVE REFERRED TO

(here enter details of all land and buildings)

IN  WITNESS  WHEREOF  SHRI  _____________________________________,

the Mortgagor (s) here to set his / her / their hands the day and year first above,

written  and  signed  by  Shri  /  Smt.  _____________________________________  in

the presence of witness :

1.

2.

Signed by Shri / Smt. ______________________________________ in the presence

of witnesses:

1.

2.

STAGE CERTIFICATE FOR RELEASE OF SECOND INSTALMENT OF ADVANCE

SANCTIONED BY THE HARYANA BUILDING AND OTHER CONSTRUCTION

WORKERS WELFARE BOARD UNDER HOUSING LOAN SCHEME

BENEFICIARY

PROPERTY

No.______________          District___________________

2.

Name_____________________

Taluk  ___________________

193

3.     Address_____________________          Village___________________

No.__________________

The Construction of building in the property detailed above by the beneficiary

specified  above    has  reached  /  completion  of    foundation  basement  and  on

completion work upto lintel level/ completion of the lintel work / completion of the

linter work and 50 % of the work of the roof and stored the materials for the  work

of  shutters  /  completion  of  the  roof  work  and  has    been  completed  40  %  of  the

finished work as per the plan and the beneficiary is eligible for the second instalment

of  the  loan,    sanctioned  by  the  Haryana  Building  and  Other  Construction  Workers'

Welfare Board.

Certified that the work valued at Rs._____________has been carried out by

the beneficiary as on________________.

Place:

Date:

Signature of District, Executive

Officer / T.E.O. or any Authorised

Officer with name and designation.

Name of office.

FORM XV

(see rule 54 (2) ]

APPLICATION FOR DISABILITY PENSION

1.

Name and address of applicant

:

2.

Age and Date of Birth

:

registration                                           :

4.

Date of payment of first subscription

:

amount and Name of Bank and Branch

194

5,

Date of payment of last subscription

:

amount and name of bank.

6.

Total amount of subscription

:

7.

Details of disease/ accident

:

8.

Nature of disability die to disease/accident

:

9.

Details of treatment in Government hospital

:

Date of admission and date of discharge.

10.

Whether the patient was in plaster ? If so,

:

for how many days ?

11.

Amount spent for treatment (should be

:

supported by medical bills countersigned

by the treating doctor)

12.

List of documents submitted

:

13.

Details of benefits received, if any                  :

before.

14.

Details of benefits received , if any

:

from Government or any other institution,

for the above treatment.

The above facts are true to my knowledge and information.

Place

:

Dated

:

Name and Signature of Applicant

FORM XVI

(see rule 55)

APPLICATION FOR INSTRUMENT LOAN

NO.

Rs:

1.

Name of the Applicant

:

2.

Father’s /Husband’s Name

:

Residential                                           :

No.

:

195

5.

Name of Bank in which contribution

:

remitted.

6.

Age and Date of Birth

:

Income

8.

Details of other properties if any, owned

:

or possessed by the applicant

9.

DETAILS OF SURETIES:

Address                           :

Occupation and Address

:

Age and Date of birth

:

monthly                         :

Details of other properties Owed/

:

Possessed by the surety

Whether the surety has offered Himself

as surety for any other Transaction

earlier, if so, the details

:

10.

Whether salary certificate from the

:

attached.

11.

PARTICULARS OF INSTRUMENTS TO  BE

PURCHASED

Description                                    :

Make                                           :

Model                                          :

(d)

Invoice price (copy enclosed)

:

(e)

Name and Address of supplier/dealer

:

12.

(a)

Amount of lean applied for

:

(b)

No. of monthly installments

:

proposed for repayment

DECLARATION

I/We confirm that the funds will be used for the stated purpose only and will not

be used for speculation and/ or  anti-social purpose.

I/We understand that the Board has the right to recall the funds if they are not

used for the state purpose.

196

I/We understand that the sanction of the facility is at the discretion of the Board

and  I/  we  will  execute  necessary  Security  Documents  as  per  the  Board’s

requirements to its satisfaction.

:

Date   :

Signature of Applicant

Surety 1.

Name and Signature

(For Office Use only)

The application submitted by Shri ------------     -------------------- employed as

---------------  in  ----------------------------------  has  been  verified.  The  certificate  of

employment and surety in respect of the Borrower/ surety has been attached along

with the undertaking by the employer.

An amount of Rs.------------------------- (Rupees--------------------------------------

-----------------------------------------------------)  may  be  sanctioned  for  the  purpose

being  the  amount  requested/  amount  eligible  75%  of  the  invoice  amount  to  be

recovered of Rs.------------------------------ (Rupees------------------------------------------

-------) in ----------------------------equal monthly installments. The last installment will

be the amount outstanding after remittance of the -----------------------------------------

installment  including  other  dues  to  the  Board  at  the  time  of  closing  of  the  loan

amount.

Sanctioned/Rejected

District Executive Officer.

Secretary

HARYANA BUILDING AND OTHER CONSTRUCTION WORKERS WELFARE

BOARD

EMPLOYMENT CERTIFICATE

Certified  that Shri/ Smt .----------------------------------- S/o D/o, W/o -----------

------------------------------ House No. ---------------Town---------------------Desam--------

-----------------Village  ---------------------Taluk---------------------  District-------------------

197

now  residing  at    House  No.  -----------------  Town--------------,  Desam  -----------------

Village-------------Taluk---------------------- District ------------------------ is permanent /

officiating/ acting/ provisional------------------------- (Designation).

DETAILS OF HIS/HER SERVICE ARE AS UNDER

1.

Date of entry into service ---------------------------------------------------------

2.

Date of which continuous service beings----------------------------------------

3.

Date of retirement ------------------------------------------------------------------

DETAILS OF HIS/ HER PAY, ETC. ARE AS UNDER

1.

Basic pay -----------------------------(a) Provident Fund-----------------

2.

Dearness Allowance-----------------(b) LIC recoveries------------------

3.      HRA------------------------------------(c)           Tax----------------------

4.      Compensatory

1.-----------------------------------

2.-----------------------------------

3.-----------------------------------

5.

Other Allowance----------------------(e) Other recoveries

1.----------------------------------

2.----------------------------------

Total (A)---------------------------------------Total (B)-------------------------------------------

NET SALARY :

(A)   -   (B) Rs.-----------------------------------------------------------

:

Date

(Office

Designation of the Head of

Officer/ Department

UNDERTAKING FOR RECOVERY FROM PAY

I -----------------------------------------(Name in full)-----------------------------------

------------(Office/department)  owe  to  Haryana  Building  and  Other  Construction

Workers Welfare Board, the sum of Rs.----------------------------(Rupees------------------

---------------------------------------------------------------)  borrower  has  undertaken  to

198

repay in equated monthly  installments of Rs.------------------------------------ on--------

--------------------the -------------------------day of every month, I hereby agree that in

case  of  default  of  payment  of  monthly  installments  in  connection  with  the  said

transactions monthly recoveries of such amounts as may be fixed by the Board from

time to time of which information will be given by the Board may be made from my

salary  at  source  and  remitted  or  paid  to  the  Board  or  its  duly  authorized

representatives.

:

Dated :

I agree to effect the above recoveries.

:                                                 Signature

Date

(Office

FORM XVII

(see rule 56)

APPLICATION FOR FUNERAL BENEFIT

1.

Name and Address of Applicant

:

2.

Relationship of applicant with the worker

:

3.

Name and address  of worker

:

Registration

5.

Date of registration

:

6.

Date of payment and first subscription,

:

amount and name of bank, branch.

7.

Date of payment of last subscription,

:

amount, name of bank, branch.

8.

Duration of membership

:

9.

Whether membership was live ?

:

10.

Date of death of the worker

:

11.

Reason for death

:

12.

Whether applicant is the nominee of worker

:

13.

If not, whether the applicant has

:

199

dependence

14.

Name, age and date of birth of the nominee

:

15.

If nominees are minor, name or guardian

:

and his relationships with the children.

16.

Whether consent letters from other

:

nominees submitted (Where the No. of

nominees is more  than one)

17.

Whether certificate of guardianship

:

submitted by the minor children

18.

Amount of benefit, applied for

:

The above facts are true to my best of knowledge and information.

Name

Date   :

FORM XVIII

[(see rule 58 (1) ]

APPLICATION FOR  DEATH BENEFIT

1.

Name and address of applicant

:

worker

3.

Name and address of the worker

:

Registration

5.

Age and Date of Birth

:

whether                              :

7.

Nature of Death (Give details)

:

8.

Details of documents submitted

:

of

for

:

The above details are true to my knowledge and information.

Place  :

:

200

FORM XIX

(see rule 58 (4) ]

REGISTER OF DEATH BENEFIT

of

Name and

Period of

Date of

Order No.

receipt of

Register No. of

remittance

death

and date

application

worker

(2)                (3)                (4)             (5)             (6)

Name and Address

Amount of

Refund of           Total         Initial

Of nominee with

Death

Amount

Relationship to

Benefit

Member

(10)

FORM XX

(see rule 59)

APPLICATION FOR MEDICAL BENEFIT

1.

Name and address of applicant

:

2.

Age and date of birth

:

No.

201

4.

Date of payment of first subscription

:

amount and name of Bank

:

5.

Date of payment of last subscription

:

amount and name of Bank

:

Total           remitted                                :

surgery                :

8.

Disability if any, due to disease of surgery

:

9.

Period of treatment as patient in

:

Government Hospitals (Date of admission

In the Hospital and date of discharge)

10.

List of documents submitted

:

11.

Details of medical benefits received,

:

if any before.

The facts mentioned above are true to my knowledge and information.

Place  :

Date

applicant

FORM XXI

(see rule 59)

APPLICATION FOR EXGRATIA MEDICAL ASSISTANCE FOR ACCIDENTS

1.

Name and address of applicant

:

2.

Age and Date of birth

:

Registration                                                  :

4.

Date of payment of first subscription,

:

amount, Challan No. and name of Bank,

branch.

5.

Date of payment of last subscription,

:

Challan No., amount, name of Bank, branch.

6.

Total amount of subscription

:

details             accident

:

8.

Nature of disability due to accident

:

9.

Whether treated in Government hospital ?

:

If so, date of admission and date of discharge.

202

10.

Whether to applicant was in  plaster,

:

If so, for  how many days.

11.

Details of documents submitted

:

12.

Financial assistance applied for

:

13.

Have you received any financial

:

assistance for treatment before ? If yes,

particulars.

The above facts are true to the best of my knowledge and information.

Place  :

Date

Signature

FORM XXII

(see rule 60)

APPLICATION FOR EDUCATION SCHOLARSHIP

1.

Name of student

:

Female

:

(a)                                             :

(b) Whether proof is attached

:

4.

Name of college  and affiliated

:

Board

5.

Name and year of course

:

6.

Date of admission to the course

:

7.

Age and Date of birth of the student

:

8.

Details of qualifying examination passed :

Name of Examination

Name of affiliated

Month and of passing

year qualifying

University/Board/

Examination.

State.

9.

203

Marks scored in the qualifying                               Maximum

examination

Subject                          Mark

Scored

marks

Total marks

10.

(a)

Name of parent of applicant

:

(b)                     No.

(c)

date of payment of first subscription

:

(d)

Date of payment of last subscription

:

(e)

No. of installments paid

:

Total

(f)

(g)

Has the membership been revived

: Yes/No

The facts mentioned above are true to my knowledge. If selected for

the scholarship, I promise that I will abide by the condition stipulated in the

Scheme.

Place  :

Dated:                                                 Name

Affidavit of the Parent of the Student

I -----------------------------(Name and address) S/o or D/o (Name and address)

----------------------------------- solemnly affirm the following:

1.

My son/daughter Shri/Smt.---------------------- is studying for ---------------------

-------- (name and years of course).

2.

I am a member of the Board since ----------(Year) with registration No.--------

3.

subscription has been paid upto ----------------------

4.

If any of the above facts are found to be wrong later, the scholarship amount

granted to the student will be remitted back by me. The decision of Secretary

in this regard will be applicable to me and it will be final and I agree with that

5.

I also agree to recover any amount of default due from me.

204

Place:

Dated:                                                         Name

(To  be  signed  before  MLA/MP/Panchayat  President/  Gazetted  Officer  of  State  or

central)

I  certify  that  Smt,/  Shri  ------------------------  who  has  signed  above  has  put

the signature in my presence.

Place

Attesting

Dated:                                                 Name

(Seal)

I------------------------Head  of  --------------------(Name  of  institution)  hereby

certify  that  Smt./Shri  ----------------------------  is  a  -------------------------year  student

of  ------------------------course.  I  have  examined  the  application  submitted  by  the

student and I am convinced that it is correct. This institution is affiliated to the -------

--------- university/ Board.

Signature

:

Name          Designation

ENQUIRY REPORT OF DISTRICT EXECUTIVE OFFICER

1.

Shri/Smt.-------------------- is a live member of this Board, having registration

No. --------------------------------------------- and is paying subscription regularly.

2.

He has paid subscription regularly from ------------------ to---------------. He has

not defaulted payment of subscription. Membership has been revived for the

period  from  ---------------to  --------------------------.  I  recommend/  do  not

recommend the application (reason for rejection).

205

Executive

FORM XXIII

(see rule 61)

APPLICATION FOR MARRIAGE ASSISTANCE

Applicant                                            :

Address                                                         :

Registration                                                  :

4.

Age and Date of Birth

:

5.

Date of payment of first subscription

:

amount and name of Bank and branch.

6.

Date of payment of last subscription,

:

amount, name of bank and branch

7.

Duration of membership

:

:

9.

If application is for the marriage of son/daughter

wife.                           :

A member of this Board

(2)

If so, has she/he applied

:

for the financial assistance

(3)

Date of birth of the son/

:

daughter who is getting

or                               :

bridegroom of the son/daughter

(5)

Date and place of marriage

:

(6)

Date and No. of the Certificate of  :

marriage.

the                            :

authority who issued the certificate

(7)

Have you applied for financial

:

206

assistance for the marriage of any

other son/daughter; if so, details

of

10.

If application is for the Marriage of self (for women worker only)

(1)

Name and  address of

:

Husband/

(2)

Date and place of marriage

:

(3)

No. and Date of Marriage

:

Certificate

Name  of authority who

:

Issued

11.

Are you

in

any financial

Assistance for the purpose from

Government or any other institution

:

The above facts are true to the best of my knowledge and information.

Place:

Dated:                                         Name

FORM XXIV

(see rule 62)

APPLICATION FOR FAMILY PENSION

1.

Name and address of applicant

:

2.

Address of the pensioner/worker

:

3.                       with

4.

Date of death of the worker

:

5.

Monthly pension received by the

:

worker.

6.

Whether applicant is receiving pension from

:

Government/Semi-Government or any other

Institution ? If yes, details Thereof.

7.

Whether applicant is receiving salary from

:

Government/Semi-Government/private

207

Institutions ? If yes, details thereof.

8.

List of documents submitted

:

The above facts are true to the best of my knowledge and information.

Place:

Date:

Name

List of documents to be submitted along-with application

1.

Death certificate of the worker.

2.

Village  officer’s  certificate  showing  relationship  between  the  applicant

and the worker.

3.

Village  Officer’s  certificate  stating  that  the  applicant  is  not  receiving

any pension from Government/Semi- Government/ Private Institution.

4.

Village  Officer’s  Certificate  stating  that  the  applicant  is  not  receiving

any salary from Government/Semi-Government/ Private Institutions.

FORM XXV

[(see rule 87)]

REGISTER OF BUILDING WORKERS EMPLOYED BY THE EMPLOYER

Name and address of establishment

Name and permanent address of

where building and other construction

establishment

work is to be carried on.

Nature and location of work----------------------------------

S.

Name and

Age and

Father’s /

Nature of

Permanent Home

No.

Surname

Sex

Husband’s

Employment/

address of workman

of

name

Designation

(Village and Taluk

workman

and Distt.)

208

(1)        (2)          (3)          (4)               (5)                      (6)

1.

2.

3.

4.

Local

Date of

Signature or thumb

Date of

Reasons for

Address

commencement of  impression of

termination of

termination

employment

workman

employment

(7)               (8)                      (9)                  (10)

1.

2.

3.

4.

If the building workers is/was beneficiary,

Remarks

the date of registration as a beneficiary, the

registration No. and the name of Welfare

Board

1.

2.

3.

4.

FORM XXVI

[(see rule 88(1) (a)]

MUSTER ROLL

209

Name  and  permanent  address  Of  the  Name   and   address   of   establishment

establishment

where   building   or   other   construction

work is carried on/is to be carried on

Nature  of  building  or  other  construction   Name and address of Employer

work

For the month of--------------------------

Sr.No.

Name of the

Father’s/         Sex          Dates

building worker

Husband’s

name

(1)              (2)                 (3)                               (5)

1.                                                                 1.

2.

2.

3.

3.

4.

4.

5.

5.

FORM XXVII

[(see rule 88 (1) (a)]

REGISTER OF WAGES

Name and address of the Establishment

Name and permanent address of

where building or other construction

Establishment

work is carried on

Nature of building or other construction

Name and address of the Employer

work

Wage period; Monthly

210

Sr.No.   Name

Serial No. in the

Designation/

No. of days

Units of

workman

register of

nature of work

worked

work

workman

done

done

(1)        (2)              (3)                   (4)                  (5)            (6)

Wages/

Basic

Dearness       Overtime    Other       payments

Total

piece

wages

allowances

(Nature of payment to

rate

be indicated)

(7)          (8)            (9)           (10)                  (11)                 (12)

Deductions, if

Net amount paid

Signature/ Thumb

Initial of employer or

any, (indicate

impression of

his representative

nature)

workman

(13)                  (14)                   (15)                     (16)

Amount

211

FORM XXVIII

(see rule 88)(I) (a)

FORM OF REGISTER OF WAGES CUM-MUSTER ROLL

Name and address of the Establishment

Name and permanent address of

where building or other construction

Establishment

work is carried on

Nature of building or other construction

Name and address of the Employer

work

Sr.No

Sr.No. in

Name of

Designation/

Daily

Total

.

Register of

employee

nature of work

attendance/

attendance/

building

units worked

units of work

workers

done.

1.          2.              3.                 4.                    5.                   6.

Wages/

Basic

Dearness      Overtime     Other

Total

piece rate

wages

allowances

payments

(Nature of

payment to be

indicated)

(7)           (8)           (9)           (10)             (11)             (12)

212

Deductions, if any,

Net amount paid

Signature/Thumb

Initial of

(indicate nature)

impression of

employer or

workman

his

representative

(13)                     (14)                     (15)

Daily

of         earned

rate of-------------------------------------------------

FORM XXIX

[(see rule 88 (I) (b),(10)]

REGISTER OF DEDUCTIONS FOR DAMAGE OR LOSS

Name     and     Address     of

establishment  where  building  address    of    building                    of

or  other  construction  work  is  workers.

employer.

carried on/is to be carried on

Nature of building or other construction work

Sr.N   Name    Father’s/

Designation/

Particulars of

Date of

of work

Husband’s

nature of

damage of loss

damage of

name

employment

loss

(1)      (2)            (3)                   (4)                    (5)                 (6)

213

Whether

Name of

Amount of

No. of

Date of

recovery

building worker

person in

deduction

installments

showed cause

against

deduction

whose

presence

building

imposed

First

instalment

Last

instalment

workers

explanation

was heard

(10)                            (12)

FORM XXX

[(see rule 88 (I) (b),(10)]

REGISTER OF FINES

Name and address of the establishment

Name and permanent address of

where building or other construction

Establishment

work is carried on/ is to be carried on

Nature of building or other construction

Name and address of the Employer

work

S. N.

Name of

Father’s/

Designation/

Act/omission

Date of

building

Husband’s

nature of

for which fine

offence.

worker

name

employment

imposed

(1)       (2)             (3)                   (4)                   (5)                (6)

214

Whether

Name of person in

Wage

Amount

Date on  Remarks

building worker

whose presence

periods and

of the

which

showed cause

building worker’s

wages

fine

fine

against fine

explanation was

payable

imposed

realized

heard.

(7)                   (8)

FORM XXXI

[(see rule 88 (I) (b),(10)]

REGISTER OF ADVANCE

Name and address of the Establishment

Name and permanent address of

where building or other construction

Establishment

work is carried on/ is to be carried

Nature of building or other construction

Name and address of the Employer

work

S.No.    Name         Father’s/

Nature of

Wage period

Date and

Husband’s

employment/

and wages

amount of

name

Designation

payable

advance given

(1)        (2)           (3)               (4)                 (5)                 (6)

215

Purpose (s) for

No. of

Date and amount

Date on which

Remarks

which advance

installments by

of each

last installment

given

which advance to

installment repaid

was repaid

be repaid

(7)                 (8)                   (9)                 (10)

FORM XXXII

[(see rule 88 (I) (c),(10)]

REGISTER OF OVERTIME

Name and address of the Establishment

Name and permanent address of

where building or other construction

Establishment

work is carried on/ is to be carried

Nature of building or other construction

Name and address of the Employer

work

S.No.

Name of the

Father’s/            Sex        Designation/

building worker

Husband’s

Nature of

overtime

216

name                        employment

(1)           (2)                 (3)            (4)           (5)                (6)

Total overtime

Normal rates

Overtime

Overtime   Date on

Remarks

worked or

of wages

rate of

earnings

which

production in case

wages

overtime

of piece rated.

wages paid

(7)                 (8)

FORM XXXIII

[(see rule 88 (2) (a)]

WAGE BOOK

Name and address of the Employer

Name and permanent address of

Establishment

Name and address of the Establishment

Nature of building or other construction

where building or other construction

work

work is carried on/ is to be carried

For

Month

217

1.

No. of days worked-------------

2.

No. of units worked in case of piece rate workers------------

3.

Rate of daily/monthly wages/piece-rate--------------------------

4.

Amount of overtime wages ---------------------------------------------

wages           ----------------------------------------

6.

Deductions, if any, on account of the following;

a      fines

loss

c

loans and advance

d

subscription towards provident fund

e

subscription towards the Building Workers Welfare fund

f

any  other  deduction  e.g.  subscriptions  to  co-operative  society  or

account   of   loans   from   co-operative   society/housing   loan,   or

contribution  to  any  relief  fund  as  per  provision  of  Clause  (P)  of  sub

section (2) of Section 7 of the Payment of Wages Act or for payment of

any premium of Life Insurance Corporation.

7.

Net amount of wages paid-----------------------------

Initials of the employer

or his Representative.

FORM XXXIV

[(see rule 88 (2) (b)]

SERVICE CERTIFICATE

Name and permanent address of the Establishment

:

Name  and  address/location  where  the  building  or  :

other construction work carried on/ to be carried on

Nature and location of work

:

Name and address of the workman

:

Age or Date of Birth

:

Identification Marks

:

Father’s/ Husband’s name

:

Sr.No.,

Total period for which employed

Nature of

rate of wages (with

218

From

To

work done

particulars of unit in

case of (piece work)

(1)           (2)                  (3)                 (4)                    (5)

If the building worker was a

Reasons/ grounds on

Remarks

beneficiary his registration No. date

which the employment

and the name of the Board.

terminated

(6)                                (7)

FORM XXXV

(see rule 89)

ANNUAL RETURN OF EMPLOYER TO BE SENT TO THE REGISTERING

OFFICER.

Year ending 31  December------------

1.

Full  name  and  full  address  of  the  establishment  of  the  building  and  other

construction work (Place, Post officer, District)

2.

Name and permanent address of the establishment.

3.

Name and address of the employer

4.

Nature of building and other construction work carried on

219

5.

Full name of the Manager or person responsible for supervision and control of

the establishment.

6.

Number of building workers ordinarily employed.

7.

Total  number  of  days  during  the  year  on  which  building  workers  were

employed.

8.

Total number of man days worked by  building workers during the year

9.

Maximum number of building workers employed on any day during the year

10.

The number of accident that took place during the year as   under;-

a. 
b.

accidents.

The number of accident resulting in disablement of building workers for

less than 48 hours, the number of building workers involved, and the

number of man days lost.

c.

The  number

of

accidents  resulting

disablement

workers

beyond  48  hours  but  not  resulting  in  any  permanent  partial  or

permanent total disablement, the number of building workers involved

and the number of man days lost on account of such accidents.

d.

The  number

of

accidents

permanent  partial  or

total

disablement the number of building workers involved and the number

of man -days lost on account of such accidents.

e.

The  number

of

accidents  resulting  in

deaths

of

building  workers

and

the number of resultant deaths.

The   Chief   Inspector   or   Inspector   appointed   by   a   State

Government  under  the  Act  shall  direct  the  owners  of  establishment

registered  under  this  Act,  to  send  the  copies  of  Annual  Returns

submitted by the employers of registered establishments in respect of

the  concerned  State  Government  or  appropriate  Government  to  the

Chief Inspector of Inspections by virtue of provisions of Section 60 of

the Act.

The  Chief  Inspector  or  Inspectors  appointed  under  this  Act  by

the State Government shall direct the owners of such establishment as

are registered under this Act by registering  officers appointed by the

State Govt. to send copies of the Annual returns to the Chief Inspector

by virtue of provisions of section 60 of the Act.

11.

Change if any, in the management of the establishment, its location, or any

other  particulars  furnished  to  the  Registering  Officer  in  the  application  for

Registering indicating also the dates.

220

Place

Employer

Date

FORM  XXXVI

[(see rule 111 (c)]

CERTIFICATE OF MEDICAL EXAMINATION

1.

Certificate  Serial  No.-------------------------  Date------------------------------------

Date----------------------------------

2.      Name                  -----------------------------------------------------------------------

Identification marks:

(I)-----------------------------------

(2)-----------------------------------

3.

Father’s Name ----------------------------------

4.      Sex-------------------------------

5.

Residence ------------------------------Son/Daughter of -------------------------------

-----------------------------------------------------------------------------------------------

------------------------------------------

6.

Date of birth, if available----------------------------------and/ or certificate age ---

----------------------------------

7.      Physical

I hereby certify that I have personally examined (name)------------------

------------Son/ daughter/ wife of ---------------------------------------------residing

of  ---------------------------------------  who  is  desirous  of  being  employed  in

building  and  construction  work  and  that  his/  her  age  as  nearly  as  can  be

ascertained  from  my  examination  is  ------------------------years  and  that  he/

she is fit for employment in ---------------------as an adult/ adolescent.

8.

Reason for

1.

refusal of certificate --------------------------------------------------------------

--------------------------------------------------------------------------------------

2.

Certificate  being  revoked--------------------------------------------------------

----------------------------------------------------------------------------------

221

Signature /Left hand Thumb

Signature with Seal

Impression of building worker

Medical Inspector / C.M. O.

Note:

1.

Exact details of cause of physical disability should be clearly  stated.

2.

Functional/productive abilities should also be stated if disability is

stated.

FORM XXXVII

(see rule 111 (d)

HEALTH REGISTER

(IN RESPECT OF PERSONS EMPLOYED IN BUILDING AND OTHER CONSTRUCTION

WORK INVOLVING HAZARDOUS PROCESSES)

Name of the Construction Medical Officer/ Medical Inspector.

(A) Mr---------------------------------From----------------------------To-------------------------

(B) Mr---------------------------------From----------------------------To-------------------------

(C) Mr---------------------------------From----------------------------To-------------------------

S. N.

Works No.

Name of             Sex        Age

building worker

birthday)

of present work

(1)        (2)               (3)             (4)           (5)                    (6)

1.

2.

3..

Date of

Reason for

Nature of job

Raw Material

Date of medical

222

leaving or

leaving

of

or bye

examination by

transfer to

transfer or

occupation

product

certifying Surgeon

other work.

discharge

handled

Medical Inspector/ CMO

(7)             (8)             (9).             (10)                   (11)

Results of medical  If   suspended   from   Certified     fit     to

examination.

work, state period of  resume  duty  on  with   unfitness        or

suspension      with

detailed reasons.

Inspector /CMO

to worker

(12)                   (13)                     (14)                   (15)

Date:

Signature with date of Medical Inspector/CMO

Note:

(i)

Column (8)- Detailed summary of reason for transfer or discharge

should be stated

(ii)

Column (12) should be expressed as fit/unfit/suspended.

FORM XXXVIII

[(see rule 228) (a)]

NOTICE OF POISONING OR  OCCUPATIONAL NOTIFIABLE DISEASES

1. 
2.

3. 
4.

5.

Name and address of the employer;

Name of the building workers and his work No., if any;

Address of the building worker;

Sex and Age;

Occupation

223

6.

State exactly what the patient was doing at the time of contracting the

disease.

7.

Nature of

poisoning or disease from which The

from

Date;

Signature

Note:

When a building worker contract any disease specified in Schedule XII

a notice in this form shall be sent forthwith to the Chief Inspector.

FORM  XXXIX

[( see rule 136 (b))]

CERTIFICATE OF ANNUAL THOROUGH EXAMINATION OF LOOSE GEARS

EXEMPTED FROM ANNEALING

(a)

Name of the Construction site where loose gears are fitted/installed/ located;

Distinguishing

Description  of  Gear

Number  of  certificate  of   Remarks

number or mark

initial  and  periodical  test

and examination

(1)                    (2)                          (3)                     (4)

Name and address of public service,

Name    and  position  of  the  Competent

association, company or firm or testing

Person   of   public   service,   association,

establishment making the test and

company or firm or testing establishment

examination.

224

I  certify  that  on  the  --------------------------day  of-------------------19------------

the  above  gear,  described  in  column  (2)  was  thoroughly  examined;  and  that  no

defects affecting its safe working condition were found other than those indicated in

column (4 ).

Signature of the Competent Person

Seal

Date

Registration/Authority number of the Competent Person.

FORM XL

[see rule136 (d)and 140(b)(ii)]

CERTIFICATE OF INITIAL AND PERIODICAL TEST AND EXAMINATION OF

LOOSE GEARS

Test Certificate No.----------------------------

(a)     In  case  of  construction  site,  Name  of  the  construction  site  where  lifted

appliances are fitted/installed/located.

Distinguishing  Description

Number

Date

Test load

Safe working

Number or

dimension and

tested

of

applied

load (SWL)

Mark

material of gear/

test

(tonnes)

(tones)

device

(1)                 (2)                            (4)

Name and address

Initial test and

Name and address

Name and position

of manufacturer or

examination

of public service

of competent

suppliers;

certificate No. and

association,

Person in public

date (only in case

company or firm or

service,

225

of peridical test

testing

association,

and examination.

establishment

company or firm or

making the test

testing

and examination

establishment

(8)                    (9)                    (10)

I certify that on the ------------------------day of -----------------19 ----------- the

above  gear  was  tested  and  examined  in  the  manner  set  forth  overleaf;  that  the

examination showed the said gear/ device  withstood the test load without injury or

deformation; and that  the safe working load of the said gear/ device  is as shown in

column 6.

Signature of the Competent Person

Seal

Date

Registration/Authority number of the Competent Person.

FORM XLI

[(see rule 140 (a)]

REGISTER OF PERIODICAL TEST-EXAMINATION OF

LIFTING APPLIANCE AND GEARS ETC.

PART-I

Initial  and  periodical  load  test  of  lifting  appliances  and  their  annual  thorough

examination.

“Thorough  examination”  means  a  visual  examination,  supplemented,  if

necessary, by other means such as a hammer test ,carried out as carefully as

the  conditions  permit,  in  order  to  arrive  at  a  reliable  conclusion  as  to  the

safety of the parts examined and if necessary, for such examination parts of

the lifting appliances and gear shall be dismantled.

(A)

Initial and periodical load tests of lifting appliance

226

Situation and

No. of

I certify that on the date on

Remarks (to

description of

certificate of

which I have appended by

be, signed

lifting appliances

test and

signature the lifting appliance

and dated)

tested with

examination of

shown in column (I) was

distinguishing

competent

tested and no defects affecting

number of marks

person

its safe working conditions

if any

were found other than those

shown in column (5)

Date and

Date and

Signature With

signature

seal

with seal

(1)                 (2)

1.

2.

(B)

Annual thorough examination

I  certify  that  on  the  date  to  which  I  have  appended  my  signature,

the  lifting  appliance  shown  in  column  (I)  was  thoroughly  examined

and no defects affecting its safe working conditions were found other

than those shown in Column (12)

Date and

Date and

Date and

Date and

Date and

Date and

Remarks to

signature

signature

signature

signature

signature

signature

be signed

with seal

with seal

with seal

with seal

with seal

with seal

and dated

(6)          (7)          (8)          (9)                         (11)

Note:

If all the lifting appliances are thoroughly examined on the same date

it will be sufficient to enter in column (I) ‘’All lifting appliance.” If not,

227

the parts which have been  thoroughly examined on the dates must be

clearly indicated.

PART-II

Initial and periodical load test of loose gears and annual thorough

examination.

List of loose gear:

The following classes of loose gears namely;-

1.

Chains made of

meleable cast

iron;

2.

3. 
4. 
5.

Plate link chains;

Chains, rings, hooks, shackles and swivels made of steel

Pitched chains;

Rings, hooks, shackles and swivels permanently attached to pitched  chains,

pulley  blocks,  container  spreaders,  trays,  slings,  baskets,  etc.  and  any  other

similar gear.

6.

hooks

and

ball bearings or

other

case

heardened parts; and

7.

Bordeaux connections.

Initial Test and periodical load test of loose gears

Distinguishing  Description

No. of

I certify that on the date to which I have

No. or marks

of loose

certificates

appended my signature  the loose gears

gear

of test and

shown in column (I) and (2) were tested

tested and

examination  and no defects affecting the safe working

examined

of

condition  were  found  other  than  those

competent

shown in column (6)

person

Date and signature

Date and signature

with seal

with seal

(1)             (2)                                    (4)

(5)

Annual thorough examination of loose gears

228

I

that on

the date to which

have appended my signature the

loose  gear  shown  in  column  (I)  and  (2)  were  thoroughly    examined

by  me  and  no  defects  effecting  their  safe  working  condition  were

found other than those shown in  column (10).

Remarks  (to

Date and

Date and

Date and

Date and

be     signed

signature with

signature with

signature with

and dated)

seal

seal

seal

seal

(6)               (7)                 (8)                 (9)                (10)

1.

2.

3.

PART-III

Annealing of Chains, Rings, Hooks, Shackles and Swivels (other than those

exempted)

(SEE PART II)

12.5 mm and smaller

If used with lifting appliance driven by power must be

chains rings, hooks,

annealed once atleast in every six months

shackles and swivels in

If used solely with lifting appliance worked by hand, must

general use

be annealed once atleast in every twelve months

Other chains, rings,

If used with lifting appliance driven by power, must be

hooks shackles and

annealed once atleast in twelve months

swivels in general use

If used solely with lifting appliance worked by hand, must

be   annealed once atleast in every two years.

Note:-

It is recommended though not required by Rules that annealing should

be carried out in a suitably constructed furnace heated to temperature

between 1100 degree and 1300 degree  Fahrenheit or 600 degree and

700 degree Centigrade, for a period between 30 and 60 minutes.

229

Distinguishing

Description    No.   of   the   I  certify  that  on  the  date  to   Remarks

No. or mark      of      gear

annealed      test      and

examination   in  Columns.  1  and  2  was  and

effectually   annealed   under   dated)

my  supervision;  that  after

being    so  annealed  every

article      was      carefully

inspected    and    that    no

defects    affecting  its  safe

working      condition   were

found   other   than   those

shown in Columns.7

Date

and

and

and

signatur

signatur

signatur

e    with        with

seal

seal

seal

(1)             (2)

FORM XLII

[(see rule 140 (b), (i)(a)Schedule I)]

CERTIFICATE OF  INITIAL AND PERIODICAL TEST AND EXAMINATION OF

WINCHES, DERRICKS AND THEIR ACCESSORY GEAR.

Test Certificate No.---------------------

(a)     In  case  of  construction  site,  Name  of  the  construction  site  where  lifted

appliances are fitted/installed/located.

230

Situation and Description of

Angle to the

Test load

Safe working load at

lifting appliances and Gear

horizontal of derrick

applied

the angle shown in

with distinguishing number

boom at which test

column (2)

of marks (if any) which have

load applied.

been tested, thoroughly

examined.

1                           2

(Degree)

Name and address of public service,

Name  and position of the Competent

association company or firm or testing

Person of public service, association,

establishment making the test and

company or firm or testing establishment.,

examination.

I  certify  that  on  the  ----------------------------day  of  19---------------------  the

lifting appliance shown in column (I) together with its necessary gear was tested in

the manner set forth overleaf in my presence; that a careful examination of the said

lifting  appliances  after  the  test  showed  that  it  had  withstood  the  test  load  without

injury or permanent deformation; and that the safe working load of the said lifting

appliance and accessory gear is as shown in column (4) .

Signature of the Competent Person

Date

Seal

Registration/Authority number of the Competent Person.

231

FORM XLIII

[(see rule 140 (b)(i)(b)]

CERTIFICATE  OF  INITIAL  AND  PERIODICAL  TEST  AND  EXAMINATION  OF

CRANES OR HOISTS  AND THEIR ACCESSORY GEAR

Test certificate No.-----------------------------------------

(a)

Name of the construction site where lifted appliances are

fitted/installed/located.

Situation and

For

jib cranes radius

Test

load applied

Safe working

description

at the test load was

jib cranes at radius

applied.

shown in Column (2)

(1)                      (2)                      (3)                      (4)

(Tonnes)

Name and address of public service,

Name  and position of the Competent

association or firm or testing establishment

Person of public service, association,

making the test and examination.

company or firm or testing establishment

I  certifying  that  on  the  ------------------------day  of  ------------------------the

above  lifting  appliance  together  with  its  accessory  gear,  was  tested  in  the  manner

set forth overleaf; that a careful examination of the said lifting appliance and gear

after the test showed that it had withstood the test load without injury or permanent

deformation; and the safe working load of the said lifting appliance and gear is as

shown in column (4).

Signature of the Competent Person

Seal

Date

(See note 3)

Registration/Authority number of the

232

Competent Person.

FORM  XLIV

[(see rule 140 (b)(iii)]

CERTIFICATE OF TEST AND EXAMINATION OF WIREROPE BEFORE BEING

TAKEN INTOUSE

Test certificate No.--------------------------

(1)

Name and address of maker  or supplier;

(2)

(a)

Circumference/ diameter of rope.

(b)    Number

(c)

Number of wires per strand

(d)    Lay

(e)     Core

(3)

Quality of wire (e.g. Best Plough steel)

(4)

(a)

Date of test of sample of rope

(b)

Load at which sample broke ( tonnes)

(c)

Safe working load of rope (tonnes)

(d)

Intended

use

5.

Name and address of public service, association, company or firm or  testing

establishment making the test and  examination.

6.

Name and position of Competent  Person in public service, association,

company or firm or testing establishment making the test and examination.

I  certify  that  the  above  particulars  are  correct  and  that  the  test  and

examination were carried out by me and no defect Effecting its safe working

load (SWL) were found.

Signature of the Competent person

Seal

Date

Registration/ Authority number of the Competent person.

233

FORM XLV

[(see rule 140 (b)(iv)]

CERTIFICATE OF ANNEALING OF LOOSE GEARS

Test Certificate No.-------------------------

(a)

Name of the construction site where lifted appliances are

fitted/installed/located.

Distinguishin

Descriptio

Number of the

Number

Date of

Defects found at

g number or

n of gear

certificate of

annealed

annealing

careful inspection

mark

test and

after annealing

examination

(1)            (2)              (3)             (4)                                  (6)

Name and address of public service,

Name  and position of the Competent

association, company or firm or testing

Person of public service association,

establishment carrying out the annealing

company or firm or testing establishment

and inspection.

I certify that on the date shown in column (5) the gear described in columns

(I)  to  (4)    was  effectually  annealed  under    my  supervision  that  after  being  so

annealed every article was carefully inspected ; and that no defects affecting its safe

working condition were found other than those indicated in column (6).

Signature of the Competent Person

Seal

Date

234

Registration/Authority of the Competent Person.

FORM XLVI

[(see rule 276 (7)]

REPORT OF ACCIDENTS AND DANGEROUS OCCURRENCES

1.

Name of the project/work

2.

Location of project/work

3.

Stage of construction work

4.      Particulars

(a)     Main

Name

Address

Phone Nos.

Nature of Business

(b)    Sub-contractor’s

Name

Address

Phone Nos

Nature of business

5.

Particulars of injured person

(a)     Name

(First)                        (Middle)                  (Surname)

(b)    Home

(c)     Occupation

(d)

Status of the worker;

Casual

(e)

Sex: Male

Female

(f)     Age

(g)    Experience

(h)

Marital status: Married/Unmarried/Divorced

6.      Particulars

(a)

Exact place where accident occurred

235

(b     )Date

(c)     Time

(d)

What the injured person was doing at the time of accident?

(e)     Weather

(f)

How long employed by you for this particular job?

(g)

Particulars of equipment/machine/tool involvedand condition of the

same after the accident occurred.

(h)

Brief description of the accident.

7.

Nature of injuries

(a) 
(b)

Fatal

Non-fatal

(c)

If non-fatal state precisely the nature of injuries.(Describe in detail the

nature of injury, for instance fracture of right arm. sprain etc.)

(d)

First Aid:

Given:

Not given:

(e)

If not, given the reasons

(f)

Name and designation of the person by whom first aid was given

(g)

If admitted to hospital,

Name of the hospital:

Address of the hospital

Phone No.

Name of the Doctor

8.

Mode of transport used

Ambulance

Truck

Tempo

Taxi

Private Car

9.

(a)

How much time was taken to shift the injured person? If very late,

state the reasons

(b)

How the reporting was made?

Telephone

Telegram

Special Messenger

Letter

236

(c)

Who  visited  the  accident  site  first  and  what  action  was  proposed  by

him?

(d)

What are the actions taken for the investigation of the accident by the

employer?   (Describe   about   photographs/video   film/measurements

taken etc.)

10.

Particulars of the persons given witness.

Address                Occupation

1.

2.

3.

4.

(b)    Whether     Temporary           Permanent

11.

Particulars in case of fatal;

Date

Time

Whether  registered  with  Building  and  other  Construction  Workers

Welfare Board

If yes, give Reg. No-------------------------

12.

Dangerous Occurrences as covered under the Regulation No. (Give details)

(a)

Collapse or failure of lifting appliance, hoist, conveyors etc.

(b)

Collapse or subsidence of soil, any wall, floor

gallery etc.

(c)

Collapse of transmission towers, pipeline, bridges etc.

(d)

explosion of receiver, vessel etc.

(e)

fire and explosion

of hazardous substances.

(g)

collapse, capsizing, toppling or collisions of transport equipment.

(h)

leakage or release of harmful toxic gases at the construction site.

(i)

failure  of  lifting  appliance,  loose  gear,  hoist  or  building  and  other

construction work machinery, transport equipment etc.

13.

Certificate from the Employer or authorized signatory.

I  certify  that  to  the  best  of  my  knowledge  and  belief,  the  above

particulars are correct in every respect.

237

Note;

If more than one person is involved, then for each person, information

is to be filled up in separate forms.

238